3GPP TSG-RAN WG4 Meeting #116	R4-2512292

# Bengaluru , IN, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133 for RedCap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability of requirements and Cell Selection for RedCap UEs with NTN needs to be introduced. Add measurement requirement for RedCap UEs with NTN |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the Applicability of requirements for RedCap UEs with satellite access in clause 3.6.20. - Add the offset applicability for RedCap UEs with satellite access in clause 4.1. - Add measurement requirement for RedCap UEs with NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The applicability of requirements and Cell Selection for RedCap UEs with NTN would be missed. - No measurement requirement for RedCap UEs with NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 3.6.20, 4.1, 9.1D, 9.2E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2509293 |  |  |  |  |  |  |  |  |

<Start of Change 1>

3.6.20 Applicability of requirements for RedCap UE with satellite access

The requirements for RedCap UEs with satellite access defined in clauses with suffix ‘X’ apply provided that UE indicates whether the UE supports (e)RedCap UE with FR1-NTN and is accessing a cell served by a Satellite Access Node (SAN) and operating in FR1-NTN bands. The requirements apply provided that serving and all neighbour satellites on the same layer are of same satellite type (LEO or GEO). The applicability defined for RedCap UEs in clause 3.6.11 is also applicable to RedCap UEs with satellite access except for FR2 related requirements.

In addition, the requirements for UE supporting 160 ms SSB periodicity assumed during initial access do not apply for RedCap UEs operating in FR1-NTN bands.

<End of Change 1>

<Start of Change 2>

4.1	Cell Selection

After a UE has switched on and a PLMN has been selected, the Cell selection process takes place, as described in TS 38.304 [1]. This process allows the UE to select a suitable cell where to camp on in order to access available services. In this process, the UE can use stored information (Stored information cell selection) or not (Initial cell selection).

The 1 Rx RedCap UE for the cell selection procedure [1] applies:

-	Qrxlevmin as the signaled value of Qrxlevmin [2] -1 dB.

-	Qqualmin as the signaled value of Qqualmin [2] -1 dB.

The above offset for Qrxlevmin and Qqualmin is also applicable to RedCap UEs with satellite access.

<End of Change 2>

<Start of Change 3>

9.1D	General measurement requirement for RedCap with satellite access

9.1D.1	Introduction

This clause contains general requirements on the RedCap UE regarding measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency, inter-frequency and L1-RSRP measurements requirements. These measurements may be used by the NG-RAN. The measurement quantities are defined in TS 38.215 [4], the measurement model is defined in TS 38.300 [10], TS 37.340 [17] and measurement accuracies are specified in clause 10. Control of measurement reporting is specified in TS 38.331 [2].

9.1D.2	Measurement gap

The requirements in clause 9.1C.2 shall apply.

9.1D.8	Concurrent measurement gaps for RedCap with SAN

9.1D.8.1	Introduction

When UE supports concurrent measurement gap pattern capability, network can provide multiple measurement gaps configured by RRC message(s) as specified in TS 38.331 [2].

9.1D.8.2	Requirements

The requirements in clause 9.1C.8.2 shall apply.

9.1D.8.3	Collision between concurrent measurement gaps

The requirements in clause 9.1C.8.3 shall apply except that

-	If UE supports parallelMeasurementWithoutRestriction-r17,

-	If UE is configured to measure 1 satellite in each of the SMTC occasions, the distance between the two occasions is equal to or smaller than 4 ms.

-	Otherwise, the distance between the two occasions is equal to or smaller than 5 ms.

-	If UE doesn’t support parallelMeasurementWithoutRestriction-r17,

-	The distance between the two occasions is equal to or smaller than 5 ms.

9.1D.8.4	Measurement gap related requirements of concurrent measurement gaps

The requirements in clause 9.1C.8.4 shall apply.

9.1D.9	Collision between SMTC and measurement gap for RedCap with satellite access

9.1D.9.1	Introduction

This clause contains definition of collision between SMTCs and measurement gap for SAN.

9.1D.9.2	Collision between SMTCs and measurement gap

The requirements in clause 9.1C.9.2 shall apply except that

-	If UE supports parallelMeasurementWithoutRestriction-r17,

-	If UE is configured to measure 1 satellite in each of the SMTC occasions, the magnitude of the distance between the SMTC occasion and the measurement gap occasion in time domain is less than or equals to 4 ms.

-	Otherwise, the magnitude of the distance between the SMTC occasion and the measurement gap occasion in time domain is less than or equals to 5 ms.

-	If UE doesn’t support parallelMeasurementWithoutRestriction-r17,

-	The magnitude of the distance between the SMTC occasion and the measurement gap occasion in time domain is less than or equals to 5 ms.

9.1D.9.3	Collision between multiple SMTCs on a SAN carrier

The requirements in clause 9.1C.9.2 shall apply except that

-	If UE supports parallelMeasurementWithoutRestriction-r17,

-	If UE is configured to measure 1 satellite in each of the SMTC occasions, the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms.

-	Otherwise, the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 5 ms.

-	If UE doesn’t support parallelMeasurementWithoutRestriction-r17,

-	The magnitude of the distance between two SMTC occasions in time domain is less than or equals to 5 ms.

<End of Change 3>

<Start of Change 4>

9.2E	NR intra-frequency measurements for RedCap with SAN

9.2E.1	Introduction

The requirements in clause 9.2E apply for intra-frequency measurements for RedCap on an SAN carrier frequency. The requirements apply provided that the valid parameters of ephemeris information, epoch time of the ephemeris, common TA, validity timer information, downlink polarization information for target NR SAN cell are send to UE.

The requirements in clause 9.2C.1 shall apply except that

-  clause 9.2C is replaced with clause 9.2E, and

-	clause 9.2C.4 is replaced with clause 9.2E.4, and

-	clause 9.2C.5 is replaced with clause 9.2E.5, and

-	clause 9.2C.6 is replaced with clause 9.2E.6

9.2E.2	Requirements applicability

The requirements in clause 9.2E apply, provided:

-	The cell being identified or measured is detectable.

-	Valid information for the satellite serving the target cell has been provided

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2Rx RedCap with NTN:

-	SS-RSRP related side conditions given in clauses 10.1.2C for FR1 and 10.1.3C for FR2-NTN, for a corresponding band,

-	SS-RSRQ related side conditions given in clauses 10.1.7C for FR1 and 10.1.8C for FR2-NTN, for a corresponding band,

-	SS-SINR related side conditions given in clauses 10.1.12C for FR1 and 10.1.13C for FR2-NTN, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.17 for a corresponding band.

-	For 1Rx RedCap with NTN:

-	SS-RSRP related side conditions given in clause 10.1.2D for FR1 for a corresponding band,

-	SS-RSRQ related side conditions given in clause 10.1.6D for FR1 for a corresponding band,

-	SS-SINR related side conditions given in clause 10.1.10D for FR1 for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.15 for a corresponding band.

9.2E.3	Number of cells and number of SSB

9.2E.3.1	Requirements for FR1

The requirements in clause 9.2C.3.1 shall apply.

9.2E.4	Measurement Reporting Requirements

9.2E.4.1	Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2D.1 (SS-RSRP for FR1), 10.1A.6D.1 (SS-RSRQ for FR1) and 10.1.10D.1 (SS-SINR for FR1).

9.2E.4.2	Event-triggered Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2D.1 (SS-RSRP for FR1), 10.1.6D.1 (SS-RSRQ for FR1) and 10.1.10D.1 (SS-SINR for FR1).

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2E.4.3.

9.2E.4.3	Event Triggered Reporting

The requirements for FR1 in clause 9.2C.4.3 shall apply except that

-	Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2D.1 (SS-RSRP for FR1), 10.1.6D.1 (SS-RSRQ for FR1) and 10.1.10D.1 (SS-SINR for FR1), and

-	clause 9.2C.5.1 is replaced with 9.2E.5.1, and

-	clause 9.2C.6.2 is replaced with 9.2E.6.2.

9.2E.5	Intra-frequency measurements without measurement gaps

9.2E.5.1	Intra-frequency cell identification

The requirements in clause 9.2C.5.1 shall apply except that:

-	TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2C.5.1-1 for 2Rx RedCap UE and table 9.2E.5.1-1 for 1Rx RedCap UE, and

-	TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2C.5.1-2 for 2Rx RedCap UE and table 9.2E.5.1-2 for 1Rx RedCap UE, and

-	TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2C.5.2-1 for 2Rx RedCap UE and table 9.2E.5.2-1 for 1Rx RedCap UE, and

Kmulti_SMTC=1

Kmulti_SMTC=NLEO,iNLEO,simul

Kmulti_SMTC=NSMTC,overlap

Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul

NLEO,i

NLEO,simul

NSMTC,overlap

-  CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap_RedCap,i in clause 9.1A.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps, or according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps, and

-	clause 9.1C.8.3 is replaced with 9.1E.8.3, and

-	clause 9.1C.9.1 is replaced with 9.1E.9.1, and

-	clause 9.2C.5.1 is replaced with 9.2E.5.1, and

-	clause 9.2C.5.2 is replaced with 9.2E.5.2.

Table 9.2E.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) for 1Rx RedCap UE with NTN

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2E.5.1-2: Time period for time index detection (FR1) for 1Rx RedCap UE with NTN

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(160 ms, ceil( 6 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(160 ms, ceil (1.5 x 6 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | Ceil( 6 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

9.2E.5.2	Measurement period

The requirements in clause 9.2C.5.2 shall apply except that:

-	The measurement period for intra-frequency measurements without gaps is as shown in table 9.2C.5.2-1 for 1Rx RedCap UE and 2Rx RedCap UE.

9.2E.5.3	Scheduling availability of UE during intra-frequency measurements

The requirements in clause 9.2C.5.3 shall apply.

9.2E.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

The requirements in clause 9.2C.5.3.1 shall apply.

9.2E.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO

The requirements in clause 9.2C.5.3.1 shall apply.

9.2E.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1

The requirements in clause 9.2B.5.3.4 shall apply.

9.2E.6	Intra-frequency measurements with measurement gaps

9.2E.6.1	Intra-frequency cell identification

The requirements in clause 9.2C.6.1 shall apply except that:

-	TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2C.6.1-1 for 2Rx RedCap UE and table 9.2E.6.1-1 for 1Rx RedCap UE, and

-	TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2C.6.1-2 for 2Rx RedCap UE and table 9.2E.6.1-2 for 1Rx RedCap UE, and

-	TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2C.6.2-1 for 2Rx RedCap UE and table 9.2E.6.1-1for 1Rx RedCap UE, and

-  clause 9.1.5.2 is replaced with 9.1E.5.2, and

-	clause 9.1C.8.3 is replaced with 9.1E.8.3.

Kmulti_SMTC=1

Kmulti_SMTC=NLEO,iNLEO,simul

Kmulti_SMTC=NSMTC,overlap

Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul

NLEO,i

NLEO,simul

NSMTC,overlap

Table 9.2E.6.1-1: Time period for PSS/SSS detection (FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2E.6.1-2: Time period for time index detection (FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(160 ms, 6 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(160 ms, ceil(1.5 x 6) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 6 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

9.2E.6.2	Intra-frequency Measurement Period

The requirements in clause 9.2C.6.3 shall apply except that:

-	The measurement period for intra-frequency measurements without gaps is as shown in table 9.2E.6.2-1 for 1Rx RedCap UE and 2Rx RedCap UE.

Table 9.2E.6.2-1: Measurement period for intra-frequency measurements with gaps (FR1) for RedCap UE

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5) x Kmulti_SMTC x Kgap x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_RedCap |

<End of Change 4>