3GPP TSG-RAN4 Meeting #116	R4-2512299

# Bengaluru, India, 25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on HO requirements for RedCap UE in NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the endorsed draftCR R4-2506794, there are [] around applicability related to SSB availability in HO requirements for HD-FDD UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on agreements in RAN4#116, remove the applicability related to SSB availability in HO requirements for HD-FDD UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HO requirements for RedCap UE in NTN are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1F (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draftCR is based on endorsed Big draftCR R4-2508462 from RAN4#115. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 6.1F	Handover for RedCap UE with satellite access

### 6.1F.1	NR SAN Handover

#### 6.1F.1.1	Introduction

The purpose of NR SAN handover is to change the NR SAN PCell to another NR SAN cell for RedCap UE. The requirements in this clause are applicable to SA NR SAN.

Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where:

-	The reference SSB of the serving cell is the SSB in the active DL BWP of serving cell

-	The reference SSB of the target cell is the SSB in the first active DL BWP of the target cell upon reconfiguration.

#### 6.1F.1.2	NR SAN FR1 – NR SAN FR1 Handover

The applicability defined in clause 6.1F.1.2 apply for the requirements in this clause.

##### 6.1F.1.2.1	Handover delay

The requirements in clause 6.1C.1.2.1 shall apply except that

-	clause 6.1C.1.2.2.1 is replaced with 6.1F.1.2.2.1, and

-	clause 6.1C.1.2.2.2 is replaced with 6.1F.1.2.2.2.

##### 6.1F.1.2.2	Interruption time

The requirements in clause 6.1C.1.2.2 shall apply except that

-	clause 9.2C.5 is replaced with 9.2E.5, and

-	clause 9.3C.4 is replaced with 9.3E.4.

6.1F.1.2.2.1	Interruption time for RACH-based handover

The requirements in clause 6.1C.1.2.2.1 shall apply for RedCap UE with 2 Rx antennas.

The requirements in clause 6.1C.1.2.2.1 shall apply for RedCap UE with 1 Rx antennas except that Tsearch is defined as

-	Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 2*Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 5* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

6.1F.1.2.2.2	Interruption time for RACH-less handover

The requirements in clause 6.1C.1.2.2.2 shall apply except that clause 6.1F.1.2.2.1 is replaced with 6.1F.1.2.2.1.

### 6.1F.2	NR SAN Conditional Handover

#### 6.1F.2.1	Introduction

The requirements in this clause are applicable to conditional handover to change the NR SAN PCell to another NR SAN cell for RedCap UE.

#### 6.1F.2.2	NR SAN FR1 – NR SAN FR1 conditional handover

The applicability defined in clause 6.1F.2.2 apply for the the requirements in this clause except that

-	clause 6.1C.2.2.2 is replaced with 6.1F.2.2.2, and

-	clause 6.1C.2.2.3 is replaced with 6.1F.2.2.3, and

-	clause 6.1C.2.2.4 is replaced with 6.1F.2.2.4.

##### 6.1F.2.2.1	Handover delay

The requirements in clause 6.1C.2.2.1 shall apply except that

-	clause 6.1C.2.2.2 is replaced with 6.1F.2.2.2, and

-	clause 6.1C.2.2.3 is replaced with 6.1F.2.2.3, and

-	clause 6.1C.2.2.4 is replaced with 6.1F.2.2.4.

##### 6.1F.2.2.2	Measurement time

The requirements in clause 6.1C.2.2.2 shall apply except that

-	clause 9.2C.5.1 is replaced with 9.2E.5.1, and

-	clause 9.2C.6.1 is replaced with 9.2E.6.1, and

-	clause 9.3C.7.1 is replaced with 9.3E.7.1.

##### 6.1F.2.2.3	Preparation time

The requirements in clause 6.1C.2.2.3 shall apply.

##### 6.1F.2.2.4	Interruption time

The requirements in clause 6.1C.2.2.4 shall apply.

#### 6.1F.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria

The applicability defined in clause 6.1C.2.3 apply for the requirements in this clause except that

-	clause 6.1C.2.3.2 is replaced with 6.1F.2.3.2, and

-	clause 6.1C.2.3.3 is replaced with 6.1F.2.3.3.

##### 6.1F.2.3.1	Handover delay

The requirements in clause 6.1C.2.3.1 shall apply except that

-	clause 6.1C.2.3.2 is replaced with 6.1F.2.3.2, and

-	clause 6.1C.2.3.3 is replaced with 6.1F.2.3.3.

##### 6.1F.2.3.2	Preparation time

The requirements in clause 6.1C.2.3.2 shall apply.

##### 6.1F.2.3.3	Interruption time

The requirements in clause 6.1C.2.3.3 shall apply for RedCap UE with 2 Rx antennas.

The requirements in clause 6.1C.2.3.3 shall apply for RedCap UE with 1 Rx antennas except that Tsearch is defined as

-	Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 2*Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 5* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

### 6.1F.3	NR SAN Satellite switching with re-synchronization

#### 6.1F.3.1	Introduction

The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing for RedCap UE. The requirements in this clause are applicable to SA NR SAN.

#### 6.1F.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization

The applicability defined in clause 6.1C.3.2 apply for the the requirements in this clause except that

-	clause 6.1C.3.2.2 is replaced with 6.1F.3.2.2, and

-	clause 6.1C.3.2.3 is replaced with 6.1F.3.2.3.

##### 6.1F.3.2.1	Satellite switching delay

The requirements in clause 6.1C.3.2.1 apply for the the requirements in this clause except that

-	clause 6.1C.3.2.2 is replaced with 6.1F.3.2.2, and

-	clause 6.1C.3.2.3 is replaced with 6.1F.3.2.3.

##### 6.1F.3.2.2	Interruption time for hard satellite switch with re-sync

The requirements in clause 6.1C.3.2.2 shall apply for RedCap UE with 2 Rx antennas except that

-	clause 6.1C.1.2.2.1 is replaced with 6.1F.1.2.2.1.

The requirements in clause 6.1C.3.2.2 shall apply for RedCap UE with 1 Rx antennas except that

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB + TSSB ms, where TSSB is the periodicity of the SSB of the source satellite. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	clause 6.1C.1.2.2.1 is replaced with 6.1F.1.2.2.1.

##### 6.1F.3.2.3	Satellite switch delay for soft satellite switch with re-sync

The requirements in clause 6.1C.3.2.3 shall apply for RedCap UE with 2 Rx antennas except that

-	clause 6.1C.1.2.2.1 is replaced with 6.1F.1.2.2.1, and

-	clause 9.2C.5.3 is replaced with 9.2E.5.3.

The requirements in clause 6.1C.3.2.2 shall apply for RedCap UE with 1 Rx antennas except that

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB + TSSB ms, where TSSB is the periodicity of the SSB of the source satellite. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	clause 6.1C.1.2.2.1 is replaced with 6.1F.1.2.2.1, and

-	clause 9.2C.5.3 is replaced with 9.2E.5.3.

<End of Change 1>