3GPP TSG-RAN4 Meeting #116	R4-2512674

# Bengalur, India, 25th – 29th, August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TS 38.104 to add unwanted emissions for SBFD (NR_duplex_evo-Core) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce R19 SBFD BS unwanted emissions and OTA unwanted emissions requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify unwanted emissions and OTA unwanted emissions requirements for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R19 SBFD unwanted emissions requirements would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.6, 12.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised form R4-2510133 |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.104>>

12.2.6	Unwanted emissions for SBFD

12.2.6.1	General

The general description for unwanted emissions as clause 6.6 shall be applied.

12.2.6.2	Minimum requirement for BS type 1-H

For occupied bandwidth, adjacent channel leakage power ratio (ACLR), operating band unwanted emissions (OBUE), and transmitter spurious emissions the same requirements as specified in clause 6.6 shall be applied.

Requirements related with multi-band and non-contiguous spectrum are not applied to SBFD.

< Next of change >

12.5.7	OTA unwanted emissions

12.5.7.1	General

The general description for OTA unwanted emissions as clause 9.7 shall be applied.

12.5.7.2	Minimum requirement for BS type 1-O and BS type 2-O

For OTA occupied bandwidth, OTA adjacent channel leakage power ratio (ACLR), OTA operating band unwanted emissions (OBUE), and OTA transmitter spurious emissions the same requirements as specified in clause 9.7 shall be applied.

Requirements related with multi-band and non-contiguous spectrum are not applied to SBFD.

<< End of change >>