3GPP TSG-RAN WG4 Meeting # 116		R4-2509068

Bengaluru, India, August 25th - 29th, 2025

Agenda item:			7.26.1

Source:	Moderator (Nokia)

Title:	Topic summary for [116][225] NR_XR_Ph3

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [116][225] NR_XR_Ph3.

This summary is based on the contributions submitted to the agenda items:

- 7.26	XR for NR Phase 3										[NR_XR_Ph3]

  - 7.26.1	Moderator summary and conclusions		[NR_XR_Ph3-Core]

  - 7.26.2	General aspects								[NR_XR_Ph3-Core]

  - 7.26.3	RRM core requirements						[NR_XR_Ph3-Core]

  - 7.26.4	RRM performance requirements			[NR_XR_Ph3-Perf]

The issues for discussion are organized in topics which are as follows:

- Topic #1: RRM core requirements

  - Sub-topic 1-1 - Scenarios for measurement skipping

  - Sub-topic 1-2 – Measurements with gaps

  - Sub-topic 1-3 – XR skipping requirements impact

  - Sub-topic 1-4 – Additional signalling aspects

  - Sub-topic 1-5 – UE capabilities

  - Sub-topic 1-6 – CR handling

- Topic #2: RRM performance requirements

  - Sub-topic 2-1 – Workplan update

  - Sub-topic 2-2 – Test case list

  - Sub-topic 2-3 – Test case configuration

  - Sub-topic 2-4 – MG cancellation model

  - Sub-topic 2-5 – DCI aspects

The RAN4 #116 meeting is the last meeting planned for RRM core requirements, therefore, the recommended priority for discussion is as follows:

- Issue 2-1-1: Workplan

- Issue 1-1-1: UE power class

- Issue 1-6-1: Specification impact

- Issue 1-3-1: Principles for measurement delay calculation

- Issue 1-3-2: Formulas for measurement delay extension without DRX

- Issue 1-3-3: Formulas for measurement delay extension with DRX

- Issue 1-3-4: Formulas for measurement delay extension for inter-RAT

- Issue 1-3-5: Formulas for measurement delay extension of concurrent gaps

- Issue 1-3-6: Formulas for measurement delay extension of L1/L2 triggered mobility

- Issue 1-3-7: Conditions for measurement skipping

- Issue 1-2-1: Type 2 gaps

- Issue 1-3-8: Measurement gap configurations relevant to skipping

- Issue 1-4-1: Skipping indication on frequency domain

- Issue 1-5-1: UE capabilities for XR – tables

- Issue 2-2-1: Scenarios for the definition of the test cases

- Issue 2-2-2: Accuracy requirements

- Issue 2-2-3: Per UE and per FR gaps

- Issue 2-2-4: Type 1 and Type 2 gaps

- Issue 2-2-6: Test target

# Topic #1: RRM core requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509957 | Nokia, Qualcomm (Rapporteurs) | Rapporteur Inputs Document for information |
| R4-2509146 | China Telecom | RRM requirements on XR enhancements Proposal 1: Use option 2 as baseline of the formula for measurement delay extension, and option 1 can be further discussed and adopted if consensus can be reached. - Option 2: Max(200 ms, ( 8 + Nskip ) × Max(MGRP , SMTC period) ) × CSSFinter Proposal 2: For the formula for measurement delay extension with DRX, option 2 is also preferred. - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter Proposal 3: No additional conditions are to be defined for measurement skipping. |
| R4-2509421 | InterDigital Belgium. LLC | NR_XR_Ph3 remaining open issues Observation 1: The RAN1 agreed CR indicates that the DCI with the cancelation order applies to the earliest MG occasion that starts at least at 3ms or 5ms depending on UE capability. If a rule is to be defined for this case, then our proposal is: Proposal 1: If two non-colliding concurrent gaps are separated by less than “X ms”, a gap cancelation would apply for the earliest measurement gap occasion that respects the “X ms” distance from the DCI reception, but not to the second gap that is less than “X ms” from the first one. Observation 2: Option 2 requires additional constraints to limit the possible long delays. Proposal 2: Support Option 2 with some additional requirements that would limit the long delay issue: - Option 2: Max( 200 ms, ( 8 + Nskip ) x Max(MGRP , SMTC period) ) x CSSFinter Observation 3: If Option 2 is agreed then the same approach shall be followed for the DRX case. Proposal 3: Support Option 2: - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter Observation 4: A supplementary requirement would be beneficial to limit the measurements delay issues and ensure measurements performance. Proposal 4: Further discuss the following proposal presented in the last meeting: - The maximum number of extensions are PSSS/SSS detection; SSB index detection; Measurement period. No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms; FR1: 12 FR2: 1.5×Mpss/sss_sync; FR1: 5 FR2: 1.5×MSSB_index; FR1: 12 FR2: 1.5×Mmeas_period. DRX cycle ≤ 320 ms; FR1: 8 FR2: Mpss/sss_sync; FR1: 3 FR2: MSSB_index; FR1: 8 FR2: Mmeas_period. DRX cycle > 320 ms; FR1: 5 FR2: 0.5×Mpss/sss_sync; FR1: 2 FR2: 0.5× MSSB_index; FR1: 5 FR2: 0.5× Mmeas_period. - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement. - Upon exceeding Xind,gaps,max the UE shall restart the time index detection. - Upon exceeding Xmeas,max the UE shall restart the measurement. |
| R4-2509458 | Apple | Discussion of RRM impact of XR for NR Phase 3 Proposal 1: Formulas for measurement delay extension without DRX: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter Proposal 2: Formulas for measurement delay extension with DRX - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter Proposal 3: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. Proposal 4: max number of occasions cancelled during a measurement period shall be defined. Values from NR-U can be reused. |
| R4-2509702 | OPPO | RRM requirements for XR enhancements Proposal 1: For measurement delay extension, RAN4 to extend the L3 delays by adding the number of skipped MG occasions on top of the legacy number of measurement samples in the measurement period. Proposal 2: RAN4 to define maximum time separation between two non-skipped gaps for measurement. |
| R4-2509958 | Nokia | Discussion on XR ph3 RRM core requirements - Measurement gap cancelation applies for all power classes, except for PC6 (HST). RAN1 spec is already clear on the UE behaviour of 2 non-colliding gaps separated by less than X ms. - Follow RAN1 specification, i.e. the DCI command with gap cancelation applies for the first gap occasion happening at least X ms after the DCI command, if 2 non-colliding concurrent gaps are separated by less than X ms, the second gap can be cancelled if a DCI is received during the time window of X ms before the first gap occasion. Formula in Option 2 results in measurement delay of infinity (no requirements) when CSSF = 2 and the measurement gap cancelation ratio approaches 50%. Formula in Option 2 results in measurement delay of infinity (no requirements) when the measurement gap cancelation ratio approaches 100/CSSF %. Even with complex configuration of overlapping between frequency layers, it is possible to schedule measurements according to Option 1 of Issue 3-1. The requirements according to Option 1 of Issue 3-1 provide a margin of 1 to 4 gap periods even in complex overlapping scenarios with CSSF>1. - RAN4 to define measurement delay extension due to measurement gap occasion cancellation for inter-frequency measurements with gap as - Max(200 ms ´ CSSFinter, 8 ´ Max(MGRP , SMTC period) ´ CSSFinter + Ncancel ´ Max(MGRP , SMTC period)), where Ncancel is the number of canceled MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Alternative proposal - RAN4 to define measurement delay extension due to measurement cancelation for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tcancel) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tcancel) ms - Where - Tcancel = Ncancel ´ Max(MGRP , SMTC period) - Ncancel is the number of canceled MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Alternative 1: RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms ´ CSSFinter, ceil(8 ´ 1,5) ´ Max(MGRP , SMTC period, DRX cycle) ´ CSSFinter + Lcancel ´ Max(MGRP , SMTC period, DRX cycle)) DRX cycle > 320 ms - 8 ´ DRX cycle ´ CSSFinter + Lcancel ´ DRX cycle - Use this formula as example and follow similar approach for the other cases. - Alternative 2: - RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + Tcancel) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter + Tcancel) ms - Where - DRX cycle ≤ 320 ms, Tcancel = Lcancel ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tcancel = Lcancel ´ DRX cycle - Extend the measurement delay for Inter-RAT E-UTRAN as - TIdentify, E-UTRAN FDD, cancel=TIdentify, E-UTRAN FDD+ceil( LCancel×Kgap_EUTRA)×MGRP - TMeasure, E-UTRAN FDD, cancel=TMeasure, E-UTRAN FDD+ceil( LCancel×Kgap_EUTRA )×MGRP - TIdentify, E-UTRAN TDD, cancel=TIdentify, E-UTRAN TDD+ceil( LCancel×Kgap_EUTRA)×MGRP - TMeasure, E-UTRAN TDD, cancel=TMeasure, E-UTRAN TDD+ceil( LCancel×Kgap_EUTRA)×MGRP - RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as - Max(200 ms ´ CSSFinter, ceil( 8 * Kgap) ´ Max(MGRP , SMTC period) ´ CSSFinter + ceil( Lcancel ´ Kgap ) ´ Max(MGRP , SMTC period)), where Lcancel is the number of non-dropped cancelled MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Cancelation of measurement gap occasions related to a certain gap pattern does not affect the measurement delay for another configured gap pattern. Our simulation results show the benefit of using about 60% cancellation ratio. In at least some scenarios cancellation ratio can be increased to about 75% without significant mobility degradation. Measurement gaps can be configured by the network to receive reports on potential carrier aggregation candidates. Maximum number of cancellations depends on the deployment, e.g. number of layers, measurement gap configuration, deployment inter-site distance. - No conditions are needed for measurement gap occasion cancellation regarding maximum number of cancelled occasions per measurement period or related to MG configuration. - The measurement delay for L1/L2 triggered mobility is determined as follows: Max(Treport, Ceil(M * Kgap) ´ Max(MGRP, SSB period)) ´ CSSFinter + Ceil(Lcancel * Kgap) ´ Max(MGRP, SSB period)) RAN4 to agree the applicability of the MG cancellation indication to the frequency range for per-UE / per-FR gaps as provided in the Draft CR38.133, clause 9.1.y.3 in R4-2509959. |
| R4-2510149 | CMCC | Discussion on RRM core requirements XR (eXtended Reality) Proposal 1: for the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms: - If concurrent gaps are separated by less than X ms and X is 3ms, it is the colliding case, and MG cancelling DCI applies to the MG surviving the collision handling rules. - If concurrent gaps are separated by less than X ms and X is 5ms, MG cancelling DCI cancels the MG occasion which is the first occasion that satisfy the X ms from the cancelling indication reception. Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, and not considering Type 2 gaps, the measurement delay extension due to measurement skipping is proposed as following: - Max(200 ms, ( 8 + Nskip )´ Max(MGRP , SMTC period)) ´ CSSFinter |
| R4-2510326 | vivo | Further discussion on RRM requirements for XR Proposal 1: Cancellation of concurrent gaps applies after gap collision handling and regardless of gap distance. Proposal 2: Option 2 is used to define requirements for gap skipping for both DRX and non-DRX cases. Proposal 3: The number of skipped gap occasions is one per each DRX cycle in the delay requirements. Proposal 4: Define new Tinter1 for gap skipping in inter-RAT measurement requirements. Proposal 5: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified which should not be larger than UE capability reported via UAI. Proposal 6: Common understanding on per-FR/per-UE gap application of gap cancellation is also captured in RAN4 specification from RRM requirements perspective. Proposal 7: No restriction on UE power classes whether the requirements for gap cancellation are applicable or not. Proposal 8: UE feature for XR is revised in the Table. <Moderator> table replicated in UE capability issue for simplicity of the discussion |
| R4-2510601 | Huawei, HiSilicon | Discussion on RRM requirements for Rel-19 XR Observation 1: Based on RAN1 spec, it is clear which MG to be cancelled for non-colliding concurrent gaps. Proposal 1: Based on RAN1 agreements, it is clear which MG is to be cancelled for non-colliding concurrent gaps. Proposal 2: Use following formula to define the delay extension. The all candidate carriers using the MG shall be extended. - Option 2: - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - the delay shall be extended accordingly for all frequencies which are candidates to be measured using the gap. Proposal 3: Clarify in RAN4 spec about the applicable frequency range as follows: The applicable range of measurement gap cancelling includes the scheduled Cell(s) and Cell(s) in the same frequency range when UE is configured with per-FR gap; otherwise, applicable range of measurement gap cancelling applies to both frequency ranges. |
| R4-2510686 | ZTE Corporation, Sanechips | Discussion on RRM aspects of R19 XR Proposal 1: For the gap canceling in the case of two non colliding concurrent gaps are separated by just larger than 4ms, NW could cancel any gap via different DCI transmission instance, the FFS part could be addressed. Proposal 2: RAN4 to define the measurement delay extension for inter-frequency measurement with gap without DRX as: Max(200 ms´ CSSFinter, 8×Max(MGRP, SMTC period)×CSSFinter + Nskip× Max(MGRP , SMTC period)). Proposal 3: RAN4 to define the measurement delay extension for inter-frequency measurement with gap with DRX as: - If DRX cycle ≤ 320 ms, Max(200 ms´ CSSFinter, Ceil(8×1.5)×Max(MGRP, SMTC period, DRX cycle)×CSSFinter + Nskip× Max(MGRP, SMTC period, DRX cycle)) - Otherwise, 8 ´ DRX cycle ´ CSSFinter + Nskip ´ DRX cycle where Nskip is the number of skipped MG occasions in the measurement period. Proposal 4: If some optimization is considered regarding the condition of measurement skipping, Option 3 is preferred by us, i.e. define a maximum separation between two non-skipped gaps for measurement. - The maximum separation could be defined as: the maximum separation guarantees the two non-successive non-skipped gaps are covered by a single measurement period. Observation 1: RAN1 TP interpret the impact frequency range as “on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]”, but what is the “applicable range of the measurement gap occasion”, no further explanation and 38.133 is referred. RAN1 focuses on the which serving cell could transmit traffic at the skipped gap occasion, instead of which frequency layer’s measurement is impacted. Proposal 5: When UE receives gap skipping indication in DCI, - For per-UE gap, the gap skipping is valid for the measurements on any frequency layer during the canceled gap occasion; - For per-FR gap, the gap skipping is valid for the measurements on any frequency layer of the same FR during the canceled gap occasion. Proposal 6: The impact of gap skipping could be captured in L3 measurement with gap, L3 measurement without gap, L1 measurement with gap, L1 measurement without gap chapters, including the impact on the scheduling restriction. |
| R4-2510709 | NTT DOCOMO, INC. | Discussions on RRM requirements for XR Observation 1: For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not and to keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip. Proposal 1: Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Capped Delay = max(Original Delay, Delay Cap) where Original Delay = max(200 ms, (8 + Nskip) × max(MGRP, SMTC period)) × CSSFinter and Delay Cap is set to a reasonable upper bound (e.g., 500 ms) Proposal 2: With DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 ´ DRX cycle ´ CSSFinter + Nskip ´ DRX cycle |
| R4-2511152 | Ericsson | On RRM core requirements for XR - Proposal 1: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: Condition; FR1: TPSS/SSS_sync_inter. No DRX; Max(600 ms, (8+XPSS/SSS,gaps) ´ Max(MGRP, SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(600 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter. Condition NOTE1; FR2-1: TPSS/SSS_sync_inter. No DRX; Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) ´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(600 ms, Ceil(1.5 ´ (Mpss/sss_sync_inter + XPSS/SSS,gaps)) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (Mpss/sss_sync_inter+ XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter. - Proposal 2: The maximum numbers of extensions of inter-frequency cell detection period are for FR1: XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. FR2: XPSS/SSS,gaps,max=1.5´Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, XPSS/SSS,gaps,max=0.5 ´ Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. - Proposal 3: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded: Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement. - Proposal 4: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively: Condition NOTE1; FR1: TSSB_measurement_period_inter NOTE2. No DRX; Max(120 ms, (3+Xind,gaps)´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(120 ms, Ceil((3+Xind,gaps) ´ 1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (3+Xind,gaps)´ DRX cycle ´ CSSFinter. Condition; FR2-1: TSSB_measurement_period_inter. No DRX; Max(200 ms, (MSSB_index_inter + Xind,gaps) ´ Max(MGRP, SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil(1.5 ´ (MSSB_index_inter + Xind,gaps)) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (MSSB_index_inter + Xind,gaps) ´ DRX cycle ´ CSSFinter. - Proposal 5: The maximum numbers of extensions of inter-frequency SSB time index detection period are for FR1: Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms. FR2: Xind,gaps,max=1.5 ´ MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xind,gaps,max=0.5 ´ MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. - Proposal 6: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded: Upon exceeding Xind,gaps,max the UE shall restart the time index detection. - Proposal 7: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively: Condition NOTE1; FR1: TSSB_measurement_period_inter NOTE2. No DRX; Max(200 ms, (8 + Xmeas) ´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(200 ms, Ceil((8 + Xmeas) ´ 1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (8 + Xmeas) ´ DRX cycle ´ CSSFinter. Condition; FR2-1: TSSB_measurement_period_inter. No DRX; Max(400 ms, (Mmeas_period_inter + Xmeas) ´ Max(MGRP , SMTC period)) ´ CSSFinter. DRX cycle ≤ 320 ms; Max(400 ms, Ceil(1.5 ´ (Mmeas_period_inter + Xmeas) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter. DRX cycle > 320 ms; (Mmeas_period_inter + Xmeas) ´ DRX cycle ´ CSSFinter. - Proposal 8: The maximum numbers of extensions of inter-frequency cell detection period are for FR1: XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. FR2: Xmeas=1.5 ´ Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, Xmeas=0.5 ´ Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms. - Proposal 9: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded: Upon exceeding Xmeas,max the UE shall restart the measurement. - Proposal 10: RAN4 to discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL, e.g., MGL≥3 ms, - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays. - Proposal 11: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.: Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. - Proposal 12: The requirements for measurement gap cancellation apply for UE power class 3. |
| R4-2511216 | MediaTek inc. | Discussion on RRM requirements for XR Proposal 1: Further information and discussion on the proposal of limiting the scope to power class 3. Proposal 2: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic. Proposal 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period)) × CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 4: RAN4 to define measurement delay extension with DRX cycle <= 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) × Max(MGRP , SMTC period, DRX cycle)) × CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 5: RAN4 to define measurement delay extension with DRX cycle > 320ms due to measurement skipping for inter-frequency measurements with gap as: ‘Ceil ((8 + Nskip) ´ Kgap) ´ DRX cycle ´ CSSFinter’, where Nskip is the number of skipped MG occasions in the measurement period. Proposal 6: RAN4 shall define the max number of occasions cancelled during a measurement period to be 50%. |
| R4-2511635 | Qualcomm Incorporated | RRM core requirements for XR enhancements Observation 1: RAN1 specification already clarifies that a measurement gap cancellation field in a DCI is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec. Proposal 1: MG cancelling DCI also applies to non-colliding concurrent MGs separated by <X ms and no spec change is needed in RAN4. Proposal 2: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples. Proposal 3: Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc. |
| R4-2509459 | Apple | Inter-RAT measurements E-UTRAN TDD measurements |
| R4-2509959 | Nokia | Draft CR38.133 Introduction of measurement gap cancelation |
| R4-2510325 | vivo | Draft CR on NR inter-frequency L1 measurement for XR |
| R4-2510602 | Huawei, HiSilicon | DraftCR on Inter-RAT measurements E-UTRAN FDD for Rel-19 XR |
| R4-2510727 | ZTE Corporation, Sanechips | Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR |
| R4-2511151 | Ericsson | draftCR38133 Inter-frequency measurement requirements with XR |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 - Scenarios for measurement skipping

Sub-topic description:

Open issues and candidate options before meeting:

#### Issue 1-1-1: UE power class

- Proposals

  - Option 1 (Ericsson): The requirements for measurement gap cancellation apply for UE power class 3.

  - Option 2 (Nokia): Measurement gap cancelation applies for all power classes, except for PC6 (HST).

  - Option 3 (vivo): No restriction on UE power classes whether the requirements for gap cancellation are applicable or not.

  - Option 4 (MediaTek): Further information and discussion on the proposal of limiting the scope to power class 3.

- Recommended WF

  - Please discuss whether there should be restriction to power class for XR MG cancellation

### Sub-topic 1-2 – Measurements with gaps

#### Issue 1-2-1: Type 2 gaps

- Proposals

  - Proposal 1 (Huawei): Based on RAN1 agreements, it is clear which MG is to be cancelled for non-colliding concurrent gaps.

  - Proposal 2 (Nokia): Follow RAN1 specification, i.e.

    - the DCI command with gap cancelation applies for the first gap occasion happening at least X ms after the DCI command,

    - if 2 non-colliding concurrent gaps are separated by less than X ms, the second gap can be cancelled if a DCI is received during the time window of X ms before the first gap occasion.

  - Proposal 3 (Qualcomm): MG cancelling DCI also applies to non-colliding concurrent MGs separated by <X ms and no spec change is needed in RAN4.

  - Proposal 4 (Interdigital): If two non-colliding concurrent gaps are separated by less than “X ms”, a gap cancelation would apply for the earliest measurement gap occasion that respects the “X ms” distance from the DCI reception, but not to the second gap that is less than “X ms” from the first one.

  - Proposal 5 (CMCC): for the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms:

    - If concurrent gaps are separated by less than X ms and X is 3ms, it is the colliding case, and MG cancelling DCI applies to the MG surviving the collision handling rules.

    - If concurrent gaps are separated by less than X ms and X is 5ms, MG cancelling DCI cancels the MG occasion which is the first occasion that satisfy the X ms from the cancelling indication reception.

  - Proposal 6 (ZTE): For the gap canceling in the case of two non colliding concurrent gaps are separated by just larger than 4ms, NW could cancel any gap via different DCI transmission instance, the FFS part could be addressed.

  - Proposal 7 (vivo): Cancellation of concurrent gaps applies after gap collision handling and regardless of gap distance.

- Recommended WF

  - Companies need to discuss expected behaviour on that situation (Picture from ZTE paper):

  - Please have in mind existing RAN1 agreements/specification.

### Sub-topic 1-3 – XR skipping requirements impact

Sub-topic description

Open issues and candidate options before meeting:

#### Issue 1-3-1: Principles for measurement delay calculation

- Background information previous agreements:

  - Agreement from RAN4 #113 R4-2420103

## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay

  - Agreement from RAN4 #114 R4-2502700

## Issue 3-1: Principles for measurement delay calculation <Agreement>: Follow NR-U-like approach for designing the formulas for measurement delay extension.

- Proposals

  - Proposal 1 (MediaTek): RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

  - Proposal 2 (Qualcomm): For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

  - Proposal 3 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

- Recommended WF

  - Proposal 1 is already agreed. Please check if further discussion is needed on this issue to guide the discussion on the design of the measurement delay formulas.

#### Issue 1-3-2: Formulas for measurement delay extension without DRX

- Proposals

  - Option 1 (Nokia, ZTE): Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Lcancel × Max(MGRP , SMTC period)),

  - Option 2 (China Telecom, Apple, CMCC, Huawei, Ericsson, MediaTek): Max( 200 ms, ( 8 + Lcancel) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

  - Option 3 (Nokia):

- Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tcancel) ms

- Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tcancel) ms

    - where Tcancel = Lcancel ´ Max(MGRP , SMTC period)

  - Option 4 (Interdigital): Support Option 2 with some additional requirements that would limit the long delay issue

  - Option 5 (OPPO): For measurement delay extension, RAN4 to extend the L3 delays by adding the number of skipped MG occasions on top of the legacy number of measurement samples in the measurement period.

  - Option 6 (NTT DOCOMO): Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Capped Delay = max(Original Delay, Delay Cap)

    - where Original Delay = max(200 ms, (8 + Nskip) × max(MGRP, SMTC period)) × CSSFinter

    - and Delay Cap is set to a reasonable upper bound (e.g., 500 ms)

- Recommended WF

  - Discuss the proposals above, where Lcancel is the number of cancelled MG occasions in the measurement period

#### Issue 1-3-3: Formulas for measurement delay extension with DRX

- Proposals

  - Option 1 (Nokia, ZTE, NTT DOCOMO): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - DRX cycle ≤ 320 ms

      - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Lcancel × Max(MGRP, SMTC period, DRX cycle) )

    - DRX cycle > 320 ms

      - 8 ´ DRX cycle ´ CSSFinter + Lcancel ´ DRX cycle

  - Option 2 (China Telecom, Interdigital, Apple, Ericsson, MediaTek): Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+ Lcancel) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+ Lcancel) ´ DRX cycle ´ CSSFinter

  - Option 3 (Nokia): RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + Tcancel) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter + Tcancel) ms

    - Where

      - DRX cycle ≤ 320 ms, Tcancel = Lcancel ´ Max(MGRP , SMTC period, DRX cycle)

      - DRX cycle > 320 ms, Tcancel = Lcancel ´ DRX cycle

- Recommended WF

  - Outcome of Issue 1-3-1 can be reused, but further discussion on Option 4 is needed.

#### Issue 1-3-4: Formulas for measurement delay extension for inter-RAT

- Proposals

  - Option 1 (Nokia): Extend the measurement delay for Inter-RAT E-UTRAN as

    - TIdentify,E-UTRAN FDD,cancel = TIdentify,E-UTRAN FDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

    - TMeasure,E-UTRAN FDD,cancel = TMeasure,E-UTRAN FDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

    - TIdentify,E-UTRAN TDD,cancel = TIdentify,E-UTRAN TDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

    - TMeasure,E-UTRAN TDD,cancel = TMeasure,E-UTRAN TDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

- Recommended WF

  - Discussion needed.

#### Issue 1-3-5: Formulas for measurement delay extension of concurrent gaps

- Proposals

  - Option 1 (Nokia): RAN4 to define measurement delay extension due to occasion cancelation for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, ceil( 8 * Kgap)   × Max(MGRP , SMTC period) × CSSFinter + ceil( Lcancel ×Kgap ) × Max(MGRP , SMTC period)),

      - where Lcancel is the number of non-dropped cancelled MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

    - Cancelation of measurement gap occasions related to a certain gap pattern does not affect the measurement delay for another configured gap pattern.

- Recommended WF

  - Discussion needed

#### Issue 1-3-6: Formulas for measurement delay extension of L1/L2 triggered mobility

- Proposals

  - Option 1 (Nokia): The measurement delay for L1/L2 triggered mobility is determined as follows: Max(Treport, Ceil(M * Kgap) × Max(MGRP, SSB period)) × CSSFinter + Ceil(Lcancel × Kgap) × Max(MGRP, SSB period))

- Recommended WF

  - Discussion needed

#### Issue 1-3-7: Conditions for measurement skipping

- Proposals from companies

  - Proposals for no conditions

    - Proposal 1 (China Telecom): No conditions are to be defined for measurement skipping.

    - Proposal 2 (Nokia): No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration

    - Proposal 3 (Qualcomm): Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

  - Proposals on max number of occasions

    - Proposal 4 (MediaTek): RAN4 shall define the max number of occasions cancelled during a measurement period to be 50%.

    - Proposal 5 (vivo): For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified which should not be larger than UE capability reported via UAI.

    - Proposal 6 (Apple): max number of occasions cancelled during a measurement period shall be defined. Values from NR-U can be reused.

    - Proposal 7 (Ericsson, Interdigital): The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

  - Proposals on maximum separation between two occasions shall be

    - Proposal 8 (Ericsson): Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

      - Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

    - Proposal 9 (OPPO): RAN4 to define maximum time separation between two non-skipped gaps for measurement.

    - Proposal 10 (ZTE): The maximum separation could be defined as: the maximum separation guarantees the two non-successive non-skipped gaps are covered by a single measurement period.

- Proposals merged by the Moderator

  - Option 1: No conditions are defined

  - Option 2: Define the maximum number of gaps skipped during the measurement period.

  - Option 3: Define a maximum separation between two non-skipped gaps for measurement

  - Option 4: Define the max number of occasions cancelled during a measurement period as a percentage of 50%.

- Recommended WF

  - Discussion on the conditions is needed. If the group agrees that conditions are necessary, we can further discuss based on one of the detailed proposals.

#### Issue 1-3-8: Measurement gap configurations relevant to skipping

- Options

  - Option 1 (Ericsson): RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL, e.g., MGL≥3 ms,

    - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Recommended WF

  - Please discuss if there is need for limiting which MG configurations can be skipped.

### Sub-topic 1-4 – Additional signalling aspects

#### Issue 1-4-1: Skipping indication on frequency domain

- Background information

  - Agreement from R4-2508285

## Issue 5-2: Skipping indication on frequency domain <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

- Proposals

  - Proposal 1 (Nokia): RAN4 to agree the applicability of the MG cancellation indication to the frequency range for per-UE / per-FR gaps as provided in the Draft CR38.133, clause 9.1.y.3 in R4-2509959.

  - Proposal 2 (vivo): Common understanding on per-FR/per-UE gap application of gap cancellation is also captured in RAN4 specification from RRM requirements perspective.

  - Proposal 3 (Huawei): Clarify in RAN4 spec about the applicable frequency range as follows:

    - The applicable range of measurement gap cancelling includes the scheduled Cell(s) and Cell(s) in the same frequency range when UE is configured with per-FR gap; otherwise, applicable range of measurement gap cancelling applies to both frequency ranges.

  - Proposal 4 (ZTE): When UE receives gap skipping indication in DCI,

    - For per-UE gap, the gap skipping is valid for the measurements on any frequency layer during the canceled gap occasion;

    - For per-FR gap, the gap skipping is valid for the measurements on any frequency layer of the same FR during the canceled gap occasion.

- Recommended WF

  - Discussion needed.

### Sub-topic 1-5 – UE capabilities

#### Issue 1-5-1: UE capabilities for XR - tables

- Proposals

  - Option 1 (vivo):

| 57.NR_XR_Ph3 | 53-1 | Recommended ratio of gap occasions for cancelation | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation | 64-1 | Yes | N/A | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Recommended WF

  - Discuss whether Option 1 is agreeable

### Sub-topic 1-6 – CR handling

#### Issue 1-6-1: Specification impact

- Background information:

  - Agreement from RAN4 #114 R4-2502700

Issue 1-3: L3 Measurements without gaps with scheduling restrictions Issue 1-4: L1 measurements with scheduling restrictions and procedures Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Proposals

  - Option 1 (ZTE): The impact of gap skipping could be captured in L3 measurement with gap, L3 measurement without gap, L1 measurement with gap, L1 measurement without gap chapters, including the impact on the scheduling restriction.

- Recommended WF

  - Please consider previous agreements and discuss whether RAN4 needs to capture impact on L3 and L1 without gaps due to XR MG cancellation.

# Topic #2: RRM performance requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509960 | Nokia, Qualcomm (Rapporteurs) | Workplan for XR ph3 RRM performance requirements |
| R4-2509703 | OPPO | Discussion on RRM performance requirements for XR Proposal 1: Consider both per-FR gap and per-UE gap in the test case. Proposal 2: Consider Rel-17 concurrent gaps in the test case. Proposal 3: Define test cases for: - Intra-frequency measurements with gaps - Inter-frequency measurements with gaps - Inter-RAT E-UTRAN FDD measurements - Inter-RAT E-UTRAN TDD measurements - Inter-frequency L1-RSRP with MG Proposal 4: Define the test cases for the cases without DRX and with DRX. Proposal 5: UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case. |
| R4-2509961 | Nokia | Discussion on XR ph3 RRM performance requirements - RAN4 to discuss which test cases to develop based on the listed test cases in the following table: <moderator> table omitted here and added on main issue - RAN4 to focus on test cases for per-UE MGs. - Test cases without DRX should have higher priority than with DRX. - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG. - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority. - Focus on non-DRX test cases for Type 2 MGs. - For Type 2 MGs test cases, include 2 gaps with partial overlap. - For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e. - Low priority MG pattern #1 with MGRP=40ms, offset=39ms - High priority MG pattern #2 with MGRP=80ms, offset=4ms - NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA. Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset - Measurement gap cancelation ratio is configured independently for each configured gap pattern. - For Type 2 MGs test cases, configure cancelation ratio as: - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0% - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset - The following aspects should be evaluated in XR test cases: - Measurement delay extension due to skipping Capability of canceling measurement gap within Toffset of 5ms or 3ms depending on UE capability Reception of PDCCH/PDSCH during canceled measurement gap occasions Transmission of PUCCH/PUSCH during canceled measurement gap occasions |
| R4-2510150 | CMCC | Discussion on RRM performance requirements for XR (eXtended Reality) Proposal 1: for measurement accuracy requirements, it is proposed to confirm previous agreements to reuse existing measurement accuracy requirements for gap cancellation. Proposal 2: it is proposed to define test cases for NR-SA. Proposal 3: it is proposed to define test cases for FR1 and FR2-1. Proposal 4: it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement. Proposal 5: it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case). Proposal 6: it is proposed to define test cases for both 3ms offset and 5ms offset. And applibility rules can be considered, e.g. the UE supporing 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline. |
| R4-2510327 | vivo | On RRM performance requirements for XR Proposal 1: Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements when gap cancellation is enabled. Proposal 2: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation. Proposal 3: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Proposal 4: Time offset for gap cancellation indication is 5ms in the tests. Proposal 5: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. |
| R4-2510603 | Huawei, HiSilicon | Discussion on RRM performance requirements for Rel-19 XR Proposal 1: RAN4 to define test cases for MG skipping to verify following behaviour and requirements: - The MG is skipped accordingly based on gNB DCI, and UE is scheduled during MG occasions and transmit ACK accordingly. - UE shall meet the extended measurement delay requirements. Proposal 2: The processing time 3ms/5ms shall be verified in the test cases. If UE supporting additional 3ms processing capability, 3ms shall be verified. |
| R4-2510690 | ZTE Corporation, Sanechips | Discussion on Perf part of R19 XR Proposal 1: Both FR1 NR SA and FR2-1 NR SA should be verified in the test. Proposal 2: The skipping in both Type-1 MG and Type-2 MG should be verified in different tests. Proposal 3: The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified: - Intra-frequency measurement with gap - Inter-frequency measurement with gap - EUTRAN measurement with gap - L1 inter-frequency measurement with gap Proposal 4: The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well. Proposal 5: The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with out test motivation. |
| R4-2511153 | Ericsson | On RRM performance issues for XR - Proposal 1: - RAN4 confirms the earlier agreement: - Reuse existing measurement accuracy requirements for UE under XR operation. - Proposal 2: A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation. - Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both. - Proposal 4: Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG. - Proposal 5: RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19. - Proposal 6: Test cases are specified only for per-UE MGs. - Proposal 7: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1. |
| R4-2511636 | Qualcomm Incorporated | RRM performance requirements for XR enhancements Proposal 1: RAN4 to consider shortlisting test cases for DCI-based Measurement Gap (MG) cancellation from the table below for FR1, aiming to cover representative scenarios across the impacted measurement types, while maintaining sufficient test coverage. <Moderator> table omitted here and added on main issue Proposal 2: RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation. Proposal 3: One additional test case for concurrent MG configurations may also be introduced. Proposal 4: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 – Workplan update

#### Issue 2-1-1: Workplan

- Background information

|  |  |  | RAN | R4RD | RAN | R4RD | R4RD | RAN | R4RD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WI code | WI or SI | Target | 108 Jun.25 | 116 | 109 Sep.25 REL-19 freeze | 116bis | 117 | 110 Dec.25 | 118 |
| NR_XR_Ph3-Core | WI | Sep.25 |  | 0,25 |  |  |  |  |  |
| NR_XR_Ph3-Perf | WI | Mar.26 |  | 0,25 |  | 0,5 | 0,5 |  | 0,5 |

- Proposals

  - Option 1 (Nokia, Qualcomm): Approve workplan update on R4-2509960

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4 #116 - August 2025 | - Approval of workplan - Initial discussion on the list of test cases |
| RAN4 #116-bis - October 2025 | - Conclusions on the list of test cases - Discussion on test parameters - Agreement of CR work split |
| RAN4 #117 - November 2025 | - Discussion on test parameters - First Draft CRs for RRM core requirements. - Draft CRs to 38.133 expected - First Big Draft CR endorsed |
| RAN4 #118 - February 2026 | - Discussion on remaining open issues for RRM perf. - Revision of CRs for RRM perf. requirements. - Approval of Big CR |

- Recommended WF

  - TBA

### Sub-topic 2-2 – Test case list

#### Issue 2-2-1: Scenarios for the definition of the test cases

- Proposals

  - Proposal 1 (CMCC): it is proposed to define test cases for NR-SA.

  - Proposal 2 (CMCC): it is proposed to define test cases for FR1 and FR2-1.

- Recommended WF:

  - Discuss if Proposals 1 and 2 can be agreed.

#### Issue 2-2-2: Accuracy requirements

- Background information

  - Agreement from RAN4 #113 R4-2420103

## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay

- Proposals

  - Proposal 1 (CMCC): for measurement accuracy requirements, it is proposed to confirm previous agreements to reuse existing measurement accuracy requirements for gap cancellation.

  - Proposal 2 (Ericsson): RAN4 confirms the earlier agreement:

    - Reuse existing measurement accuracy requirements for UE under XR operation.

  - Proposal 3 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

- Recommended WF:

  - Discuss whether confirmation of previous agreement is needed, and whether accuracy test case should be defined.

#### Issue 2-2-3: Per UE and per FR gaps

- Company Proposals

  - Proposal 1 (OPPO): Consider both per-FR gap and per-UE gap in the test case.

  - Proposal 2 (Nokia): RAN4 to focus on test cases for per-UE MGs.

  - Proposal 3 (Ericsson): Test cases are specified only for per-UE MGs.

  - Proposal 4 (Qualcomm): RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation.

- Proposals merged by the moderator

  - Option 1: Test cases are specified only for per-UE MGs

  - Option 2: Test cases are specified for both per-UE and per-FR MGs

- Recommended WF

  - Discussion needed

#### Issue 2-2-4: Type 1 and Type 2 gaps

- Company Proposals

  - Proposal 1 (OPPO): Consider Rel-17 concurrent gaps in the test case.

  - Proposal 2 (CMCC): it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case).

  - Proposal 3 (ZTE): The skipping in both Type-1 MG and Type-2 MG should be verified in different tests.

  - Proposal 4 (Ericsson): Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

  - Proposal 5 (Ericsson): RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

  - Proposal 6 (Qualcomm): One additional test case for concurrent MG configurations may also be introduced.

- Merged proposals by the moderator

  - Option 1: Test cases are defined for both Type 1 and Type 2 gaps

  - Option 2: Test cases are defined only for Type 1 gaps

- Recommended WF:

  - Discussion needed.

#### Issue 2-2-5: DRX proposals

- Company proposals:

  - Proposal 1 (OPPO): Define the test cases for the cases without DRX and with DRX.

  - Proposal 2 (Nokia): Test cases without DRX should have higher priority than with DRX.

    - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG.

    - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority.

    - Focus on non-DRX test cases for Type 2 MGs.

  - Proposal 3 (Ericsson): Each type of test case is specified for either DRX or non-DRX but not both.

- Recommended WF:

  - Discuss the proposals having in mind the test coverage and workload.

#### Issue 2-2-6: Test target

- Company proposals

  - Proposal 1 (OPPO): UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

  - Proposal 2 (Nokia): The following aspects should be evaluated in XR test cases:

    - Measurement delay extension due to skipping

    - Capability of canceling measurement gap within Toffset of 5ms or 3ms depending on UE capability

    - Reception of PDCCH/PDSCH during canceled measurement gap occasions

    - Transmission of PUCCH/PUSCH during canceled measurement gap occasions

  - Proposal 3 (Huawei): RAN4 to define test cases for MG skipping to verify following behaviour and requirements:

    - The MG is skipped accordingly based on gNB DCI, and UE is scheduled during MG occasions and transmit ACK accordingly.

    - UE shall meet the extended measurement delay requirements.

  - Proposal 4 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

  - Proposal 5 (ZTE): The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with other test motivation.

- Proposals merged by the moderator:

  - RRM test cases should verify

    - Proposal 1: accuracy requirements

    - Proposal 2: delay extension with gap cancellation

    - Proposal 3: Reception/transmission without scheduling restrictions during cancelled gap occasions

    - Proposal 4: The UE processing timeline of receiving the DCI-based MG skipping indication

- Recommended WF:

  - Discuss which proposals are agreeable.

#### Issue 2-2-7: List of test cases

- Company proposals

  - Proposal 1 (OPPO): Define test cases for:

    - Intra-frequency measurements with gaps

    - Inter-frequency measurements with gaps

    - Inter-RAT E-UTRAN FDD measurements

    - Inter-RAT E-UTRAN TDD measurements

    - Inter-frequency L1-RSRP with MG

  - Proposal 4 (CMCC): it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement.

  - Proposal 5 (vivo): Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements when gap cancellation is enabled.

  - Proposal 6 (ZTE): The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified:

    - Intra-frequency measurement with gap

    - Inter-frequency measurement with gap

    - EUTRAN measurement with gap

    - L1 inter-frequency measurement with gap

  - Proposal 7 (Ericsson): A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

  - Proposal 8 (Qualcomm): RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

  - Proposal 9 (ZTE): The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well.

- Proposals merged by the moderator

  - Define test cases for

    - Proposal 1: Intra-frequency measurements with gaps

    - Proposal 2: Inter-frequency measurements with gaps

    - Proposal 3: Inter-RAT E-UTRAN FDD measurements

    - Proposal 4: Inter-RAT E-UTRAN TDD measurements

    - Proposal 5: Inter-frequency L1-RSRP with MG

    - Proposal 6: L3 and L1 measurements without gaps

- Recommended WF

  - Discuss the merged proposals 1 to 5, and whether reduced number of tests are defined for each of the proposals.

#### Issue 2-2-8: Complete list of test cases for discussion

- Proposals

  - Proposal 1: Evaluate which test cases from the list below to be developed

| Gap type | Intra/inter/inter-RAT | Frequency range | DRX | Included/not included |
| --- | --- | --- | --- | --- |
| Type 1 gap per FR | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 1 gap per UE | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 2 gaps per FR | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 2 gaps per UE | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| LTM measurements with gaps | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |

- Recommended WF:

  - Discuss the proposals. Use the outcome of the previous issues to delete or add items to the table.

### Sub-topic 2-3 – Test case configuration

#### Issue 2-3-1: Type 2 test configurations

- Proposals

  - Proposal 1 (Nokia): For Type 2 MGs test cases, include 2 gaps with partial overlap.

  - Proposal 2 (Nokia): For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e.

    - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

    - High priority MG pattern #2 with MGRP=80ms, offset=4ms

- Recommended WF:

  - Discuss the proposals

### Sub-topic 2-4 – MG cancellation model

#### Issue 2-4-1: Cancelation model for XR test cases

- Proposals

  - Proposal 1 (Nokia): Measurement gap cancelation is determined randomly by the test equipment.

  - Proposal 2 (Nokia): For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern.

- Recommended WF

  - Discuss the proposals

#### Issue 2-4-2: Cancelation ratio

- Proposals

  - Proposal 1 (Nokia):

    - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

    - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

  - Proposal 2 (Nokia): For Type 2 MGs test cases, configure cancelation ratio as:

    - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%

    - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset

  - Proposal 3 (vivo): Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

- Recommended WF:

  - Discuss the proposals

### Sub-topic 2-5 – DCI aspects

#### Issue 2-5-1: Toffset between DCI and start of MG

- Proposals

  - Proposal 1 (CMCC): it is proposed to define test cases for both 3ms offset and 5ms offset. And applibility rules can be considered, e.g. the UE supporing 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline.

  - Proposal 2 (vivo): Time offset for gap cancellation indication is 5ms in the tests.

  - Proposal 3 (Huawei): The processing time 3ms/5ms shall be verified in the test cases. If UE supporting additional 3ms processing capability, 3ms shall be verified.

- Recommended WF

  - Discuss the proposals

#### Issue 2-5-2: Types of DCI with cancellation indication

- Proposals

  - Proposal 2 (vivo): In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

- Recommended WF

  - Discuss the proposal

#### Issue 2-5-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

- Proposals

  - Proposal 1 (vivo): RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

- Recommended WF

  - Discuss the proposal