| 3GPP TR 38.746 V0.5.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	TR Maintenance	7

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL	8

5.10	CA_nX-nY	8

5.10.1	Configurations	8

5.10.2	Maximum output power	8

5.10.3	REFSENS requirements	8

5.10.3.1	Power class 2 case a	8

5.10.3.2	Power class 2 case b	8

5.10.3.3	Power class 2 case c	8

5.10.3.4	Power class 1.5 for single uplink nX	8

5.10.3.5	Power class 1.5 for single uplink nY	8

5.10.4	∆TIB and ∆RIB values	9

6	High Power UE CA of 3 bands DL and 1 or 2 bands UL	9

6.9	CA_nX-nY-nZ	9

6.9.1	Configurations	9

6.9.2	Maximum output power	9

6.9.3	REFSENS requirements	9

6.9.3.1	Power class 2 case a	9

6.9.4	∆TIB and ∆RIB values	9

7	Power class 2 CA with SUL	10

7.x	CA_nW-nX_SUL_nY-nZ	10

7.x.1	Configurations	10

7.x.2	Maximum output power	10

7.x.3	REFSENS requirements	10

7.x.4	∆TIB and ∆RIB values	10

Annex <A> (informative): Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is one of four technical reports for High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s) under Rel-19 timeframe. The objective is to gather the relevant background information and studies in order to complete the band-combination specific requirements for the newly requested band combinations for power class 2 and power class 1.5 UE. Table 1-1 lists the band combinations covered by this TR with three power class cases for uplink.

Table 1-1 High power UE (power class 1.5 or 2) inter-band CA/DC and SUL band combinations

| # | Band combination List | Power class cases for uplink |
| --- | --- | --- |
| 1 | High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with/without Supplementary uplink (SUL) with high power on TDD band(s) Note: Including PC3 FDD/TDD+ PC3 TDD case | PC2 inter-band CA/DC: 1UL(TDD): PC2 on TDD band 2UL (TDD+TDD): PC3 or PC2 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC3 or PC2 on TDD band |
|  |  | PC1.5 inter-band CA/DC: 1UL(TDD): PC 1.5 on TDD band 2UL (TDD+TDD): PC3, PC2 or PC1.5 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC1.5 on TDD band |
|  |  | PC2 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC2 on TDD band PC1.5 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC1.5 on TDD band NUL = Normal Uplink in contrast to SUL. |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-241679: “Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

SUL 				Supplementary uplink

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

# 4	Background

At 3GPP RAN#104 meeting, a basket Work Item on “High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)” [2] was approved for Rel-19. There are four objectives of the core part of the basket work item. The objectives covered by this TR are as follows:

Specify the band-combination specific RF requirements for all band combinations cases captured in table 1-1. The band combinations are defined in the attached excel file of this WI.

The requirements that need to analyse and specify include

  - Maximum output power, Tx power tolerance and A-MPR (Additional Maximum Power Reduction) requirements if needed

  - Analysing combinations that have self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

  - Other additional impact to the requirements due to the high power on UL, if necessary

The present document is a technical report for this basket Work Item.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

# 5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

## 5.X	CA_nX-nY

### 5.X.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY | nX8,9,13,14, nY8,9,13,14, CA_nX-nY8,9,13,14 | nX |  |  |
|  |  | nY |  |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination … NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.X.2	Maximum output power

<Editor’s note: In table 5.X.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.X.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 5.X.2-2 UE Power Class 1.5 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |
|  | Case c | 29dBm | 29dBm | 23dBm |
|  | Case d | 29dBm | 26dBm | 29dBm |
|  | Case e | 29dBm | 29dBm | 26dBm |

### 5.X.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

#### 5.X.3.1	Power class 2 case a

#### 5.X.3.2	Power class 2 case b

#### 5.X.3.3	Power class 2 case c

#### 5.X.3.4	Power class 1.5 case a

#### 5.X.3.5	Power class 1.5 case b

#### 5.X.3.6	Power class 1.5 case c

#### 5.X.3.7	Power class 1.5 case d

#### 5.X.3.8	Power class 1.5 case e

#### 5.X.3.9	Power class 1.5 for single uplink nX

#### 5.X.3.10	Power class 1.5 for single uplink nY

### 5.X.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 5.1	CA_n1-n78

### 5.1.1	Operating bands for CA

Table 5.1.1-1:  CA band combination of band n1+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n1-n78 | n1, n78 |

### 5.1.2	Maximum output power for inter-band CA

Table 5.1.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.1.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.1.3	MSD scenario studies

Table 5.13-1 and 5.1.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n1-n78.

Table 5.1.3-1: MSD scenarios for CA_n1-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | 3 * n1 – n78 | n1 | Yes |

Table 5.1.3-2: MSD scenarios for CA_n1-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | 3 * f n1 – f n78 | n1 | No |

### 5.1.4	REFSENS requirements

#### 5.1.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.1.4.2	REFSENS requirements for total power class 1.5

- The MSD due to IMD4 for PC1.5 3Tx are specified in the following table. The IMD4 MSD values for CA_n1-n78 and CA_n2-n77 are the same. Therefore, the CA_n1-n78 PC1.5 MSD values are reused from CA_n2-n77 PC1.5 MSD values.

Table 5.1.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 |  | 1950 | 5 | 25 | 2140 | 26.4 | FDD | IMD4 |
|  | n78 | 3710 |  | 10 | 50 | 3710 | N/A | TDD | N/A |

- Cross band MSD is defined for CA_n2-n77 therefore it is also needed for CA_n1-n78 since band n1 is even closer than band n2. Same MSD value as cor CA_n2-n77 are used.

Table 5.1.4.2-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n1 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 2.7 | >ACLR2 |

## 5.2	CA_n3-n78

### 5.2.1	Operating bands for CA

Table 5.2.1-1:  CA band combination of band n3+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n3-n78 | n3, n78 |

### 5.2.2	Maximum output power for inter-band CA

Table 5.2.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.2.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.2.3	MSD scenario studies

Table 5.10.3-1 and 5.10.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n3-n78.

Table 5.2.3-1: MSD scenarios for CA_n3-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL2) | n3 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | n78 – n3 | n3 | Yes |
| IMD4 | 3 * n3 – n78 | n3 | Yes |

Table 5.2.3-2: MSD scenarios for CA_n3-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL2) | n3 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | f n78 – f n3 | n3 | No |
| IMD4 | 3 * f n3 – f n78 | n3 | No |

### 5.2.4	REFSENS requirements

#### 5.2.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD2 has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.2.4.2	REFSENS requirements for total power class 1.5

- The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

- The MSD due to IMD2 and IMD4 for PC1.5 3Tx are specified in the following table. The IMD2 and IMD4 MSD values for CA_n3-n78 and CA_n2-n77 are the same. Therefore, the CA_n3-n78 PC1.5 MSD values are reused from CA_n2-n77 PC1.5 MSD values.

Table 5.2.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78 | n3 |  | 1750 | 5 | 25 | 1845 | 35.2 | FDD | IMD2 |
|  | n78 |  | 3595 | 10 | 50 | 3595 | N/A | TDD | N/A |
|  | n3 | 1770 |  | 5 | 25 | 1865 | 26.4 | FDD | IMD4 |
|  | n78 | 3445 |  | 10 | 50 | 3445 | N/A | TDD | N/A |

Harmonic mixing defined in Table 5.10.4.2-2 below.

Table 5.2.4.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

## 5.3	CA_n7-n78

### 5.3.1	Operating bands for CA

Table 5.3.1-1:  CA band combination of band n7+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n7-n78 | n7, n78 |

### 5.3.2	Maximum output power for inter-band CA

Table 5.3.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.3.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.3.3	MSD scenario studies

Table 5.10.3-1 and 5.10.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n7-n78.

Table 5.3.3-1: MSD scenarios for CA_n7-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n78 | n7 | Yes |
| IMD | N/A | N/A | N/A |

Table 5.3.3-2: MSD scenarios for CA_n7-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n78 | n7 | Yes |
| IMD | N/A | N/A | N/A |

### 5.3.4	REFSENS requirements

#### 5.3.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to cross band isolation has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.3.4.2	REFSENS requirements for total power class 1.5

- The 2Tx MSD due to cross band isolation is specified below with same MSD value as used for CA_n7-n78.

Table 5.3.4.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 9.0 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

## 5.4	CA_n28-n78

### 5.4.1	Operating bands for CA

Table 5.4.1-1:  CA band combination of band n28+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n28-n78 | n28, n78 |

### 5.4.2	Maximum output power for inter-band CA

Table 5.4.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.4.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.4.3	MSD scenario studies

Table 5.10.3-1 and 5.10.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n28-n78.

Table 5.4.3-1: MSD scenarios for CA_n28-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL5) | n28 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | 4 * n28 – n78 | n28 | No |

Table 5.4.3-2: MSD scenarios for CA_n28-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL5) | n28 | No |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | 4 * f n28 – f n78 | n28 | No |

### 5.4.4	REFSENS requirements

#### 5.4.4.1	REFSENS requirements for total power class 2

- The MSD due to IMD5 for PC2 is specified in the following table.

Table 5.4.4.1-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78 | n28 |  | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n78 | 3582.5 |  | 10 | 50 | 3582.5 | N/A | TDD | N/A |

#### 5.4.4.2	REFSENS requirements for total power class 1.5

- The MSD due to IMD5 for PC1.5 3Tx are specified in the following table. The IMD5 MSD values for CA_n28-n78 and CA_n5-n77 are the same. Therefore, the CA_n28-n78 PC1.5 MSD values are reused from CA_n5-n77 PC1.5 MSD values.

Table 5.4.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78 | n28 |  | 705.5 | 5 | 25 | 760.5 | 24.3 | FDD | IMD5 |
|  | n78 | 3582.5 |  | 10 | 50 | 3582.5 | N/A | TDD | N/A |

## 5.5	CA_n1-n77

### 5.5.1	Operating bands for CA

Table 5.5.1-1:  CA band combination of band n1+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n1-n77 | n1, n77 |

### 5.5.2	Maximum output power for inter-band CA

Table 5.5.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

### 5.5.3	MSD scenario studies

For inter-band UL CA_n1A-n77A, IMD2 and IMD4 fall into n1 DL, and the corresponding MSD values are already specified in Table 7.3A.5-1a of latest version of TS38.101-1[4].

Table 5.10.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n1-n77.

Table 5.5.3-1: MSD scenarios for CA_n1-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | 1*f_n77-1*f_n1 | n1 DL | Yes |
| IMD4 | 3*f_n1-1*f_n77 | n1 DL | Yes |

### 5.5.4	REFSENS requirements

#### 5.5.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to IMD2 has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

## 5.6	CA_n3-n77

### 5.6.1	Operating bands for CA

Table 5.6.1-1:  CA band combination of band n3+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n3-n77 | n3, n77 |

### 5.6.2	Maximum output power for inter-band CA

Table 5.6.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n77A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

### 5.6.3	MSD scenario studies

The 2nd order harmonic mixing falls into Rx frequencies of n3, and the corresponding MSD values are already specified in Table 7.3A.4.4a-1 of latest version of TS38.101-1[4].

For inter-band UL CA_n3A-n77A, IMD2, IMD4 and IMD5 of dual UL falls into n3 DL, and the corresponding MSD values are already specified in Table 7.3A.5-1a of latest version of TS38.101-1[4].

Table 5.10.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n3-n77.

Table 5.6.3-1: MSD scenarios for CA_n3-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77(UL1/DL2) | n3 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | 1*f_n77 - 1*f_n3 | n3 DL | Yes |
| IMD4 | 3*f_n3 – 1*f_n77 | n3 DL | Yes |

### 5.6.4	REFSENS requirements

#### 5.6.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD2 has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

## 5.7	CA_n28-n77

### 5.7.1	Operating bands for CA

Table 5.7.1-1:  CA band combination of band n28+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n28-n77 | n28, n77 |

### 5.7.2	Maximum output power for inter-band CA

Table 5.7.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n77A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

### 5.7.3	MSD scenario studies

The 5th order harmonic mixing falls into Rx frequencies of n28, and the corresponding MSD values are already specified in Table 7.3A.4.4a-1 of latest version of TS38.101-1[4].

For inter-band UL CA_n28A-n77A, IMD5 of dual UL falls into n28 DL, and the corresponding MSD values are already specified in Table 7.3A.5-1a of latest version of TS38.101-1[4].

Table 5.10.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n28-n77.

Table 5.7.3-1: MSD scenarios for CA_n28-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77(UL1/DL5) | n28 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | 1*f_n28 - 4*f_n77 | n28 DL | Yes |

### 5.7.4	REFSENS requirements

#### 5.7.4.1	REFSENS requirements for total power class 2

- The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

- The 2Tx MSD due to IMD5 has already been specified, which is applicable for 3Tx.

## 5.8	CA_n26-n78

### 5.8.1	Operating bands for CA

Table 5.8.1-1:  CA band combination of band n26+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n26-n78 | n26, n78 |

### 5.8.2	Maximum output power for inter-band CA

Table 5.8.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.8.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.8.3	MSD scenario studies

Table 5.10.3-1 and 5.10.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for CA_n26-n78.

Table 5.8.3-1: MSD scenarios for CA_n26-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL4) | n26 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | f n78 – 3 * f n26 | n26 | Yes |

Table 5.8.3-2: MSD scenarios for CA_n26-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL4) | n26 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | f n78 – 3 * f n26 | n26 | No |

### 5.8.4	REFSENS requirements

#### 5.8.4.1	REFSENS requirements for total power class 2

- The MSD due to harmonic mixing PC2 is already specified in 38.101-1.

- The MSD due to IMD4 for PC2 is already specified in 38.101-1.

#### 5.8.4.2	REFSENS requirements for total power class 1.5

- The MSD due to harmonic mixing PC1.5 is already specified in 38.101-1.

- The MSD due to IMD5 for PC1.5 3Tx are specified in the following table. MSD value is reused from CA_n5-n77,

Table 5.8.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 |  | 836.5 | 5 | 25 | 881.5 | 25.5 | FDD | IMD4 |
|  | n78 | 3391 |  | 10 | 50 | 3391 | N/A | TDD | N/A |

## 5.9	CA_n25-n77 3Tx

### 5.9.1	Operating bands for CA and Configurations

Table 5.9.1-1:  CA band combination of band n25+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n25-n77 | n25, n77 |

Table 5.9.1-2:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | n258 n778,9 CA_n25A-n77A8,13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 5.9.2	Maximum output power for inter-band CA

Table 5.10.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.10.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.9.3	MSD scenario studies

Table 5.10.3-1 and 5.10.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for CA_n77-n85.

Table 5.10.3-1: MSD scenarios for CA_n77-n85 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77 (UL1/DL2) | n25 | Yes |
| Cross band leakage | n77 | n25 | Yes |
| IMD2 | f n77 – f n25 | n25 | Yes |
| IMD4 | 3*f n25 – f n77 | n25 | Yes |

### 5.9.4	REFSENS requirements

#### 5.9.4.1	REFSENS requirements for total power class 2

- The MSD due to harmonic mixing PC2 is already specified in 38.101-1.

- The MSD due to cross band leakage PC2 is already specified in 38.101-1.

- The MSD due to IMD2 for PC2 is already specified in 38.101-1.

- The MSD due to IMD4 for PC2 is already specified in 38.101-1.

## 5.10	CA_n77-n85 3Tx

### 5.10.1	Operating bands for CA and Configurations

Table 5.10.1-1:  CA band combination of band n77+n85

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n77-n85 | n77, n85 |

Table 5.10.1-2:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n77A-n85A | n778,9 CA_n77A-n85A8,13,14 | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

### 5.10.2	Maximum output power for inter-band CA

Table 5.10.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n77A-n85A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.10.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n77A-n85A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.10.3	MSD scenario studies

Table 5.10.3-1 and 5.10.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for CA_n77-n85.

Table 5.10.3-1: MSD scenarios for CA_n77-n85 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77 (UL1/DL5) | n85 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | f n77 – 4 * f n85 | n85 | Yes |

Table 5.10.3-2: MSD scenarios for CA_n26-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77 (UL1/DL5) | n85 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | f n77 – 4 * f n85 | n85 | No |

### 5.10.4	REFSENS requirements

#### 5.10.4.1	REFSENS requirements for total power class 2

- The MSD due to harmonic mixing PC2 is already specified in 38.101-1.

- The MSD due to IMD5 for PC2 is already specified in 38.101-1.

#### 5.10.4.2	REFSENS requirements for total power class 1.5

- The MSD due to harmonic mixing PC1.5 is already specified in 38.101-1.

For UL CA_n77A-n85A PC1.5, since the n25 power remains PC3, the n41 power increases by 4.7 dB compared to PC2 UL CA_n77A-n85A.

For the CA_n77A-n85A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 11.7 dB to16.2 dB, using the following equation:

The MSD due to IMD5 for PC1.5 3Tx is specified in the following table.

Table 5.10.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n77-n85 | n77 |  | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 702 |  | 5 | 20 | 732 | 16.2 | FDD | IMD5 |

## 5.11	CA_n7-n78

### 5.11.1	Operating bands for CA

Table 5.11.1-1:  CA band combination of band n7+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n7-n78 | n7, n78 |

### 5.11.2	Maximum output power for inter-band CA

Table 5.11.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.11.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.11.3	MSD scenario studies

Table 5.11.3-1 and 5.11.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n7-n78.

Table 5.11.3-1: MSD scenarios for CA_n7-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n78 | n7 | Yes |
| IMD | N/A | N/A | N/A |

Table 5.11.3-2: MSD scenarios for CA_n7-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n78 | n7 | Yes |
| IMD | N/A | N/A | N/A |

### 5.11.4	REFSENS requirements

#### 5.11.4.1	REFSENS requirements for total power class 2

-	The 2Tx MSD due to cross band isolation has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.11.4.2	REFSENS requirements for total power class 1.5

The PC1.5 MSD due to IMD4 is specified using the same value as for CA_n7-n77

Table 5.11.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n78 | n7 | 2520 | 5 | 25 | 2640 | 29.7 | FDD | IMD4 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |

-	The 2Tx MSD due to crossband isolation is specified below with same MSD value as used for CA_n7-n77.

Table 5.11.4.2-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 9.0 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

## 5.12	CA_n74-n77

### 5.12.1	Configurations

Table 5.12.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n74A-n77A | n778 CA_n74A-n77A8 | n74 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. |  |  |  |  |

### 5.12.2	Maximum output power for inter-band CA

The UE Power class 2 for uplink Configuration for CA_n74-n77 is shown in Table 5.6.2-1.

Table 5.12.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n74-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.12.3	MSD scenario studies

For single UL n77, there is no IMD issue and no harmonic mixing issue and no cross-band leakage issue. Therefore, there is no MSD issue. For inter-band UL CA_n74-n77, with high power on n77, IMD5 and IMD6 fall into n74 DL. However, MSD test point is not valid as there is no possible MSD where the n74DL is at duplex distance of n74UL. Therefore, there is no MSD for this scenario.

### 5.12.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.13	CA_n18-n77

### 5.13.1	Configurations

Table 5.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n77(2A) | CA_n77(2A)8 | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.13.2	Maximum output powers

It is omitted here because it is already defined.

#### 5.13.3	REFSENS requirements.

Based on analysis,

- there is no IMD issue and no harmonic issue for UL n77 to DL CA_n18-n77.

- 4th harmonic mixing of n18 DL falls into n77 UL from 3440 to 3500MHz, within the range of global band 3300-4200MHz.

#### Unaffected parts, such as inter-band cases, are omitted.

#### 5.13.3.1	Power class 2 for single uplink n77

In the latest 3GPP TS 38.101-1-i90, the impact of the 4th harmonic mixing of n18 to n77 UL for CA_n18-n77 is not included because of the historical background.

The PC3 case is proposed in R4-2507279  and the corresponding MSD values are 5.4dB for DL BW =5 MHz and 2.5 dB for DL BW=15 MHz.

For PC2 case, when PC3 MSD is 5.4, ΔMSD = 2.3, when PC3 MSD is 2.5, ΔMSD = 1.6, and the MSD values are calculated as 7.7 and 4.1 respectively and are presented in Table 5.13.5.

Table 5.13.3.1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 7.7 | NOTE 5 | UL1/DL4 X |
| n77 | n18 | 10 | 15 | 25 | 15 | 4.1 | NOTE 5 | UL1/DL4 X |
| Unaffected part (NOTE 8) is omitted. NOTE X: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 5.13.4	Power class 2 for uplink CA_n77(2A)

Band n18 is affected by IMD components IMD4 (2, -2), IMD6 (3, -3) and IMD11 (6, -5), resulting from the uplink operation of CA_n77(2A). For this TDD uplink band, the MSD analysis results corresponding to IMD6 and IMD11 can be disregarded. The MSD value aligns with the PC3 case [3], consistent with the guideline specified in reference [5].

Table 5.13.4-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n77 | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3915 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

### 5.13.5	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.14	CA_n25-n41

### 5.14.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.14.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C | n258 n418,9 CA_n25A-n41A8,13,14 CA_n41C8,9 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  | n258 n418,9 CA_n25A-n41A8,13,14 CA_n41C8,9 CA_n25A-n41C8,13,14 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination … NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.14.2	Maximum output power

Table 5.14.2-1 UE Power Class 1.5 for uplink intra-band CA

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class |
| --- | --- | --- | --- |
| CA_n41C | Case a | 29dBm | 29dBm |

Table 5.14.2-2 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 5.14.2-3 UE Power Class 1.5 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |

### 5.14.3	REFSENS requirements

#### The PC1.5 IMD

#### 5.14.3.1	UL CA_n41C Power Class 1.5 case a

The PC1.5 MSD for UL CA_n41C into DL n25 is the same as for PC3 and PC2.

Table 5.14.3-1 UE Power Class 1.5 MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

#### 5.14.3.2	UL CA_n25A-n41C Power Class 2 case a, b, d

For CA_n25A-n41C triple beat the key is that the FDD band increases by 3dB for inter-band PC2 but the CA_n41C is subject to MPR/A-MPR to meet IMD and only increases by 2dB for PC2 vs PC3. Therefore, IMD power increase is 2*3+1*2=8dB for PC2. Thus,

•	PC2 CA_n25A-n41C triple beat 15.3+8=23.3dB

Table 5.14.3.2-1 UE Power Class 1.5 MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | 1860 | 5 | 25 | 1940 | 23.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |

#### 5.14.3.3	UL CA_n25A-n41C Power Class 1.5 case a, b

For CA_n25A-n41C triple beat the key is that the FDD band increases by 6dB with inter-band PC1.5 but the CA_n41C is subject to MPR/A-MPR to meet IMD and only increases by 3dB for PC1.5 vs PC3. Therefore, IMD power increase is 2*6+1*3=15dB for PC1.5. Thus,

•	PC1.5 CA_n25A-n41C triple beat 15.3+15=30.3dB

Table 5.14.3.3-1 UE Power Class 1.5 MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | 1860 | 5 | 25 | 1940 | 30.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |

### 5.14.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 5.15	CA_n74-n77

### 5.15.1	Configurations

Table 5.15.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n74A-n77(2A) | n77(2A)8 | n74 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.15.2	Maximum output power

Already in clause 5.15.1.

### 5.15.3	REFSENS requirements

Based on co-existence studies presented for PC3(in R4-2510047), IMD6/7 of ULCA_n77(2A) falls into the n74DL, and the MSD requirements are defined as following.

Table 5.15.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n74-n77 | n74 | N/A | 5 | N/A | 1500 | 4.6 | FDD | IMD6 |
|  | n7712 | 3505 | 10 | 1(Rbstart=26) | 3505 | N/A | TDD | N/A |
|  |  | 4005 | 10 | 1(Rbstart=26) | 4005 |  |  |  |
|  | n74 | N/A | 5 | N/A | 1505 | 1.7 | FDD | IMD7 |
|  | n7712 | 3305 | 10 | 1(Rbstart=26) | 3305 | N/A | TDD | N/A |
|  |  | 3905 | 10 | 1(Rbstart=26) | 3905 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

### 5.15.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.16	CA_n5-n77

### 5.16.1	Configurations

Table 5.16.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n77C | n778,9 CA_n5A-n77A8 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | n778,9 CA_n5A-n77A CA_n77C CA_n5A-n77C | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n5B-n77A | n778,9 CA_n5A-n77A8 CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n778,9 CA_n5A-n77A CA_n5B | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5B-n77C | n778,9 CA_n5A-n77A8 CA_n5B CA_n77C | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | CA_n5B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | n778,9 CA_n5A-n77A CA_n5B CA_n77C CA_n5A-n77C | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

Note: On top of the existing specification, the new added configurations are highlighted in yellow for convenience.

### 5.16.2	MSD scenario studies

#### 5.16.2.1	Harmonic Issues

Issues have been identified as captured in TR 38.719-02-01 where it is noted that UL4/DL and UL5/DL1 issues excists for band n5. Since this band is not impacted by the addition in this TP no additioan MSD is needed.

#### 5.16.2.2	Harmonic Mixing Issues:

Issues have been identified as captured in TR 38.719-02-01 where it is noted that UL1/DL4 issues excists for band n77. When adding HPUE support it is needed to verify MSD has been defined. In this case where PC2 and PC1.5 is added for n77 MSD has already need defined in TS 38.101-1 Table 7.3A.4-4a-1 and Table 7.3A.4-4b for PC2 and PC1.5 respectively

#### 5.16.2.3	Cross-band isolation Issues:

No cross-band isolation issues were identified.

#### 5.16.2.4	IMD Issues:

IMD2, 4 and 5 issues have been identified as captured in TR 38.719-02-01. Corresponding MSD have already been defined for PC2 and PC1.5 in TS 38.101-1 Table 7.3A.5-1a and Table 7.3A.5-1b respectively.

### 5.16.3	REFSENS requirements

#### 5.16.3.1	Power class 2 for single uplink n77

No additional MSD is needed defined

#### 5.16.3.2	Power class 1.5 for single uplink n77

No additional MSD is needed defined

## 5.17	CA_n48-n77

### 5.17.1	Configurations

Table 5.17.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n77C | n778,9 CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 506, 606, 706, 806, 906, 1006 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | n778,9 CA_n77C | n48 | See n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48B-n77A | CA_n48B n778,9 | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | n778,9 CA_n48B | n48 | CA_n48B_BCS4 and 5 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48B-n77C | CA_n48B n778,9 CA_n77C | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS2 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | n778,9 CA_n48B | n48 | CA_n48B_BCS4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

Note: On top of the existing specification, the new added configurations are highlighted in yellow for convenience.

### 5.17.2	MSD scenario studies

#### 5.17.2.1	Harmonic Issues

This combination has not been captured in TR 38.719-02-01 hence the study is included here for conviencen.

Table 5.17.2-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n48 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n77 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 3300 | 4200 | N/A | - | - | - | - | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n48 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 3550 | 3700 | N/A | - | - | - | - | UL harmonic |
| DL23 | 7100 | 7400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 10650 | 11100 | - | - | N/A | - | N/A |  |
| DL4 | 14200 | 14800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 17750 | 18500 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

No issues found.

#### 5.17.2.2	Harmonic Mixing Issues:

The study has been included in the above section. No issues found

#### 5.17.2.3	Cross-band isolation Issues:

This combination has not been captured in TR 38.719-02-01 hence the study is included here for conviencen.

Table 5.17.1.3-2: Cross-band isolation analysis

| Bands | n48 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 3550 | 3700 | 3300 | 4200 |
| fDL (MHz) | 3550 | 3700 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 10 | 40 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3510 | 3740 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3470 | 3780 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3430 | 3820 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 3390 | 3860 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 3350 | 3900 | 2800 | 4700 |
| Analysis | There is an issue of ACLR1ACLR1 into n77 |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Cross-band isolation issues were identified. However, according to TS 38.101-1 Table 5.2A.2.1-1:

NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination

The syncrinization will mean there is no need for additional MSD defined.

#### 5.17.2.4	IMD Issues:

This combination has not been captured in TR 38.719-02-01 hence the study is included here for conviencen.

Table 5.17.2.2-1: Band n48 and Band n77 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 400 - 650 |  | 6850 - 7900 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2900 - 4100 |  | 2900 - 4850 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10400 - 11600 |  | 10150 - 12100 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6450 - 7800 |  | 6200 - 9050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 800 - 1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 13950 - 15300 |  | 13450 - 16300 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13700 - 15800 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9500 - 13250 |  | 10000 - 11500 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2500 - 5500 |  | 2250 - 4500 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16750 - 20500 |  | 17500 - 19000 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17000 - 20000 |  | 17250 - 19500 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

No IMD issues found.

### 5.17.3	REFSENS requirements

#### 5.17.3.1	Power class 2 for single uplink n77

No additional MSD is needed defined

#### 5.17.3.2	Power class 1.5 for single uplink n77

No additional MSD is needed defined

## 5.18	CA_n66-n77

### 5.18.1	Operating bands for CA

Table 5.18.1-1:   NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n77C | n778,9 CA_n77C8 CA_n66A-n77A8 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | n778,9 CA_n77C8 CA_n66A-n77A CA_n66A-n77C | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n66(2A)-n77C | n778,9 CA_n77C8 CA_n66A-n77A8 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  | n778,9 CA_n77C8 CA_n66A-n77A CA_n66A-n77C | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |

Note: The above foot-script note number is same as clause 5.5A.3.1 in TS38.101-1

Note: On top of the existing specification, the new added configurations are highlighted in yellow for convenience.

### 5.18.2	MSD scenario studies

For harmonic MSD by HPUE n77:

- According to the co-existence study in TR38.719-02-01,  there is an issue found at  n66 UL2 harmonic falls into n77 DL. However, due to the power of band n66 is still kept as PC3, so there is no additional harmonic MSD need to be defined comparing to the existing harmonic MSD requirements in the TS38.101-1.

For harmonic mixing MSD caused by HPUE n77:

- According to the co-existence study in TR38.719-02-01,  the frequency range of n77 UL2 is from 6600~8400MHz, while the frequency range of n66 DL3 is from 6330~6600MHz, it can be seen that the lower edge of  n77 UL2 is exact the same as the upper edge of n66 DL3. However, according to the following guidelines,

- When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the 
ULx/DLy cell is marked “D” for direct hitnWhen the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss.

- For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders.

We can see that there is no need to define the near-missing harmonic mixing MSD for HPUE n77 for CA_n66-n77 for BCS0, 1, 4 and5.

For cross-band isolation MSD by HPUE n77

-  According to the co-existence study in TR38.719-02-01, there is no issue found for cross-band isolation MSD for n77 into n66, there is for PC2 and PC1.5. The same n77A cross-band isolation MSD will also apply for UL CA_n77C for power class 2 and power class 1.5.

For IMD MSD caused by PC2 CA_n77C

- According to the co-existence study in TR38.719-02-01, IMD 13 of n77 falls into n66. however,  due to n66 and n77 are not the two bands in adjacent band group, there is no need to check IMD 13 MSD according to the following guidance

- For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified.

### 5.18.3	REFSENS requirements

#### 5.18.3.1	REFSENS requirements for total power class 2

Comparing the existing MSD in TS38.101-1, there are no additional MSD requirement needed to be specified.

#### 5.18.3.2	REFSENS requirements for total power class 1.5

Comparing the existing MSD in TS38.101-1, there are no additional MSD requirement needed to be specified.

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.X	CA_nX-nY-nZ

### 6.X.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.2-1x unless additional clarification>

Table 6.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY-nZ | nX7,9, nY7,9, CA_nX-nY7,9 | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.X.2	Maximum output power

<Editor’s note: In table 6.X.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.X.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 6.X.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |
|  | Case c | 29dBm | 29dBm | 23dBm |
|  | Case d | 29dBm | 26dBm | 29dBm |
|  | Case e | 29dBm | 29dBm | 26dBm |

### 6.X.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. The requirements in this TR are intended to be power class 2 cases based, however how to address in the spec will be further discussed. >

#### 6.X.3.1	Power class 2 case a

### 6.X.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 6.1	CA_n1-n18-n41

### 6.1.1	Configurations

Table 6.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n18A-n41A | CA_n1A-n41A7 CA_n18A-n41A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.1.2	Maximum output power

Table 6.1.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.1.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.1.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1A-n41A falls into n18 DL. The MSD exception is defined as below.

Table 6.1.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n18-n41 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 12.4 | FDD | IMD5 |

### 6.1.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.2	CA_n3-n18-n41

### 6.2.1	Configurations

Table 6.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n18A-n41A | CA_n3A-n41A7 CA_n18A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.2.2	Maximum output power

Table 6.2.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.2.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.2.3.1	Power class 2 case a, b

Based on calculation, IMD2 and IMD3 of dual UL CA_n3A-n41A fall into n18 DL, and IMD2 of dual UL CA_n18A-n41A falls in to n3 DL. The MSD exceptions are defined as follows.

Table 6.2.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n18-n41 | n18 | N/A | 5 | N/A | 865 | 34.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 875 | 28 | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 34.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |

### 6.2.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.3	CA_n18-n28-n41

### 6.3.1	Configurations

Table 6.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n28A-n41A | CA_n18A-n41A7 CA_n28A-n41A7 | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.3.2	Maximum output power

Table 6.3.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.3.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.3.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n18A-n41A falls into n28 DL. The MSD exception is defined as below.

Table 6.3.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n28-n41 | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 795 | 16.9 | FDD | IMD5 |

### 6.3.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.4	CA_n1-n18-n77

### 6.4.1	Configurations

Table 6.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n18A-n77A CA_n1A-n18A-n77(2A) | CA_n1A-n77A7 CA_n18A-n77A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.4.2	Maximum output power

Table 6.4.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.4.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.4.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1A-n77A falls into n18 DL, IMD3 of dual UL CA_n18A-n77A falls into n1 DL, the MSD exceptions are defined as below

Table 6.4.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n18-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n1-n18-n77(2A) | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 18.6 | FDD | IMD5 |
|  | n1 | N/A | 5 | N/A | 2120 | 25.4 | FDD | IMD3 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |

### 6.4.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

R4-2414295-298

## 6.5	CA_n1-n26-n78

### 6.5.1	Configurations

Table 6.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n26A-n78A | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78C | CA_n78C CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n26(2A)-n78C | CA_n26(2A) CA_n78C CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.5.2	Maximum output power

Table 6.5.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n26-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.5.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.5.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1A-n78A falls into n26 DL.

Based on calculation, IMD3 of dual UL CA_n26A-n78A falls into n1 DL.

The MSD exception is defined as below.

Table 6.5.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n26-n78 | n1 | N/A | 5 | N/A | 2122 | 27.0 | FDD | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 885 | 15.3 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |

### 6.5.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.6	CA_n3-n26-n78

### 6.6.1	Configurations

Table 6.6.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| CA_n3A-n26A-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n26A-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3B-n26A-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26A-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78B_BCS0 |  |
| CA_n3B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.6.2	Maximum output power

Table 6.6.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n26-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.6.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.6.3.1	Power class 2 case a, b

Based on calculation, no IMD of dual UL CA_n3A-n78A falls into n26 DL.

Based on calculation, IMD3 of dual UL CA_n26A-n78A falls into n3 DL.

The MSD exception is defined as below.

Table 6.6.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| CA_n3-n26-n78 | n3 | N/A | 5 | N/A | 1862 | 24.6 | FDD | IMD3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |

### 6.6.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.9	CA_n25-n41-n77 2Tx and 3Tx

### 6.9.1	Operating bands for CA and Configurations

Table 6.9.1-1:  CA band combination of band n25+n41+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n25-n41-n77 | n25, n41, n77 |

Table 6.9.1-2:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | n417,9 n777,9 CA_n25A-n41A7,13,14 CA_n25A-n77A7,13 CA_n41A-n77A7,9,13,14 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |  |

### 6.9.2	Maximum output power for inter-band CA

Changes needed to the 3Tx tables in 38.101-1 highlighted in yellow. 2Tx UL PC1.5 Is being completed at RAN4#116.

Table 6.9.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO and/or TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n77A | 29 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 6.9.2-2: Void

Table 6.9.2-3: UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n77A | 29x | +2/-3 | 269 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: Void NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 9: The UE supports either PC3 or PC2 within each NR TDD band. NOTE x: The UE supports PC2 within each band. |  |  |  |  |  |  |  |  |

### 6.9.3	MSD scenario studies

3BDL/2BUL configurations only have MSD requirements for intermodulation.

Table 6.9.3-1 and 6.9.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for CA_n25-n41-n77.

Table 6.9.3-1: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 2

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | IMD3 | 2*f n41 – f n25 | n77 | Yes |
|  | IMD5 | 3*f n41 – 2*f n25 | n77 | Yes |
| CA_n25A-n77A | IMD5 | 2*f n77 – 3*f n25 | n41 | Yes (but not testable for US deployments) |
| CA_n41A-n77A | IMD3 | 2*f n41 – f n77 | n25 | Yes (but not testable for US deployments) |
|  | IMD4 | 2*f n77 - 2*f n41 | n25 | Yes |

Table 6.9.3-2: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 1.5

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | IMD3 | 2*f n41 – f n25 | n77 | No |
|  | IMD5 | 3*f n41 – 2*f n25 | n77 | No |
| CA_n25A-n41A | IMD5 | 2*f n77 – 3*f n25 | n41 | Yes (but not testable for US deployments) |
| CA_n41A-n77A | IMD3 | 2*f n41 – f n77 | n25 | Yes (but not testable for US deployments) |
|  | IMD4 | 2*f n77 - 2*f n41 | n25 | Yes |

### 6.9.4	REFSENS requirements

#### 6.9.4.1	REFSENS requirements for total power class 2

- The MSD due to intermodulation PC2 for UL CA_n25A-n41A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n25A-n77A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n41A-n77A is already specified in 38.101-1.

#### 6.9.4.2	REFSENS requirements for total power class 1.5

For UL CA_n25A-n41A PC1.5 with 3Tx, since the n25 power remains PC3, the n41 power increases by 4.7 dB compared to PC2 UL CA_n25A-n41A.

For the CA_n25A-n41A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=2*4.7 = 9.4 dB, so the MSD increases from 23.7 dB to 33.1 dB, using the following equation:

For the CA_n25A-n41A IMD5, since only the n41 power is changing by 4.7 dB, the interferer increases by X=3*4.7 = 14.1 dB, so the MSD increases from 17.2 dB to 31.2 dB, using the equation above.

The MSD due to IMDs for PC1.5 3Tx are specified in the following table. At the time of this TP, the 3DL/2UL PC1.5 table does not yet exist in 38.101.1.

Table 6.9.4.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41-n77 | n25 |  | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2670 |  | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3470 | 33.1 | TDD | IMD3 |
|  | n25 | 1900 |  | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 |  | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3775 | 31.2 | TDD | IMD5 |

#### 6.9.4.3	REFSENS requirements for total PC2 and PC1.5 based on LUT

The FDD+TDD PC1.5 MSD in 6.9.4.2 was based on 23 dBm in the FDD band and 27.8 dBm in the TDD band. The 26+26 dBm MSD can be calculated using the Look Up Table approach.

Table 6.9.4.3-1 UE PC35A-n41A-n77A with U MSD

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n25-n41 | n77 | IMD3 | 14.8 |
|  |  | IMD5 | 4.2 |
| CA_n25-n77 | n41 | IMD5 | 5.3 |
| CA_n41-n77 | n25 | IMD3 | 17.6 |
|  |  | IMD4 | 8.6 |

### 6.9.5	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.10	CA_n41-n66-n77 2Tx and 3Tx

### 6.10.1	Operating bands for CA and Configurations

Table 6.10.1-1:  CA band combination of band n25+n41+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n41-n66-n77 | n25, n41, n77 |

Table 6.10.1-1:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7,13,14 CA_n41A-n77A7,9,13,14 CA_n66A-n77A7,13,14 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 6.10.2	Maximum output power for inter-band CA

No changes needed to the 3Tx tables in 38.101-1. Changes for UL CA_n41A-n77A PC1.5 are captured in 6.9.2.

Table 6.10.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 6.10.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 6.10.3	MSD scenario studies

3BDL/2BUL configurations only have MSD requirements for intermodulation.

Table 6.10.3-1 and 6.10.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for DL CA_n41-n66-n77.

Table 6.10.3-1: Intermodulation MSD scenarios for DL CA_n41A-n66A-n77A with total power class 2

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | Yes |
| CA_n41A-n77A | IMD4 | 2*fn77-2*fn41 | n66 | Yes |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n41 | Yes (But not testable for US n77 configurations) |

Table 6.10.3-2: Intermodulation MSD scenarios for DL CA_n411A-n66A-n77A with total power class 1.5

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | No |
| CA_n41A-n77A | IMD4 | 2*fn77-2*fn41 | n66 | Yes |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n41 | No |

### 6.10.4	REFSENS requirements

#### 6.10.4.1	REFSENS requirements for total power class 2

- The MSD due to intermodulation PC2 for UL CA_n41A-n66A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n66A-n77A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n66A-n77A is already specified in 38.101-1.

#### 6.10.4.2	REFSENS requirements for total power class 1.5

For 2Tx UL CA_n41A-n77A the power increase by 6 dB compared to the PC3 case. The LUT approach should be used to calculate the IMD4 MSD.

For 3Tx UL CA_n41A-n66A PC1.5, since the n66 power remains PC3, the n41 power increases by 4.7 dB compared to UL CA_n41A-n66A PC2.

For the UL CA_n41A-n66A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=2*4.7 = 9.4 dB, so the MSD increases from 25.0 dB to 34.4 dB, using the following equation:

New_MSD

For the UL CA_n66A-n77A IMD5, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 18.7 dB to 23.4 dB, using the equation above.

The MSD due to IMDs for PC1.5 3Tx are specified in the following table. At the time of this TP, the 3DL/2UL PC1.5 table does not yet exist in 38.101.1.

Table 6.10.4.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n66-n77 | n41 |  | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 |  | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3470 | 34.4 | TDD | IMD31,2 |
|  | n41 | N/A |  | 5 | N/A | 2670 | 23.4 | TDD | IMD55 |
|  | n66 | 1715 |  | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 |  | 10 | 50 | 4190 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |  |  |

#### 6.10.4.3	REFSENS requirements based on LUT for PC2 and PC1.5

The PC1.5 analysis in 6.10.4.2 used 23 dBm for n66 and with 27.8 dBm for n41 and n66. The Look Up Table approach analysis uses equal power and can be used to determine the PC2 and PC1.5 MSD.

Table 6.10.4.3-1 UE PC2 and PC1.5 LUT MSD

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n41A-n66A | n77 | IMD3 | 16.1 |
| CA_n41A-n77A | n66 | IMD4 | 9.0 |
| CA_n66A-n77A | n41 | IMD5 | 5.2 |

6.11	Void

## 6.11	CA_n41-n66-n77 3Tx

### 6.11.1	Operating bands for CA and Configurations

Table 6.11.1-1:  CA band combination of band n25+n41+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n25-n41-n77 | n25, n41, n77 |

Table 6.11.1-1:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7,13,14 CA_n41A-n77A7 CA_n66A-n77A7,13,14 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 6.11.2	Maximum output power for inter-band CA

No changes needed to the 3Tx tables in 38.101-1.

Table 6.11.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 6.11.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 6.11.3	MSD scenario studies

3BDL/2BUL configurations only have MSD requirements for intermodulation.

Table 6.11.3-1 and 6.11.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for DL CA_n41-n66-n77.

Table 6.11.3-1: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 2

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | Yes |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n41 | Yes (But not testable for US n77 configurations) |

Table 6.11.3-2: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 1.5

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | No |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n77 | No |

### 6.11.4	REFSENS requirements

#### 6.11.4.1	REFSENS requirements for total power class 2

- The MSD due to intermodulation PC2 for UL CA_n41A-n66A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n66A-n77A is already specified in 38.101-1.

#### 6.11.4.2	REFSENS requirements for total power class 1.5

For UL CA_n41A-n66A PC1.5, since the n66 power remains PC3, the n41 power increases by 4.7 dB compared to UL CA_n41A-n66A PC2.

For the UL CA_n41A-n66A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=2*4.7 = 9.4 dB, so the MSD increases from 25.0 dB to 34.4 dB, using the following equation:

New_MSD

For the UL CA_n66A-n77A IMD5, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 18.7 dB to 23.4 dB, using the equation above.

The MSD due to IMDs for PC1.5 3Tx are specified in the following table. At the time of this TP, the 3DL/2UL PC1.5 table does not yet exist in 38.101.1.

Table 6.11.4.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n66-n77 | n41 |  | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 |  | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3470 | 34.4 | TDD | IMD31,2 |
|  | n41 | N/A |  | 5 | N/A | 2670 | 23.4 | TDD | IMD55 |
|  | n66 | 1715 |  | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 |  | 10 | 50 | 4190 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: This band combination is specified with support of two band inter-band UL CA with UL MIMO or Tx diversity capabilities, and the transmitter shall be set at min (+23 dBm, PCMAX_L,f,c) for the UL band with single Tx antenna connector as defined in clause 6.2A.4, and set at min (+27.8 dBm, PCMAX_L,f,c) for the UL band with two Tx antenna connectors as defined in clause 6.2H.3.4 or 6.2L.3.4. |  |  |  |  |  |  |  |  |  |  |

## 6.12	CA_n41-n71-n77 2Tx and 3Tx

### 6.12.1	Operating bands for CA and Configurations

Table 6.12.1-1:  CA band combination of band n41+n71+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n41-n71-n77 | n25, n41, n77 |

Table 6.12.1-1:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7,13,14 CA_n41A-n77A7,9,13,14 CA_n71A-n77A7,13,14 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 6.12.2	Maximum output power for inter-band CA

No changes needed to the 3Tx tables in 38.101-1.

Table 6.12.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 6.12.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

Changes for 2Tx UL CA_n41A-n77A PC1.5 are captured in 6.9.2.

### 6.12.3	MSD scenario studies

3BDL/2BUL configurations only have MSD requirements for intermodulation.

Table 6.12.3-1 and 6.12.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for DL CA_n41-n71-n77.

Table 6.12.3-1: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 2

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | IMD2 | f n41+ f n71 | n77 | Yes |
|  | IMD3 | f n41 + 2*f n71 | n77 | Yes |
|  | IMD4 | 2*f n41 - 2*f n71 | n77 | Yes |
| CA_n71A-n77A | IMD2 | f n77 – f n71 | n41 | Yes |
|  | IMD3 | f n77 – 2*f n71 | n41 | Yes |

Table 6.12.3-2: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 1.5

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | IMD2 | f n41+ f n71 | n77 | No |
|  | IMD3 | f n41 + 2*f n71 | n77 | No |
|  | IMD4 | 2*f n41 - 2*f n71 | n77 | No |
| CA_n71A-n77A | IMD2 | f n77 – f n71 | n41 | No |
|  | IMD3 | f n77 – 2*f n71 | n41 | No |

### 6.12.4	REFSENS requirements

#### 6.12.4.1	REFSENS requirements for total power class 2

- The MSD due to intermodulation PC2 for UL CA_n41A-n71A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n71A-n77A is already specified in 38.101-1.

#### 6.12.4.2	REFSENS requirements for total power class 1.5

For UL CA_n41A-n71A PC1.5, since the n71 power remains PC3, the n41 power increases by 4.7 dB compared to UL CA_n41A-n71A PC2.

For the UL CA_n41A-n71A IMD2, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 35.1 dB to 39.8 dB, using the following equation:

New_MSD

For the UL CA_n41A-n71A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 25.2 dB to 29.9 dB, using the equation above.

For the UL CA_n41A-n71A IMD4, since only the n41 power is changing by 4.7 dB, the interferer increases by X=2*4.7 = 9.4 dB, so the MSD increases from 21.9 dB to 31.3 dB, using the

For the UL CA_n71A-n77A IMD2, since only the n77 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 34.7 dB to 39.4 dB, using the equation above.

For the UL CA_n71A-n77A IMD3, since only the n77 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 24.4 dB to 29.1 dB, using the equation above.

The MSD due to IMDs for PC1.5 3Tx are specified in the following table. At the time of this TP, the 3DL/2UL PC1.5 table does not yet exist in 38.101.1.

Table 6.12.4.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71-n77 | n41 |  | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 |  | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3308 | 39.8 | TDD | IMD21,5 |
|  | n41 | 2564 |  | 5 | 25 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 |  | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3950 | 29.9 | TDD | IMD31 |
|  | n41 | 2580 |  | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 |  | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3774 | 31.3 | TDD | IMD41 |
|  | n41 | N/A |  | 5 | N/A | 2615 | 39.4 | TDD | IMD25 |
|  | n71 | 693 |  | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 |  | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | N/A |  | 5 | N/A | 2564 | 29.1 | TDD | IMD3 |
|  | n71 | 693 |  | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 |  | 10 | 50 | 3950 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |  |  |

#### For 2Tx and 3Tx UL CA_n41A-n77A there is an IMD2 into the n71 DL The PC3 MSD is 30.8 dB. The PC1.5 MSD calculated based on the Look Up Table approach is 42.8 dB. 6.12.4.3	REFSENS requirements for total power class 2 and power clase 3 based on the LUT

The MSD requirements 38.101-1 are based on calculations, but the PC1.5 MSD for CA_n41-n71 and CA_n71-n77 were based on 23+29 dBm. The LUT approach is used to calculate the equal power MSD.

Table 6.12.4,3-1 PC3 MSD

| Uplink CA configuration | Victim Band | Power class 2 cases for CA_nX-nY | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n41-n71 | n77 | IMD2 | 29.1 |
|  |  | IMD3 | 16.3 |
|  |  | IMD4 | 10.3 |
| CA_n71-n77 | n41 | IMD2 | 28.7 |
|  |  | IMD3 | 15.5 |
| CA_n41-n77 | n71 | IMD2 | 30.8 |

## 6.13	CA_n41-n77-n79

### 6.13.1	Operating bands for CA and Configurations

Table 6.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A-n79A | n417, 9 n777, 9 n797, 9 CA_n41A-n77A7 CA_n41A-n79A7 CA_n77A-n79A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77(2A)-n79A | n417, 9 n777, 9 n797, 9 CA_n41A-n77A7 CA_n41A-n79A7 CA_n77A-n79A7 CA_n77(2A)7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(2A) _BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.13.2	Maximum output power for inter-band CA

Table 6.13.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n41-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n77-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.13.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.13.3.1	Power class 2 case a, b, c, d

Based on Reference sensitivity exceptions due to harmonic interference for CA in TS38.101-1[2], 3rd harmonics of n41 may fall into n77 DL. And 3rd harmonics of n77 also may fall into n41 DL. However, non-simultaneous Rx-Tx operation for this combination is assumed, therefore no MSD is needed for single uplink.

PC3’s UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n41-n77, CA_n41-n79 and CA_n77-n79 in TR38.718-03-01[3], and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings. Note that there is no IMD impact on CA_n41-n77 to n79 Rx and that on CA_n41-n79 to n77 Rx since CA_n77-n79 is synchronous operation.

–   3rd, 4th and 5th order IMD generated by dual uplink of Band n77 + Band n79 may fall into own Rx of Band n41.

And MSD for PC2 UL CA can be calculated from its PC3’s result.

If the input signal increases by 3dB, the IMD3 increases by 3*3 = 9dB.

So, the MSD exception is defined as below.

Table 6.13.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77-n79 | n77 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | 19.7 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

For CA_n77(2A), fallback combination’s study has already done in TS38.101-1 Table 7.3A.5-1a: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA[2], the MSD exception is defined as below.

Table 6.13.3-2: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77-n79 | n41 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 32 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 16: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range |  |  |  |  |  |  |  |  |

### 6.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.14	CA_n3-n41-n79

### 6.14.1	Operating bands for CA and Configurations

Table 6.14.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A-n79A | n3 n417, 9 n797, 9 CA_n3A-n41A7 CA_n3A-n79A7 CA_n41A-n79A7 | n3 | 15, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.14.2	Maximum output power

Table 6.14.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.14.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.14.3.1	Power class 2 case a, b, c, d

Based on Reference sensitivity exceptions due to harmonic interference for CA in TS38.101-1[2], no MSD is needed for single uplink.

UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n3-n41, CA_n3-n79 and n41-n79 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings.

–   2nd and 5th order IMD generated by dual uplink of Band n3 + Band n41 may fall into own Rx of Band n79.

–   2nd and 5th order IMD generated by dual uplink of Band n3 + Band n79 may fall into own Rx of Band n41.

–   2nd and 5th order IMD generated by dual uplink of Band n41 + Band n79 may fall into own Rx of Band n3.

And MSD for PC2 UL CA can be calculated from its PC3’s result.

If the input signal increases by 3dB, the IMD2 increases by 3*2 = 6dB.

So, the MSD exception is defined as below.

Table 6.14.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n79 | n3 | 1755 | 5 | 25 | 1850 | 35.4 | FDD | IMD21 |
|  | n41 | 2570 | 10 | 50 | 2570 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | 36.2 | TDD | IMD21 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 36.8 | TDD | IMD21 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.14.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.15	CA_n28-n41-n79

### 6.15.1	Operating bands for CA and Configurations

Table 6.15.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A-n79A | n28 n417, 9 n797, 9 CA_n28A-n41A7 CA_n28A-n79A7 CA_n41A-n79A7 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.15.2	Maximum output power

Table 6.15.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.15.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

### 6.15.3.1	Power class 2 case a, b, c, d

Based on Reference sensitivity exceptions due to harmonic interference for CA in TS38.101-1[2], no MSD is needed for single uplink.

PC3’s UE co-existence studies for the UL CA configurations of CA_n28A-n41A, CA_n28A-n79A, CA_n41A-n79A have already captured in TR 38.716-02-00[4], where:

- 3rd and 5th intermodulations caused by 2UL CA_n41A-n79A may fall into band n28 own DL Rx range.

- 3rd and 4th intermodulations caused by 2UL CA_n28A-n41A may fall into band n79 own DL Rx range.

-  4th intermodulations caused by 2UL CA_n28A-n79A may fall into band n41 own DL Rx range.

And MSD for PC2 UL CA can be calculated from its PC3’s result.

If the input signal increases by 3dB, the IMD3 increases by 3*3 = 9dB, and the IMD4 increases by 3*4 = 12dB.

So, the MSD exception is defined as below.

Table 6.15.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n79 | n28 | 725 | 5 | 25 | 780 | 22 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4480 | 40 | 216 | 4600 | 19.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | 2645 | 10 | 50 | 2645 | 22.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.15.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.16	CA_n3-n78-n79

### 6.16.1	Configurations

Table 6.16.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A-n79A5 | n787,9 n797 CA_n3A-n78A7 CA_n3A-n79A7 CA_n78A-n79A5,7 | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.16.2	Maximum output power

Table 6.16.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.16.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.16.3.1	Power class 2 case a, b, c, d

Based on Reference sensitivity exceptions due to harmonic interference for CA in TS38.101-1[2], no MSD is needed for single uplink.

PC3’s UE co-existence studies for the UL CA configurations of CA_ n3A-n78A, CA_ n3A-n79A, CA_ n78A-n79A have already captured in TR 38.719-03-01[3], where:

The 3rd IMD generated by UL CA_n3A-n78A may fall into own Rx of Band n79.

The 5th IMD generated by UL CA_n3A-n79A may fall into own Rx of Band n78.

The 3rd, 4th and 5th IMD generated by UL CA_n78A-n79A may fall into own Rx of Band n3.

And MSD for PC2 UL CA can be calculated from its PC3’s result.

If the input signal increases by 3dB, the IMD3 increases by 3*3 = 9dB. So, the MSD exception is defined as below.

Table 6.16.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78-n79 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4860 | 22.1 | TDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3480 | 8.5 | TDD | IMD5 |
|  | n79 | 4410 | 10 | 50 | 4410 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 24.7 | FDD | IMD31 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n79 | 4840 | 10 | 50 | 4840 | N/A | TDD | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 11.5 | FDD | IMD4 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n79 | 4700 | 10 | 50 | 4700 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.16.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.17	CA_n28-n41-n74

### 6.17.1	Configurations

Table 6.17.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A-n74A | CA_n28A-n41A7 CA_n41A-n74A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n74 | 5, 10, 15, 20 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.17.2	Maximum output power

Table 6.17.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n74 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 26dBm | 23dBm |

### 6.17.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.17.3.1	Power class 2 case a, b

Based on calculation, for dual UL CA_n28A-n41, the 2nd order harmonic of n28 drops to n74 DL, while for dual UL CA_n41A-n74A, IMD5 affects n28 DL.

As to the impact of 2nd order harmonic of n28 to n74 DL, using the method agreed upon in previous meeting [4], for 1UL MSD values, 3dB and 6dB should be added to the corresponding PC3 values, respectively. The results are shown in following table.

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n28 | n74 | 5 | 15 | 12 | 5 | 31.1 | 34.1 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n74 | 5 | 15 | 12 | 20 | 21 | 24 | NOTE 2 | UL2/DL1 direct-hit |

The MSD exceptions due to the IMD5 are defined as follows:

- For the PC3 case, the MSD value corresponding to IMD5 to n28 is 3.5dB, as proposed in [3].

- For the PC2 case, introducing an additional 15dB interference to the PC3 scenario resulted in a 14dB increase in DESENS, leading to an overall MSD of 17.5dB.

The calculated MSD value and the relevant test points are presented in Table 6.x.3-1.

Table 6.17.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n74 | n28 | N/A | 5 | N/A | 782 | 17.5 | FDD | IMD5 |
|  | n41 | 2569 | 10 | 50 | 2569 | N/A | TDD | N/A |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |

### 6.17.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.18	CA_n28-n74-n77

### 6.18.1	Configurations

Table 6.18.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n74A-n77A | CA_n28A-n77A7 CA_n74A-n77A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.18.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n74-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.18.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.18.3.1	Power class 2 case a, b

Based on calculation

- IMD4 due to UL CA_n28A-n77A falls into n74 DL

- IMD3/4 due to UL CA_n74A-n77A falls into n28 DL

The MSD exceptions corresponding to the IMD4 when UL CA_n28A-n77A falls into n74, MSD for PC2 is calculated by are defined as below by adding 12dB to interference which caused 9dB PC3 MSD, resulting in 20.7dB IMD5 to n74

Moreover, for the cases of IMD3/4 due to UL CA_n74A-n77A falls into n28 DL,

for IMD3, adding 9dB to the PC3 MSD value, 18.7, resulting in PC2 MSD, 27.7dB

for IMD4 adding 12dB to interference to the 13.1dB PC3 MSD, resulting in 25.1dB

Table 6.18.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n74-n77 | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n74 | N/A | 5 | 25 | 1500 | 20.7 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 27.7 | FDD | IMD3 |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 3689 | 10 | 50 | 1818.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 25.1 | FDD | IMD4 |
|  | n74 | 1458 | 5 | 25 | 1516 | N/A | FDD | N/A |
|  | n77 | 3589 | 10 | 50 | 1818.5 | N/A | FDD | N/A |

### 6.18.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.19	CA_n25-n41-n66

### 6.19.1	Configurations

Table 6.19.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | n257 n417,9 n667 CA_n25A-n41A7 CA_n25A-n66A7 CA_n41A-n66A7,13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 6.19.2	Maximum output power

<Editor’s note: In table 6.X.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.19.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n66 | Case c | 26dBm | 26dBm | 23dBm |

Table 6.19.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n66 | Case c | 29dBm | 29dBm | 23dBm |

### 6.19.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.19.3.1	Power class 2 case c

There is an IMD4 from UL CA_n41A-n66A into the n25 DL. The PC3 MSD is 11.0 dB. The PC2 MSD is calculated with the LUT approach.

#### 6.19.3.2	Power class 1.5 case c

There is an IMD4 from UL CA_n41A-n66A into the n25 DL. The PC3 MSD is 11.0 dB. The PC1.5 MSD is calculated with the LUT approach.

### 6.19.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 6.20	CA_n25-n66-n77

### 6.20.1	Configurations

Table 6.20.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | n257 n667 n777,9 CA_n25A-n66A7 CA_n25A-n77A7,13,14 CA_n66A-n77A7,13,14 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 6.20.2	Maximum output power

Table 6.20.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n77 | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n77 | Case b | 26dBm | 23dBm | 26dBm |

Table 6.20.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n77 | Case b | 29dBm | 23dBm | 29dBm |
| CA_n66-n77 | Case b | 29dBm | 23dBm | 29dBm |

### 6.20.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.20.3.1	Power Class 2 and Power Class 1.5 MSD

There are several IMDs for the two uplink configurations. The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD.

Table 6.20.3-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n25-n77 | n66 | IMD2 | 29.2 |
|  |  | IMD4 | 10.4 |
|  |  | IMD5 | 4.0 |
| CA_n66-n77 | n25 | IMD2 | 32.1 |
|  |  | IMD4 | 9.1 |
|  |  | IMD5 | 2.1 |

### 6.20.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 6.21	CA_n25-n71-n77

### 6.21.1	Configurations

Table 6.21.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | n257 n717 n777,9 CA_n25A-n71A7 CA_n25A-n77A7,13,14 CA_n71A-n77A7,13,14 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 6.21.2	Maximum output power

Only UL combinations with IMD MSD are included in this section.

Table 6.21.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71-n77 | Case b | 26dBm | 23dBm | 26dBm |

Table 6.21.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71-n77 | Case b | 29dBm | 23dBm | 29dBm |

### 6.21.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.21.3.1	Power Class 2 and Power Class 1.5 MSD

There is and IMD3 from CA_n71A-n77A into n25.  The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD. There is no MSD for UL CA_n25-n77.

Table 6.21.3-1 UE PC2 and PC1.5 LUT MSD

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) | PC2 MSD (dB) | P{C1.5 MSD (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n71-n77 | n25 | IMD3 | 16.5 | 25.5 | 34,5 |

### 6.21.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 6.22	CA_n66-n71-n77

### 6.22.1	Configurations

Table 6.22.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | n667 n717 n777,9 CA_n66A-n71A7 CA_n66A-n77A7,13,14 CA_n71A-n77A7,13,14 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 6.22.2	Maximum output power

Table 6.22.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n77 | Case b | 26dBm | 23dBm | 26dBm |
| CA_n71-n77 | Case b | 26dBm | 23dBm | 26dBm |

Table 6.22.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n77 | Case b | 29dBm | 23dBm | 29dBm |
| CA_n71-n77 | Case b | 29dBm | 23dBm | 29dBm |

### 6.22.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.22.3.1	Power Class 2 and Power Class 1.5 MSD

There are IMD3s for the two uplink configurations. The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD.

Table 6.22.3-1 UE PC2 and PC1.5 LUT MSD

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n66-n77 | n71 | IMD3 | 15.3 |
| CA_n71-n77 | n66 | IMD3 | 15.5 |

### 6.22.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 6.23	CA_n41-n74-n77

### 6.23.1	Configurations

Table 6.23.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n74A-n77A | CA_n41A-n74A7 CA_n41A-n77A7 CA_n74A-n77A7 | n41 | 10, 15, 20, 40,50, 60, 80, 90,100 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.23.2	Maximum output power

Table 6.23.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n74 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n74-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.23.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.23.3.1	Power class 2 case a, b

Based on calculation

- IMD2/3/4 due to UL CA_n41A-n77A falls into n74 DL

- IMD2/5 due to UL CA_n74A-n77A falls into n41 DL

- IMD2/3/5 due to UL CA_n41A-n74A falls into n77 DL

The MSD exceptions corresponding to the above IMDs for PC2 are shown in Table 6.23.3-1. They are evaluated by calculated by adding the corresponding interference defined in [4] 12dB to the MSDs for PC2 defined in [3].

Table 6.23.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n74-n77 | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 35.2 | FDD | IMD22 |
|  | n77 | 4150 | 10 | 50 | 4150 | NA | TDD | N/A |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 24 | FDD | IMD3 |
|  | n77 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 34 | TDD | IMD2 |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4150 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2518 | 13.9 | TDD | IMD5 |
|  | n74 | 1454 | 5 | 25 | 1502 | N/A | FDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4100 | 33 | TDD | IMD2 |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1498 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3600 | 23 | TDD | IMD31 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.23.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.24	CA_n1-n78-n79

### 6.24.1	Configurations

Table 6.24.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A-n79A | CA_n1A-n78A7 CA_n1A-n79A7 CA_n78A-n79A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.24.2	Maximum output power

Table 6.24.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n1-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.24.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.24.3.1	Power class 2 case a, b, c, d

#### Based on calculation,

- IMD3/IMD5 due to UL CA_n1A-n78A falls into n79 DL. The IMD3 value is calculated by adding 9dB to the PC3 MSD value, resulting in 24.9dB.

- IMD5 due to UL CA_n1A-n79A falls into n78 DL. The IMD5 value is calculated by adding 12.4dB to the PC3 MSD value, resulting in 17dB.

- IMD3/IMD4/IMD5 due to UL CA_n78A-n79A falls into n1 DL. The IMD3 value is calculated by adding 9dB to the PC3 MSD value, resulting in 24.6dB. PC2 IMD4 is 3.8dB.

#### The MSD exception is defined as below.

Table 6.24.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4870 | 24.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3490 | 17 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 24.6 | FDD | IMD31 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 3.8 | FDD | IMD41 |
|  | n78 | 3590 | 10 | 50 | 3590 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |

### 6.24.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.25	CA_n28-n78-n79

### 6.25.1	Configurations

Table 6.25.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n78A-n79A | CA_n28A-n78A7 CA_n28A-n79A7 CA_n78A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  | CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.25.2	Maximum output power

Table 6.25.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.25.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.25.3.1	Power class 2 case a, b, c, d

#### Based on calculation,

- IMD2/IMD3/IMD5 due to UL CA_n28A-n78A fall into n79 DL. The IMD2 value is calculated by adding 6dB to the PC3 MSD value, resulting in 32.2dB. PC2 IMD5 value is 5.7dB.

- IMD2/IMD3 due to UL CA_n28A-n79A fall into n78 DL. The IMD2 value is calculated by adding 6dB to the PC3 MSD value, resulting in 32.9dB. PC2 IMD3 value is 16.8dB.

  - NOTE: As the PC3 IMD3 may have been missed, this PC2 value may be revised at RAN4#116-Bis with new the requirement for PC3.

- IMD2 /IMD5 due to UL CA_n78A-n79A fall into n28 DL. The IMD2 value is calculated by adding 6dB to the PC3 MSD value, resulting in 22.2dB. PC2 IMD5 value is 5.7dB.

#### The MSD exception is defined as below.

Table 6.25.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78-n79 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 32.2 | TDD | IMD23,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 5.7 | TDD | IMD53,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 32.9 | TDD | IMD23 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3420 | 16.8 | TDD | IMD33 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 800 | 22.2 | FDD | IMD2 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 5.7 | FDD | IMD5 |
|  | n78 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
| NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.25.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

# 7	High Power UE CA with SUL

## 7.x	CA_nW-nX_SUL_nY-nZ

### 7.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5C-4 unless additional clarification>

Table 7.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink configuration | NR Band | Channel Bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nW-nX_SUL_nY-nZ | SUL_nY-nZ | nW |  |  |
|  |  | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |

### 7.x.2	Maximum output power

### 7.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

### 7.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

# Annex A:		Valid UL configurations

For High Power UE CA of 2 or 3 bands DL with 1 or 2 bands UL, the following UL configurations are applicable, subject to the general requirements:

-  One band UL with one CC: nXA, nYA

-  One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A)

-  two band UL with one CC per band: CA_nXA-nYA

-  two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

# Annex B (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2412268 |  |  |  | TR skeleton | 0.1.0 |
| 2024-10 | RAN4 #112-bis | R4-2414979 |  |  |  |  | 0.2.0 |