3GPP TSG-RAN WG4 #116bis	R4-2513073

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.22.2		UE demodulation for time/frequency interleaving

Source:	Qualcomm Incorporated

Title:						Demodulation for time-frequency interleaving for 5G Broadcast

Document for:	Discussion

# Introduction

In RAN4 #116, RAN4 work on Rel-19 LTE-based 5G Broadcast Phase 2 was started according to the work item description in [1] and the way forward was agreed in [2]. In this paper we provide simulation results for the topics identified in the way forward in [2].

# Discussion

In [2], the following way forward was approved.

Proposal 1: Both (M, N) = (8, 4) and (M, N) = (32, 16) should be chosen for defining demod requirements for time-frequency interleaving.

Proposal 2: The corresponding TBS for (M, N) = (8, 4) should be moderate and the corresponding TBS for (M, N) = (32, 16) should be low. Exact values are FFS.

Proposal 3: As a starting point for the simulations, we propose to use 31704 bits scaled TBS for a 7992 bits base TBS for (M, N) = (8, 4) with 16QAM and for (M, N) = (32, 16), we propose 45352 bits scaled TBS for a 2856 bits base TBS with QPSK.

Proposal 4: Demod requirements should be defined for SCS = 1.25 kHz.

Proposal 5: Demod requirements should be introduced for 8 MHz channel bandwidth.

Proposal 6: Time-frequency interleaving should be tested for Doppler frequencies of 5 Hz and 50 Hz.

Proposal 7: For SCS = 1.25 kHz and Doppler frequency fD = 5 Hz, the channel model in section B.2.6.2-1 of [3] should be reused.

Proposal 8: For SCS = 1.25 kHz and Doppler frequency fD = 50 Hz, the channel model in section B.2.6.2-1 of [3] should be enhanced accordingly.

Proposal 9: In the demod requirement test cases, both time and frequency interleaving should be enabled jointly.

## Simulation Results

The way forward outlines all parameters that are needed to provide simulation results. In the following we provide our alignment results according to these agreements.

| Bandwidth [MHz] | RMC | Interleaving parameter M, N | TBS [bits] | Code rate over N RVs per TB | Doppler [Hz] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 8 MHz | FRC-1 | M = 8, N = 4 | 31704 | 0.41 | 5 Hz | 10.2 dB |
|  |  |  |  |  | 50 Hz | 8.6 dB |
| 8 MHz | FRC-2 | M = 32, N = 16 | 45352 | 0.3 | 5 Hz | 1 dB |
|  |  |  |  |  | 50 Hz | 0.4 dB |

We provide results for different interleaving depths and Doppler shifts of 5 Hz and 50 Hz. The subcarrier spacing for all cases is 1.25 kHz. It is also seen from the results that additional gains are seen if the Doppler is changed from 5 Hz to 50 Hz. This demonstrates the robustness of the time/frequency interleaving.

The results show that the simulation assumptions in the way forward [2] can be confirmed.

Proposal 1: RAN4 to confirm that demod requirements are defined for SCS = 1.25 kHz.

Proposal 2: RAN4 to confirm that demod requirements are defined for (M, N) = (8, 4) and (M, N) = (32, 16).

Proposal 3: RAN4 to confirm the TB sizes of 31704 bits for (M, N) = (8, 4) and of 45352 for (M, N) = (32, 16).

Proposal 4: RAN4 to confirm that demod requirements are defined for 8 MHz channel bandwidth.

Proposal 5: RAN4 to confirm that demod requirements are defined for the channel model channel model in section B.2.6.2-1 in [3] for 5 Hz and 50 Hz.

Proposal 6: RAN4 to confirm that both time and frequency domain interleaving is enabled in the demod test cases.

## Details of the Reference Measurement Channel

In this section, we provide the details of the fixed reference channels used in the simulations above.

Table 2-1: Fix Reference Channel for Time/Frequency Interleaving

| Parameter | PMCH |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | FRC-1 | FRC-2 |
| Channel bandwidth | MHZ | 8 | 8 |
| Subcarrier spacing | kHz | 1.25 | 1.25 |
| Allocated resource blocks |  | 40 | 40 |
| Allocated slots per 40 ms (Note 1) |  | 32 | 39 |
| Time domain spreading and scaling (M, N) |  | (8, 4) | (32, 16) |
| Frequency domain interleaving |  | Enabled Codebook cycling disabled | Enabled Codebook cycling disabled |
| Modulation |  | 16QAM | QPSK |
| Target coding rate |  | 0.41 | 0.3 |
| Information bit payload (Note 2) |  |  |  |
| For each slot | Bits | 31704 | 45352 |
| Number of code blocks per subframe |  | 6 | 6 |
| Binary channel bits per subframe |  |  |  |
| For each slot | Bits | 19200 | 9600 |
| Transport block size before scaling with N | Bits | 7992 | 2825 |
| Transport block size after scaling with N | Bits | 31704 | 45353 |
| MBMS UE category |  |  |  |
| Note 1: First subframe every 40 ms is allocated for non-MBMS transmission Note 2: Zero OFDM symbols are reserved for PDCCH and no CRS allocated as per TS 36.211 [4] Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 bits is allocated to each Code Block (otherwise L = 0 bits) |  |  |  |

# Conclusions

In this paper we provided our views on the demodulation performance requirements for time-frequency interleaving in LTE-based 5G broadcast. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 to confirm that demod requirements are defined for SCS = 1.25 kHz.

Proposal 2: RAN4 to confirm that demod requirements are defined for (M, N) = (8, 4) and (M, N) = (32, 16).

Proposal 3: RAN4 to confirm the TB sizes of 31704 bits for (M, N) = (8, 4) and of 45352 for (M, N) = (32, 16).

Proposal 4: RAN4 to confirm that demod requirements are defined for 8 MHz channel bandwidth.

Proposal 5: RAN4 to confirm that demod requirements are defined for the channel model channel model in section B.2.6.2-1 in [3] for 5 Hz and 50 Hz.

Proposal 6: RAN4 to confirm that both time and frequency domain interleaving is enabled in the demod test cases.

# References

- RP-250794, Revised WID on LTE-based 5G Broadcast Phase 2, RAN #107, March 2025

- R4-2512620, WF on 5G_Broadcast_Ph2, RAN4 #116, August 2025

- TS 36.101, User Equipment (UE) radio transmission and reception (Release 19), v.19.2.0, June 2025