3GPP TSG-RAN WG4 Meeting #116-bis	                                                                                       R4-2513235

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on future spectrum sharing for 5G NR and 6G

Agenda item:	8.10

Document for:	Approval

# Introduction

The study for 6G Radio Access Technology [1] aims to develop one non-backward compatible radio access technology, and the motivation is deeply rooted in the growing demands and technological evolution expected in the coming decade. It is anticipated that 6G RAT will coexist with NR through spectrum sharing, the Multi-RAT spectrum sharing (MRSS) has become a widely discussed topic. This paper is initially shares some views on future spectrum sharing for 5G NR and 6G.

# Discussion

When introducing the Dynamic spectrum sharing (DSS) scheme for 5G NR to share spectrum with LTE, a critical issue arises: LTE will continuously transmit an “always-on” CRS signal with fixed time-frequency resource allocation, while NR PDSCH need to avoid the CRS signal to ensure no collision between two different RATs. This issue catalysed a key solution of the DSS: CRS rate matching. By puncturing REs that are not available for NR PDSCH, NR UE will be scheduled to avoid the REs used by LTE CRS.

However, in the 6G era, we are fortunate to no longer face this issue, as NR does not have CRS-like signals that occupy the entire spectrum and always on. In contrast, spectrum sharing for 5G/6G will present some new “sweet troubles”. Given the significantly greater flexibility of the NR scheduling compared to LTE, it is foreseeable that spectrum sharing between 5G/6G RATs will resolve the majority of collisions by scheduling. If no CRS-like signal design exists in 6G, these potential scheduling requests will likely to be accomplished by both 5G and 6G RATs.

Therefore, the first priority is to specify the scenario for 5G/6G MRSS. This will help determine the direction for subsequent cooperation between different RATs and prove beneficial for identifying potential solutions.

Observation 1: RAN4 should specify the scenario of 5G/6G MRSS.

From our understanding, it is necessary to clarify whether the spectrum sharing between 5G/6G is tightly coupled. It is foreseeable that spectrum sharing is more likely to be deployed by same operator or co-operators (other operators cannot use certain license spectrum). Furthermore, it is necessary to explore whether MRSS is only applicable to co-location BS or can also be applied to non-co-location BS. When MRSS is deployed in a co-located scenario, the technical solution could be relatively clean and simple. This is because under co-location deployment, 5G and 6G RATs could minimise collisions and maximise spectrum efficiency through a tightly coordinated scheduled mechanism.

However, the non co-coloted scenario should also be considered, which will enhance the flexibility of 6G deployment. Non co-located scenario also introduces new challenges, such as RAT may conduct some unexpected scheduling behaviours, which requires additional design to address potential collisions caused by such unexpected scheduling.

In summary, MRSS requires network side monitoring and precise scheduling, whilst affected UEs need to avoid the occupied time-frequency resources by other RATs. Consequently, specific network side design considerations should be implemented for MRSS to ensure its proper functioning.

Proposal 1: The multi-RAT spectrum sharing should be discussed from network perspective and intra operator, both co-located and non co-located scenario should be considered.

In addition, based on current discussions and relatively common understanding, 6G will be deployed directly by Standalone (SA) mode, but the access resources for 6G RAT might be significantly different from 5G RAT. For instance, from a frequency domain perspective, the spectrum allocations under MRSS deployment for 5G/6G may partially or fully overlap. Moreover, 6G RAT may adopt entirely new synchronization signals designed distinct from 5G SSB. Given that synchronization signals transmission occupies fixed time-frequency resources and is periodic, a robust mechanism should be specified during MRSS design to prevent potential collisions.

Proposal 2: The multi-RAT spectrum sharing should accommodate different synchronization signals design of NR and 6G.

Considering that the NR SSB design has been adopted by millions of existing devices, modifying the NR design might be challengeable. Therefore, the transmission of 6G synchronization signal need to avoid the time-frequency resources occupied by NR SSB during spectrum sharing. As the frequency point of NR SSB transmission usually aligned with the sync raster, the potential frequency point of NR SSB is predictable from a frequency domain perspective. This issue can be resolved in a “tricky” manner: by configuring some offset for 6G sync raster from NR sync raster. Thereafter, the 6G synchronization signal could avoid NR SSB transmission naturally.

However, the specific offset for 6G sync raster might be difficult to derive. For NR sync raster, the interval of sync raster staring from 1200 kHz, and progressively increases with higher frequency (1.44MHz or more). Considering that the NR SSB will occupy 20RBs (approximately 5MHz), if the sync raster design of 6G is similar to NR design, such simple offset will still be unable to accurately avoid NR SSB transmissions. One possible solution is to substantially increase the interval of the 6G sync raster (or reduce it), which may help to stagger the sync raster. Anyway, since the synchronization signal of 6G will be different from NR SSB, a new design of sync raster could also be considered.

Proposal 3: The new sync raster design for 6G could be leveraged in the design of the multi-RAT spectrum sharing mechanism.

In addition, with the further integration of TN and NTN networks, the MRSS should also incorporate NTN networks into its considerations. Unfortunately, given that TN and NTN network are not integrated within NR, they can, to a certain extent, be regarded as two independent legacy networks. Therefore, the MRSS incorporating NTN network should encompass two different, but complementary deployment scenarios: NR TN + 6G NTN, and 6G TN + NR NTN.

Proposal 4: The MRSS should incorporate NTN network in the scope, including 6G TN + 6G NTN,  NR TN + 6G NTN, as well as 6G TN + NR NTN.

# Conclusion

This contribution provides our initial views on future spectrum sharing for 5G NR and 6G. The following proposals are concluded as follows:

Observation 1: RAN4 should specify the scenario of 5G/6G MRSS.

Proposal 1: The multi-RAT spectrum sharing should be discussed from network perspective and intra operator, both co-located and non co-located scenario should be considered.

Proposal 2: The multi-RAT spectrum sharing should accommodate different synchronization signals design of NR and 6G.

Proposal 3: The new sync raster design for 6G could be leveraged in the design of the multi-RAT spectrum sharing mechanism.

Proposal 4: The MRSS should incorporate NTN network in the scope, including 6G TN + 6G NTN, NR TN + 6G NTN, as well as 6G TN + NR NTN.

# References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN #108