3GPP TSG-RAN WG4 Meeting #116bis           	 R4-2513396

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.20.4

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for NR NTN

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WI was endorsed to enhance the NTN for NR with the following objectives， which can be categorized as following [1]

- Objective 1: Downlink coverage enhancements

- Objective 2: Uplink Capacity/Cell Throughput Enhancement

- Objective 3: Signalling of intended service area of a broadcast service

- Objective 4: Support of regenerative payload

- Objective 5: Support RedCap UEs with NR NTN operating in FR1-NTN bands

In the last RAN4 meeting, the test scope and test setup for NR NTN phase 3 was discussed, the related agreement was captured into the WF in [2] as following

- SAN demodulation requirements - PUSCH requirement with OCC length 4 - Consider OCC length 2 only to define PUSCH demodulation requirements - Test setup for OCC length 2 - OCC group configuration - Consider one OCC group with aggregation factor 2 only. Initial RV configured as 0. - SCS/CBW - FDD 15KHz/ 5MHz - FDD 30KHz/10MHz - Frequency offset of two multiplexed UEs - Consider setting the same frequency offset for two UEs. - Evaluate two frequency offsets: 0 Hz and 200 Hz. Down select to one value according to the simulation results. - If there is no performance difference, select 200Hz. - Channel model - Apply the same channel model to the multiplex UEs. - Consider NTN-TDLC5-200. Interested companies can provide the simulation results with NTN-TDLA100-200. - Down select one channel model according to the simulation results. - MCS - Apply inter-slot OCC length 2MCS5 (QPSK, CR=99/1024) in MCS index Table 2 (TS 38.214 Table 6.1.4.1-2) - Test metric - SNR at 70% of maximum throughput - Measure the PUSCH throughputs per UE - UE1 uses Orthogonal sequence index 0 - UE2 uses Orthogonal sequence index 1 - Set the same SNR for PUSCH from two UEs if there is no performance difference - FFS how to write the specification, e.g., define PUSCH demodulation requirements for each UE. - Transmission/Reception signal power - Assume the same receiving power for two UEs at gNB side - Assume the same transmission power from two UEs. - Antenna configuration - 1Tx-1Rx - 1Tx-2Rx - FFS whether to apply the existing SAN PUSCH demodulation requirements applicability between 1Rx and 2Rx requirement or not - Simulation alignment - In RAN4#116bis, interested companies are encouraged to provide simulation results of SAN demodulation for PUSCH with OCC length 2

In this contribution, the view on remaining issue of test setup on NR NTN PUSCH demodulation requirement is provided. Meanwhile, the initial simulation results are provided for alignment purpose

# 2	Test Setup

In this section, the view on the remaining issue of test setup for NR PUSCH requirement was provided.

Frequency offset Assumption

One of key factor to enable inter-slots OCC is that UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Regarding the frequency offset, both 0Hz and 200Hz were considered for evaluation. Based on our evaluation, there is around 0.2dB performance degradation with non-zero frequency offset for two UEs, Since the cross-slot OCC will be very sensitive to Doppler and frequency error, it will impact the orthogonality across slots.

Therefore, we prefer to consider 0Hz as frequency offset for specifying PUSCH requirement with inter-slot OCC feature.

Proposal 1: Configure the same frequency offset as 0Hz for two UEs to specify the PUSCH requirement with inter-slot OCC feature.

Channel Model for two UEs

Regarding the channel model for specifying the PUSCH requirement, both NTN-TDLA100-200 and NTN-TDLC5-200 were considered for further evaluation. Based on the evaluation, one channel model was down selected for requirement.

Based on RAN1 evaluation, the channel model with NTN-TDLC rural was selected. Based on our evaluation, compared with single-UE performance without OCC, the performance degradation with inter-slot OCC enabling in TDLA100-200 channel condition is larger than that in NTN-TDLC channel condition. Therefore, we would like to use the NTN-TDLC5-200 for specifying requirement of PUSCH with inter-slot OCC.

Proposal 2: Select the channel model with NTN-TDLC5-200 for specifying PUSCH requirement with OCC feature.

Antenna configuration

In the last meeting, both 1Rx and 2Rx were considered for requirement definition of NR NTN PUSCH with inter-OCC feature. In Rel-17/Rel-18, RAN4 has also introduced the SAN demodulation for both 1Rx and 2Rx antenna structure considering the implementation of different vendors. To reduce the test efforts and allow the implementation of different vendors, the corresponding test applicability was introduced, and the related tested will be proceeded pending declaration.

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.48 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. If performance requirement is not specified for the highest number of supported connectors, the tests shall be done by using performance requirement for the closest number of connectors lower than this highest number of supported connectors.

We think the same test applicability rule for different antenna configuration can be applied for OCC feature

Proposal 3: RAN4 apply the same test applicability rule for different antenna configuration for NR NTN PUSCH with inter-OCC feature testing

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.48 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. If performance requirement is not specified for the highest number of supported connectors, the tests shall be done by using performance requirement for the closest number of connectors lower than this highest number of supported connectors.

SNR requirement

For NR NTN with inter-slot OCC feature, two UEs will be configured for transmission with the same channel condition and same frequency offset. It was expected the requirement for each UE PUSCH transmission should be same. Regarding how to write the specification, RAN4 will further discuss. Since UE is differentiated with the OCC index, we can set the SNR per UE, similar as multiple-TRP scenario as following example

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 2 | 0 | TBD |
|  |  |  |  |  |  |  | 1 | TBD |

Proposal 4: RAN4 can consider the following example to specify the PUSCH SNR requirement with inter-slot OCC feature

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 2 | 0 | TBD |
|  |  |  |  |  |  |  | 1 | TBD |

# 3	Simulation Results

In this section, based on agreed simulation assumption, the initial simulation results are provided for alignment purpose

Table 1: Simulation Assumption for PUSCH requirement with supporting inter-slot OCC

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | UE0 | UE1 |
|  |  | 0 | 1 |
|  | DM-RS sequence generation | U, V=0, group hopping and sequence hopping are disabled |  |
| Time domain resource assignment | PUSCH mapping type | A, B |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |

Table 2: Initial simulation results for 15KHz with supporting inter-slot OCC, mapping A

| CASE | UE | Tx/Rx | SCS | BW | DMRS | Channel | Target Tput per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -10.53 |
| 1 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -7.75 |
| 2 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -9.69 |
| 3 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -6.26 |
| 4 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | 2 | -10.59 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -10.55 |
| 5 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -10.4 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -10.32 |
| 6 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz |  |  | -7.36 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -7.28 |
| 7 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -7.14 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -7.09 |
| 8 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | 2 | -7.44 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -7.36 |
| 9 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -7.27 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -7.24 |
| 10 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0 Hz | 2 | 2 | 1.7 |
|  | 1 |  |  |  |  |  |  |  | 0 Hz |  |  | 1.85 |
| 11 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | 1.71 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | 1.74 |

Table 3: Initial simulation results for 30 KHz with supporting inter-slot OCC, mapping A

| CASE | UE | Tx/Rx | SCS | BW | DMRS | Channel | Target Tput per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -10.35 |
| 1 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -7.38 |
| 2 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -9.59 |
| 3 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | N/A | -6.17 |
| 4 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | 2 | -10.54 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -10.58 |
| 5 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -9.75 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -9.79 |
| 6 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 0Hz | 2 | 2 | -7.32 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -7.32 |
| 7 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -6.52 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -6.53 |
| 8 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | 2 | -9.58 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -9.56 |
| 9 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -8.88 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -8.85 |
| 10 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 0Hz | 2 | 2 | -5.25 |
|  | 1 |  |  |  |  |  |  |  | 0Hz |  |  | -5.26 |
| 11 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLA100-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -4.37 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -4.38 |

Compared with the single UE without OCC, there is a minor performance degradation for TDLC channel condition with inter-slot OCC enabling, while large performance degradation was suffered for TDLA channel condition.

Compared with zero frequency offset scenario for each UE, around 0.7 dB performance degradation was suffered with non-zero frequency offset for both NTN TDLC5-200 and NTN TDLA100-200 channel condition, especially for 30KHz SCS scenario.

Observation 1: Compared with the single UE without OCC, there is a minor performance degradation for TDLC channel condition with inter-slot OCC enabling, while large performance degradation was suffered for TDLA channel condition

Observation 2: Compared with zero frequency offset scenario for each UE, around 0.7 dB performance degradation was suffered with non-zero frequency offset for both NTN TDLC5-200 and NTN TDLA100-200 channel condition, especially for 30KHz SCS scenario.

# 4	Conclusion

In this contribution, the view on the test setup for NR NTN PUSCH with inter-slot OCC feature demodulation requirement was provided. Meanwhile, the initial simulation results are provided for alignment purpose

Proposal 1: Configure the same frequency offset as 0Hz for two UEs to specify the PUSCH requirement with inter-slot OCC feature.

Proposal 2: Select the channel model with NTN-TDLC5-200 for specifying PUSCH requirement with OCC feature.

Proposal 3: RAN4 apply the same test applicability rule for different antenna configuration for NR NTN PUSCH with inter-OCC feature testing

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.48 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. If performance requirement is not specified for the highest number of supported connectors, the tests shall be done by using performance requirement for the closest number of connectors lower than this highest number of supported connectors.

Proposal 4: RAN4 can consider the following example to specify the PUSCH SNR requirement with inter-slot OCC feature

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 2 | 0 | TBD |
|  |  |  |  |  |  |  | 1 | TBD |

Observation 1: Compared with the single UE without OCC, there is a minor performance degradation for TDLC channel condition with inter-slot OCC enabling, while large performance degradation was suffered for TDLA channel condition

Observation 2: Compared with zero frequency offset scenario for each UE, around 0.7 dB performance degradation was suffered with non-zero frequency offset for both NTN TDLC5-200 and NTN TDLA100-200 channel condition, especially for 30KHz SCS scenario.

# References

RP-240775, Non-Terrestrial Networks (NTN) for NR Phase 3,

R4-2508755, Way Forward for [116][331] NR_NTN_Ph3_demod