3GPP TSG-RAN WG4 Meeting #116bis	R4-2513428

# Prague, Czech Republic, October 13 – 17, 2025

Title: 	Discussion on 6Rx requirements with interference

Source: 	Huawei, HiSilicon

Agenda item:	7.5.4

Document for:	Discussion

Background

A new WID NR demodulation performance: Phase 6[1] was approved, where one objective is to define 6Rx UE requirements with interference:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

In contribution we provide our overviews.

- Discussions

General

Rx correlation will reduce the spatial degree of freedom to mitigate interference, resulting in bad performance under the presence of interference, hence it’s beneficial to evaluate the performance with certain UE antenna correlation for handheld UE with interference. For FWA, low correlation can be considered.

Proposal 1: Evaluate the performance with certain UE antenna correlation for handheld UE with interference, such as α = 0 and β = 0.07179.

According to the WID, two scenarios shall be considered: one is case with inter-cell interference and the other is case with intra cell inter user interference. We will discuss them one by one in the below text:

PDSCH requirements with inter-cell interference

To save the time for discussion, it’s proposed to reuse the test setup and interference model of 2Rx/4Rx requirements, except for rank and MCSthat need further evaluation and only TDD should be considered.

Proposal 2: RAN4 to reuse the interference model used for 2Rx/Rx performance requirements definition, further discuss rank and MCS withonly TDD considered.

For 2Rx/4Rx requirements, Rank1 with MCS 13 was selected, considering 6Rx UE require lower SNR, it’s proposed to consider Rank1 with MCS 17 for evaluation to ensure the reasonable SNR range.

Proposal 3: RAN4 to use Rank1 and MCS 17 as starting point for evaluation.

PDSCH requirements with MU-MIMO

Rank 2+2 would be sufficient and it’s proposed to reuse the test setup from 2Rx/4Rx requirements except the MCS that need further evaluation, MCS17 can considered as starting point.

Proposal 4: RAN4 to consider Rank 2+2 and reuse test setup from 2Rx/4Rx requirements, but consider MCS17 as starting point for evaluation.

CQI reporting requirements with inter-cell interference

Checking the progress of Rel-19 6Rx requirements discussion, CQI requirements under fading channel has been precluded and there is no conclusion on CQI requirements under AWGN. As discussed in Rel-19 requirements WI, CQI index acquired by using 6Rx is not expected to change so much compared to 4Rx since the SINR is increased by  10log64=1.7dB. Furthermore, it seems strange that RAN4 define advanced requirements (CQI requirements with interference) under the condition that there are no basic requirements (CQI requirements under fading channel) in the spec.

Proposal 5: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

- Conclusion

In this contribution we provide our general views on 6Rx requirements with interference, the proposals are:

General

Proposal 1: Evaluate the performance with certain UE antenna correlation for handheld UE with interference, such as α = 0 and β = 0.07179.

PDSCH requirements with inter-cell interference

Proposal 2: RAN4 to reuse the interference model used for 2Rx/Rx performance requirements definition, further discuss rank and MCS withonly TDD considered.

Proposal 3: RAN4 to use Rank1 and MCS 17 as starting point for evaluation.

PDSCH requirements with MU-MIMO

Proposal 4: RAN4 to consider Rank 2+2 and reuse test setup from 2Rx/4Rx requirements, but consider MCS17 as starting point for evaluation.

CQI reporting requirements with inter-cell interference

Proposal 5: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

- Reference

[1]   RP-252957	New WID: NR demodulation performance: Phase 6