3GPP TSG-RAN WG4 Meeting #116bis	R4-2513431

Prague, Czech Republic, October 13 – 17, 2025

Title: 	On demodulation requirements for NonCol_intraB_ENDC_NR_CA for new type UE

Source: 	Huawei, HiSilicon

Agenda item:	6.2.3

Document for:	Discussion

Background

RAN4 shall start the discussion on demodulation requirements for Non-collocated CA/ENDC scenario for type 4 UE. This contribution provides our over views.

Discussion

In Rel-19, Type 4a and Type 4b UE were introduced.  Compared to Type 2 UE which support 2Rx/2layers on each CC, type 4a UE support 2Rx/2layers on one of the CCs and 4Rx/4layers on the other CC and type 4b support 4Rx/4layers on both CCs.  It is noted that Type 4a UE is not supported for CA scenario. According to the RF agreement, Type 4 UE support maximum 25dB power imbalance and 33 us MRTD, which is same as Type 2 UE.

Based on the above discussion, it’s proposed to only define CA requirements with Type 4a receiver. Generally, the Rel-18 non-collocated CA test setup can be reused. We propose the following test setup as starting point:

CC1: 2x4, Rank2, QPSK, 0.30

CC2: 2x4, Rank2, 256QAM, 0.69

MRTD:33us, Power imbalance: 25dB

Proposal 1: Only define CA requirements with Type 4a  receiver.

Proposal 2: Use following test setup as starting point:

- CC1: 2x4, Rank2, QPSK, 0.30

- CC2: 2x4, Rank2, 256QAM, 0.69

- MRTD:33us, power imbalance: 25dB

Considering type 4 UE has 8Rx and each CC should be transmitted in 4 out of 8Rx, the antenna connection rules should be specified:

“For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx,  4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration”

Proposal 3: Specify the following antenna connection:

- “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration”

- Conclusion

In this contribution we provide our views on demodulation requirements for NonCol_intraB_ENDC_NR_CA for new type UE. The proposals are:

Proposal 1: Only define CA requirements with Type 4a receiver.

Proposal 2: Use following test setup as starting point:

- CC1: 2x4, Rank2, QPSK, 0.30

- CC2: 2x4, Rank2, 256QAM, 0.69

- MRTD:33us, power imbalance: 25dB

Proposal 3: Specify the following antenna connection:

- “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration”