3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513522

Prague, Czech Republic, October 13th ‒ October 17th, 2025

Agenda item:			6.2.1

Source:	Moderator (Ericsson)

Title:	Topic summary for [116bis][216] NonCol_intraB_ENDC_NR_CA_Ph2_demod

Document for:	Information

# Introduction

This topic summary summarizes the issues on Rel-19 WI: Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2 (RP-240838).

This topic summary treats AI 6.2.2.

# Topic #1: Work plan

This topic discusses the work plan of WI Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2 UE demodulation performance part.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513699 | KDDI, Ericsson | Work plan |

## Open issues summary

This issue discusses the work plan of UE demodulation performance part.

### Sub-topic 1-1	Work plan

Issue 1-1-1: Work plan

- Work plan proposal

  - RAN4#116bis, October 2025 (0.25 TU for Demod/RRM performance)

    - Discuss the scope of UE demodulation requirements.

    - Agree with the initial simulation assumption.

  - RAN4#117, November 2025 (0.25 TU for Demod/RRM performance)

    - Update the simulation assumption if necessary.

    - Collect the initial simulation results.

    - Decide the CR work split.

  - RAN4#118, February 2026 (0.25 TU for Demod/RRM performance)

    - Collect the simulation results.

    - Agree with the draft CRs for TS38.101-4.

    - Agree with the big CR for TS38.101-4 (email approval).

- Recommended WF

  - Confirm the work plan.

# Topic #2: UE demodulation requirements

This topic discusses the UE demodulation requirements.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513431 | Huawei, HiSilicon | Proposal 1: Only define CA requirements with Type 4a receiver. Proposal 2: Use following test setup as starting point: - CC1: 2x4, Rank2, QPSK, 0.30 - CC2: 2x4, Rank2, 256QAM, 0.69 - MRTD:33us, power imbalance: 25dB Proposal 3: Specify the following antenna connection: - “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration” |
| R4-2513976 | Ericsson | Observation 1: In Rel-18, RAN4 defined Type 2 UE PDSCH demodulation requirements for TDD+TDD NR-CA scenario. Observation 2: Rel-18 Type 2 UE demodulation requirement configures rank 1 with MCS4 in 64QAM table (QPSK 0.30) on for one carrier and rank 2 with MCS21 in 256QAM table (256QAM 0.69) for another carrier. Observation 3: Rel-18 Type 2 UE demodulation requirement configures the timing offset of the SCell from the PCell to 33us. Observation 4: In Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank so that the difference of SNRs of two CCs is less than 25dB. Proposal 1: Define Type 4a UE PDSCH demodulation requirement for 2CC NR-CA scenario, with the configuration FR1 TDD SCS=30kHz + FR1 TDD SCS=30kHz. Proposal 2: RAN4 defines Type 4 UE demodulation requirement, where at least one CC configures rank 3 or rank 4. Proposal 3: RAN4 defines Type 4 UE demodulation requirement where the timing offset of the SCell from the PCell is 33us. Proposal 4: RAN4 sets the MCS/rank of two CCs for Type 4 UE demodulation requirements so that the difference of SNRs of two CCs is less than 25dB, where the SNRs are average of companies’ impairment results. Proposal 5: Assume Tx EVM=3% for both QPSK and 256QAM as same assumption as Rel-18 Type 2 UE demodulation requirements. Proposal 6: Configure CBW/SCS=40MHz/30kHz and 4x4 static channel in TS 38.101-4 B.1.1. Proposal 7: RAN4 should evaluate the following MCS/rank combinations for Type 4 UE demodulation requirements: rank 2 with MCS4/MCS5 in 64QAM MCS table for CC#1 and rank 4 with MCS21/MCS22 in 256QAM MCS table for CC#2. |
| R4-2514287 | Nokia | Observation 1: The objective of the core part provides the following assumptions: up-to 4-layer DL MIMO per CC, MRTD>CP and up to 25 dB power imbalance. Observation 2: With power imbalance of up to 25 dB, each CC can have different MCS and different number of layers. Observation 3: For 2Rx case, TS 38.101-4 Clause 5.2A.2.6 uses the following parameters: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for lower power carrier and 256QAM with code rate 0.69 for the higher power carrier Observation 4: Type 4a FWA UE has 4 antennas for CC#1 and 2 antennas for CC#2, while Type 4b FWA UE has 4 antennas for both CCs. Observation 5: The 25dB power difference is hard on the TEs. While AWGN channels can be used to get enough power out of them, they may not be MIMO/multi-layer compatible. Proposal 1: RAN4 to specify PDSCH requirements for NR-CA with 2 CCs with different MCSs and number of layers for the 2 CCs to reflect the power imbalance, in which at least one of the CC shall take the maximum possible number of layers. Proposal 2: The following parameters can be used as a starting point: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for the lower power carrier and 256QAM with code rate 0.69 for the higher power carrier. Proposal 3: In the case of Type 4a FWA UE, the lower power carrier is assumed to use the lower number of Rx antennas, and, hence, the lower number of layers. Proposal 4: RAN4 to discuss how to enable up-to 4-layer if AWGN channels are used. |
| R4-2514290 | Qualcomm | Observation 1: Existing requirements for NR UE supporting non-colocated intra-band NR CA are defined with rank equal [2,1] for the strong and weak CC respectively, The SNR imbalance between the two CCs is 24.2dB. Proposal 1: RAN4 to introsduce NR-CA PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. Proposal 2: RAN4 not to introduce EN-DC PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. Proposal 3: If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, they should be applicable only if the UE indicates support for the intraBandNR-CA-non-collocated-r19 features and providing nonCollocatedTypeNR-CA-v19xy with value “type4”. Proposal 4: RAN4 should confirm with RAN5 that the existing test condition for intraBandNonColocatedCA-r18 which defines a minimum gap of 80MHz for the two CCs is applicable to new requirements in this WI, if introduced. Proposal 5: Simulation assumptions should align with the existing requirements in TS 38.101-4, Section 5.2A.2.6 where applicable. Proposal 6: The Rank to be considered for initial evaluation should be Rank 4 for the “stronger” CC, and Rank 2 for the “weaker” CC. Proposal 7: The final choice of MCS and Rank should consider a maximum SNR difference of 25dB. |

## Open issues summary

### Sub-topic 2-1 Test scope

Issue 2-1-1: Test scenario

- Proposals

  - Only define CA requirements with Type 4a receiver (Huawei, HiSilicon)

  - Define Type 4a UE PDSCH demodulation requirement for 2CC NR-CA scenario, with the configuration FR1 TDD SCS=30kHz + FR1 TDD SCS=30kHz. (Ericsson)

  - RAN4 to specify PDSCH requirements for NR-CA with 2 CCs with different MCSs and number of layers for the 2 CCs to reflect the power imbalance, in which at least one of the CC shall take the maximum possible number of layers. (Nokia)

  - RAN4 to introsduce NR-CA PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. (Qualcomm)

  - RAN4 not to introduce EN-DC PDSCH demodulation simulation for Type 4 UEs supporting intra-band non-collocated. (Qualcomm)

- Observations

  - Type 4a FWA UE has 4 antennas for CC#1 and 2 antennas for CC#2, while Type 4b FWA UE has 4 antennas for both CCs. (Nokia)

- Recommended WF

  - Moderator: According to UE RF conclusion, NR-CA is only applicable for Type 4b (See Appendix).

  - Define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario.

    - Specify FR1 TDD + FR1 TDD only.

Issue 2-1-2: Received time difference between two carriers

- Observation (Ericsson):

  - Rel-18 Type 2 UE demodulation requirement configures the timing offset of the SCell from the PCell to 33us.

- Proposals (Huawei, HiSilicon, Ericsson):

  - Timing offset of the SCell from the PCell is 33us.

- Recommended WF

  - Configure the timing offset of the SCell from the PCell is 33 µs.

### Sub-topic 2-2 Simulation assumption

Issue 2-2-1: Channel bandwidth and SCS

- Proposals (Ericsson, Nokia, Qualcomm):

  - CBW=40MHz and SCS=30kHz

- Recommended WF

  - Configure CBW=40MHz and SCS=30kHz for both CCs.

Issue 2-2-2: MCS/rank

- Proposals

  - Option 1 (Huawei, HiSilicon):

    - CC#1: Rank 2, QPSK 0.30

    - CC#2: Rank 2, 256QAM 0.69

  - Option 2 (Ericsson):

    - CC#1: Rank 2, MCS4/MCS5 in 64QAM MCS table

    - CC#2: Rank 4, MCS21/MCS22 in 256QAM MCS table

  - Option 3 (Nokia):

    - QPSK with code rate 0.30 for the lower power carrier and 256QAM with code rate 0.69 for the higher power carrier.

  - Option 4 (Qualcomm):

    - The Rank to be considered for initial evaluation should be Rank 4 for the “stronger” CC, and Rank 2 for the “weaker” CC.

- Recommended WF

Evaluate the following rank/MCS options.

  - For weaker CC:

    - Option 1: Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1)

    - Option 2: Rank 2 + QPSK 0.37 (MCS5 in MCS index table 1)

  - For stronger CC:

    - Option 1: Rank 2 + 256QAM 0.69 (MCS21 in MCS index table 2)

    - Option 2: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2)

    - Option 3: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2)

Issue 2-2-3: Channel model and antenna configuration

According to the RAN4 RF assumption, Type 4b UE has 8 Rx antennas, and it can use 4 of 8Rx antennas for one CC and other 4 of 8Rx antennas for another CC. In Rel-18, Type 2 UE demodulation requirements set 2x2 static channel model.

- Proposals

  - Use 2x4 (Huawei)

  - Specify the following antenna connection: “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration” (Huawei, HiSilicon)

  - Use 4x4 static channel in TS 38.101-4 B.1.1 (Ericsson)

  - In the case of Type 4a FWA UE, the lower power carrier is assumed to use the lower number of Rx antennas, and, hence, the lower number of layers (Nokia)

  - RAN4 to discuss how to enable up-to 4-layer if AWGN channels are used. (Nokia)

- Recommended WF

  - Use the static channel model specified in TS 38.101-4 B.1.1.

    - Option 1: 2x4 for each CC

    - Option 2: 2x4 for weaker CC, 4x4 for stronger CC

    - Option 3: 4x4 for each CC

  - Specify the following antenna connection in TS 38.101-4:

    - “For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration”

Issue 2-2-4: How to choose MCS/rank

- Observation (Ericsson):

  - In Rel-18 Type 2 UE demodulation requirement, RAN4 set MCS/rank so that the difference of SNRs of two CCs is less than 25dB.

- Proposals

  - Power imbalance: 25dB (Huawei, HiSilicon)

  - RAN4 sets the MCS/rank of two CCs for Type 4 UE demodulation requirements so that the difference of SNRs of two CCs is less than 25dB, where the SNRs are average of companies’ impairment results (Ericsson)

  - The final choice of MCS and Rank should consider a maximum SNR difference of 25dB (Qualcomm)

- Recommended WF

  - The final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results.

Issue 2-2-5: Tx EVM assumption

- Proposals (Ericsson)

  - Assume Tx EVM=3% for both QPSK and 256QAM as the same assumption as Rel-18 Type 2 UE demodulation requirements.

- Recommended WF

  - Assume Tx EVM=3% for both CCs.

### Sub-topic 2-3 Applicability

Issue 2-3-1: Separation of two CCs

- Proposals

  - RAN4 should confirm with RAN5 that the existing test condition for intraBandNonColocatedCA-r18 which defines a minimum gap of 80MHz for the two CCs is applicable to new requirements in this WI, if introduced (Qualcomm).

- Recommended WF

  - Discuss whether to send a LS to RAN5 on the separation of CC as same as Rel-18.

  - Moderator thinks it is reasonable to send a similar LS to RAN5 as we have sent in Rel-18, because of the fact that:

    - In Rel-18, RAN4 sent LS R4-2316951 to RAN5.

RF conformance tests should introduce the following test condition in RAN5: - Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R18. The same agreement should be applied to NR-CA PDSCH demodulation requirement.

    - In RAN4#116, RF session agreed to send LS R4-2511768 to RAN5.

RAN4 agreed that the following test condition should be introduced in the corresponding conformance test case for both type of UEs. - Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC in R19

Issue 2-3-2: Test applicability

- Proposals

  - If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, they should be applicable only if the UE indicates support for the intraBandNR-CA-non-collocated-r19 features and providing nonCollocatedTypeNR-CA-v19xy with value “type4”. (Qualcomm)

- Recommended WF

  - If new PDSCH demodulation requirements for Type 4 UEs supporting intra-band non-collocated NR-CA are introduced, it is applicable for UEs with the capability of:

    - Supporting ‘intraBandNR-CA-non-collocated-r19’,

    - Providing ‘nonCollocatedTypeNR-CA-v1900’ with value ‘type4’.

# Appendix (WID: RP-242083, WF: R4-2217734)

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4a | 1 (NR) | 4 | 6 total | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and FWA |
|  | 2 (LTE/NR) | 2 |  | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 8 total | 4 | 4Rx | NR-CA EN-DC | 25dB full range | Requires 8 antennas and LNA |
|  | 2 (LTE/NR) | 4 |  | 4 | 4Rx |  |  |  |