# 3GPP TSG-RAN WG4 Meeting #116bis		R4-2513592

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Apple

Title:	On RAN4 Impacts for RRM measurement prediction in AIML Mobility

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis in the scope of Rel-19 SI ‎[1] and ‎‎[2]. Agreements achieved during this meeting are summarized in the WF ‎[3].

Some of the issues related to AI/ML Mobility RRM measurement prediction use case require further discussion. In this paper, we provide some additional views on issues related to RRM requirements a

Discussion

Performance Metrics

The following scenarios have been prioritized as high priority in RAN2:

| Scenario | Prioritization | Evaluation scenario combination | Objective | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | Measurement reduction | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | Measurement reduction | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | Mobility optimization | Intra-cell |

Cell Level Relative Accuracy

During RAN4#115 meeting, the following agreement was made on relative RSRP accuracy.

Issue 2-1: Relative RSRP accuracy definition

Agreement:

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

During RAN4#116 the following agreement was reached:

Relative RSRP accuracy definition

Agreement:

Relative RSRP accuracy definition

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

Relative RSRP accuracy evaluates how well a prediction preserves the future power difference between two cells on the same frequency.
At time t, cell 1 predicts its RSRP at t + Δt1, and cell 2 provides a measured or predicted RSRP at t + Δt2.
The predicted differential is

( P_pred1(t + Δt1) - P_pred2(t + Δt2) )

and the true differential is

( P_true1(t + Δt1) - P_true2(t + Δt2) )

The relative RSRP accuracy is the difference between these two differentials:

( P_pred1(t + Δt1) - P_pred2(t + Δt2) )

- ( P_true1(t + Δt1) - P_true2(t + Δt2) )

This expression quantifies how closely the predicted cell-to-cell RSRP gap matches the actual gap at their respective future times.

RAN4 is invited to adopt a clear time-indexed definition of relative RSRP accuracy for intra-frequency temporal prediction. At time t, cell 1 predicts its Layer-3 RSRP at t + Δt1, while cell 2 provides a reported (measured or predicted) RSRP at t + Δt2. The metric compares the predicted power gap to the true power gap at those future times and is expressed as ( P_pred1(t + Δt1) - P_rep2(t + Δt2)) -( P_true1(t + Δt1) - P_true2(t + Δt2))

This definition captures how accurately the prediction preserves the cell-to-cell RSRP difference that drives mobility decisions, while explicitly recognizing that cell 2’s future RSRP may be either measured or predicted when reported.

From RAN4#115 we concluded on FFS ” Relative RSRP accuracy for Beam level measurements “

The relative beam-level RSRP accuracy could be defined as the difference between the predicted (or measured) beam-level RSRP of a serving or neighbor beam and a reference beam, compared to the corresponding difference in their ground truth RSRP values, where both beams belong to cells operating on the same frequency layer. Mathematically: [Predicted RSRPBeam 1-Reported or Predicted RSRPBeam 2-Ground Truth RSRPBeam 1-Ground Truth RSRPBeam 2]

Beam 1 and Beam 2 are two beams of interest, either from the same cell or from different intra-frequency neighbor cells.

It remains for further study (FFS) how the testing of relative RSRP accuracy should extend to beam-level RSRP measurements or predictions within the same cell because several open technical questions and ambiguities have not yet been fully addressed by RAN4 or RAN2.

A further challenge lies in the ambiguity of the reference definition. At the cell level, the reference is typically unambiguous, one serving cell compared against a neighbor. At the beam level, the choice is less clear. Should the reference always be the serving beam within the serving cell, or could it be any beam in the same cell? Should the metric consider “best-beam vs. secondary-beam” comparisons within a cell, or only “best beam” comparisons across cells? Each choice leads to different statistical characteristics and potential biases in the KPI. For example, measuring relative accuracy between best and secondary beams may highlight intra-cell angular prediction fidelity, while using only best-beam references may emphasize inter-cell competitiveness in beam selection. This ambiguity needs resolution to ensure a consistent and interpretable KPI definition.

The proposed metric compares the predicted (or measured) RSRP of one beam against a reference beam and subtracts the corresponding ground-truth difference. Two key ambiguities remain:

- Domain of the prediction (time vs. frequency):
It is not yet agreed whether the “predicted RSRP” refers to temporal prediction (e.g., beam 1 at t+Δt predicted from t) or to frequency-domain inference (e.g., predicting another layer’s RSRP from current-layer data).

- Relationship between Beam 1 and Beam 2:
The formula allows Beam 1 and Beam 2 to belong either to the same cell or to different intra-frequency neighbor cells, but the choice of reference is not yet fixed.
– If Beam 2 is always a serving beam in the same cell, the KPI measures intra-cell angular prediction fidelity(best vs. secondary beam).
– If Beam 2 can be any beam across cells, the KPI emphasizes inter-cell competitiveness in beam selection (best-beam vs. neighbor-beam).

Clarify Beam-level Relative RSRP Accuracy KPI

RAN4 shall explicitly define the beam-level relative RSRP accuracy metric, covering both the prediction domain and the valid reference-beam relationship, to enable consistent and interoperable testing.

- Specify whether the KPI applies to temporal prediction (future-time beam RSRP) or frequency-domain inference (other-layer RSRP), or provide separate formulas for each.

- Define the reference-beam relationship: intra-cell (best vs. secondary beam) or inter-cell (best vs. neighbor beam), or adopt distinct KPI sub-modes.

- Provide the exact formula (Pred B1 – Pred/Reported B2) – (GT B1 – GT B2) and state how measured vs. predicted values are used in each case.

Inter-frequency Relative Accuracy

In RAN4#115 the following agreement was reached:

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

In RAN4#116 the following agreement was reached:

Agreement:

Relative RSRP accuracy definition,

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

- Note: It is not precluded to update the definition based on further RAN2 progress in WI phase.

According to our understanding,  for inter-frequency L3-RSRP, the proposed relative-accuracy metric covers two practical cases.

(A) Time-domain, cross-frequency comparison: cell 1 on f1 predicts its RSRP at t+Δt1, P_pred1_f1(t+Δt1), while cell 2 on f2 (same FR as f1) provides a reported value at t+Δt2, P_rep2_f2(t+Δt2) (where “reported” may be measured or predicted). The metric compares the predicted inter-cell gap to the true inter-cell gap at those future times: ( P_pred1_f1(t+Δt1) − P_rep2_f2(t+Δt2) ) − ( P_true1_f1(t+Δt1) − P_true2_f2(t+Δt2) ). This isolates temporal prediction error when the two cells reside on different frequencies within the same FR. The inter-cell gap is simply the difference in received power (RSRP) between two cells at the times and frequencies of interest.
It represents how much stronger one cell’s signal is than the other’s and is the key quantity used for mobility and handover decisions.

(B) Frequency-domain prediction across cells: cell 1 on frequency f1 is predicted across frequency from another cell or cells, while cell 2 on frequency f2 provides a reported value that may itself be predicted across frequency from other cell(s) or directly measured. This case captures inter-frequency prediction where both the target cell and the reference cell can rely on cross-frequency inference, and the relative RSRP accuracy is evaluated by comparing the predicted inter-cell power gap with the corresponding true gap.

New Definition of Relative Inter-Frequency L3-RSRP Accuracy

RAN4 shall define relative inter-frequency accuracy through two complementary cases to ensure consistent evaluation of cross-frequency prediction.

- Case A – Time-domain, cross-frequency: Cell 1 on f₁ predicts its L3-RSRP at t + Δt₁ while Cell 2 on f₂ (same FR) provides a reported value at t + Δt₂ (measured or predicted); the KPI compares predicted vs. true inter-cell power gaps to assess future-time signal differences.

- Case B – Frequency-domain, cross-cell: Cell 1 on f₁ is predicted across frequency using measurements from other cells, while Cell 2 on f₂ provides a reported value (predicted or measured); the KPI compares predicted vs. true inter-cell power gaps for cross-frequency inference.

- Explicitly state the mathematical KPI as (Pred f1 – Pred/Reported f2) – (GT f1 – GT f2) for both cases.

Ground Truth Derivation

In the RAN4#114 meeting, the following agreements were made for FR1 Ground truth definition for RSRP accuracy:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Following agreement was reached in RAN4#114bis meeting:

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Following is the RAN4#114 agreement for AIML Beam management usecase:

Agreement: - Use CDL-based channel model as starting point

Alternative 1, the ground truth for FR1 can be derived by TE based on the transmitted or received power:

Alternative 1 proposes deriving the FR1 ground truth (GT) from the transmitted or received power at the test equipment (TE), consistent with the legacy L3 RSRP accuracy method. This works well in the AWGN channel model used in past conformance tests, where signal power is constant over time and space. However, in fading CDL channels, which RAN4 has agreed to consider for FR2 beam management this method becomes problematic. The rapid variations in channel conditions make it difficult to align the exact time periods of measured and predicted samples, which can introduce GT inaccuracies.

Additionally, the TE does not have access to the UE’s proprietary L1 filtering parameters, such as filtering coefficients and the number of samples, further complicating accurate GT derivation. While the main advantage of Alternative 1 is that it is cheat-proof, since the UE cannot influence TE-based calculations, it is not well suited for AI/ML mobility in fading channels. The combination of channel variability and hidden UE filtering behavior can lead to significant mismatches between the derived GT and the UE’s actual measurement context.

Alternative 1, where the FR1 ground truth is derived at the TE from transmitted or received power (legacy approach), is not well suited for AI/ML mobility scenarios. In fading CDL channels, rapid signal fluctuations make precise time alignment between measured and predicted samples difficult, leading to potential inaccuracies in ground truth. Furthermore, the TE lacks knowledge of the UE’s proprietary L1 filtering parameters, such as filtering coefficients and sample counts, which further limits its ability to replicate the UE’s measurement context.

Alternative 2, the ground truth for FR1 is derived based on the measurements reported by UE based on certain conditions:

Alternative 2 derives the FR1 ground truth from the measurements reported by the UE under specific conditions. This approach avoids the complexity of accounting for channel variations or UE implementation-specific parameters (such as L1 filtering details) since these factors are inherently handled by the UE when generating measurement reports. As a result, the TE receives ground truth values already aligned with the UE’s internal processing, simplifying the derivation process.

A potential challenge with this method arises when a timing window is configured, as some UEs may not be capable of measuring and predicting simultaneously. For temporal domain Case A, this is not an issue since measurements are always performed and can be reported as ground truth. For temporal domain Case B, where some measurements are skipped, continuous measurement values are not always available. This can be addressed by enabling the UE to perform legacy measurements and AI/ML predictions in parallel (when capable) or by running two separate test iterations, one for legacy measurement reporting (to establish ground truth) and one for AI/ML prediction, under identical channel conditions.

S(I)NR variations during the test may also pose a challenge for this alternative, as prediction and measurement could occur under slightly different conditions. This can be mitigated by maintaining an average SINR during the test and ensuring that ground truth measurements are taken at higher SINR levels for accuracy.

Additionally, safeguards must be in place to prevent UE cheating, which can be achieved through dedicated test procedures, as outlined in supporting documentation.

Alternative 2, where the FR1 ground truth is derived from UE-reported measurements under defined conditions, is better suited for AIML Mobility because channel variations and L1 filtering effects are inherently accounted for by the UE when producing the ground truth measurements

Anti-cheating strategy for Alt-2 testing

- Twin-cell single-run
Concept: The test equipment (TE) creates two parallel cells with identical channel realizations but a secret power offset. In a single run, the UE must (a) predict Cell A’s RSRP over the prediction window and (b) measure Cell B’s RSRP over the same window. Cell B serves as ground truth.

  - The UE must submit its prediction for time t before the measurement at t exists, so it cannot copy a later measurement into the prediction field.

  - A hidden, fixed (or time-varying) power offset known only to the TE prevents simply reusing Cell A predictions as Cell B measurements.

  - In the twin-cell single-run setup the prediction and the ground-truth measurement are generated during the same test run and at the same instant in time, just on two different cells (Cell A and Cell B). Because both cells experience exactly the same instantaneous fading realization, there is no need to repeat the scenario in a second run to collect ground truth.

Based on the discussion above we have the following proposal:

Anti-cheating strategy for Alt-2 testing (Twin-cell single-run)

RAN4 shall adopt a twin-cell test method to guarantee that reported prediction accuracy reflects true model performance rather than post-run adjustments.

- Create two parallel cells with identical fading but a hidden, fixed or time-varying power offset known only to the test equipment.

- Require the UE to predict Cell A’s L3-RSRP for future times while simultaneously measuring Cell B’s RSRP as ground truth.

- Enforce time-stamped prediction submission before measurements exist, preventing back-filling.

- Because prediction and measurement occur simultaneously on two synchronized cells, any RSRP gap directly indicates genuine prediction error, not fading drift.

Measurement Error Model for AI/ML Mobility

### In this section we propose an RF-Error Model for mobility measurements based on TS 38 300 Fig 9.2.4-1.

Figure 1: 3GPP TS 38.300 Measurement Model

During RAN4#116 the following agreement was reached:

Impacts of measurement error on prediction accuracy

Agreement:

RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy:

- As a baseline:

  - For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

  - For RF error model, use a truncated Gaussian distribution.

Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains

## Baseband (BB) Error Model

Purpose: Capture inaccuracies that occur inside the UE’s baseband receiver and signal processing (e.g., channel estimation, filtering, quantization).

- Origin – purely digital/PHY domain (post-RF), such as estimator noise and finite-resolution effects.

- Dependence on SINR – BB error is not constant; its spread grows when the instantaneous SINR is low and shrinks at high SINR.

- Generation method – Instead of an analytic formula, use link-level simulations (LLS) to obtain the typical difference between the “true” Layer-3 RSRP and the RSRP a UE would report.

- Independence – Errors are drawn independently per beam, per cell, and per measurement occasion, using the SINR of each occasion as an input to the LLS

## RF Error Model

Purpose: Represent analog front-end imperfections in the gNB or UE that bias or jitter the measured received power.

- Origin – residual RF gain/phase miscalibration, hardware temperature drift, antenna-coupler tolerances, and other analog impairments before the baseband stage.

- Statistical form – The baseline model is a zero-mean truncated Gaussian (normal) distribution, typically applied independently to every transmit beam.

  - FR1 examples: standard deviation around 2.5 dB, truncated at ±2.5 dB.

  - FR2 examples: around 4 dB, truncated at ±4 dB.

- Application – A random RF error draw is added to each Tx beam before any digital processing

RF measurement errors originate in the analog front-end, whose gain and phase characteristics vary with frequency. Because each carrier band uses different hardware paths and calibration tables, the residual gain/phase offsets left after factory alignment naturally differ across bands and can drift differently with temperature and aging. This means the magnitude and statistics of the residual RF error (its mean bias and variance) are expected to vary between frequency layers.

To reflect this reality in testing and simulation, RAN4 can keep the agreed truncated-Gaussian RF error model but allow layer-specific parameters, for example, different standard deviations or clipping ranges for FR1 and FR2. When multiple bands share parts of the same RF front-end, an optional inter-layer correlation coefficient can be added to capture partially shared biases. These variations depend on how far apart the frequencies are. Close-frequency layers (e.g., two 100 MHz carriers within the same FR1 band or two neighboring sub-bands on a single RFIC) often share most of the RF path and calibration constants. Their residual gain and phase offsets therefore tend to be partially correlated: a common bias from the shared amplifiers, couplers, or temperature drift will affect both, while smaller per-chain mismatches create a modest differential error. For such cases, a realistic model can keep the same truncated-Gaussian shape while (i) using nearly equal σ values for the two layers and (ii) adding a cross-layer correlation coefficient to capture the shared bias.

When the frequency separation is large, for example FR1 vs. FR2 or low-band vs. mmWave, the physical front-end is different: antennas, filters, duplexers, PAs, and LNAs are all separate, and factory calibration is performed independently. In these cases, residual errors are effectively independent across bands, and their magnitudes can differ significantly because high-frequency circuits typically have larger gain and phase uncertainties. Here it is appropriate to model distinct σ parameters and truncation limits for each frequency layer, with negligible cross-layer correlation.

We propose to model the frequency dependency as follows:

The RF-error on each carrier layer is modeled as a constant, zero-mean truncated normal in dB,

E_RF(f) ~ TG(0, σ_RF(f)^2, ±T(f)),

where every FR1 layer shares the same spread and clipping (σ_RF(f)=σ_FR1, T(f)=T_FR1) and every FR2 layer shares a larger spread and looser clipping (σ_RF(f)=σ_FR2, T(f)=T_FR2), with σ_FR2 > σ_FR1 and T_FR2 > T_FR1. This reflects that mmWave hardware typically exhibits greater residual gain and phase uncertainty. The layer-to-layer dependence is described entirely by a distance-decay kernel

Corr[E_RF(f1), E_RF(f2)] = ρ0 · exp( - ( |f1 - f2| / Δf0 )^p ).

Here ρ0 sets the maximum possible correlation when two layers are at essentially the same frequency (|f1−f2| → 0), representing the amount of bias shared by carriers on the same RF chain. Δf0 is the characteristic frequency spacing at which the correlation falls to ρ0 / e; larger Δf0 means correlation persists over wider frequency separations, while smaller Δf0 makes it drop quickly. The exponent p controls how sharply that decay occurs: p = 1 gives a simple exponential roll-off, while p = 2 produces a Gaussian-like shape that is steeper near zero separation and softer at longer distances. Together these parameters provide a compact, physically motivated way to capture how RF measurement errors remain partially correlated for close-frequency layers and become effectively independent for widely separated bands.

We can also make the model band aware. Then we can make the correlation also band-aware. Within each band, use its own kernel; across bands, we can set correlation to zero (or a tiny ε if we want to allow residual cross-band coupling):

Corr[E_RF(f1), E_RF(f2)] =

{  ρ0_FR1 · exp( - ( |f1 - f2| / Δf0_FR1 )^p_FR1 ),   if f1,f2 ∈ FR1

ρ0_FR2 · exp( - ( |f1 - f2| / Δf0_FR2 )^p_FR2 ),     if f1,f2 ∈ FR2

0,                                                   if band(f1) ≠ band(f2) }

Roles: ρ0_FRx sets the maximum within-band correlation as the two layers coincide; Δf0_FRx is the within-band spacing where correlation drops to ρ0_FRx/e; p_FRx shapes the decay (p=1 exponential, p=2 Gaussian-like). This lets FR1 and FR2 have different variance/tails and different correlation persistence, matching the fact that mmWave chains (FR2) typically show larger spread and shorter shared-bias coherence than sub-6 GHz (FR1).

An RF receive (Rx) chain is the entire analog path from a UE antenna element or sub-array through its dedicated low-noise amplifier, filters, mixers, and gain/phase calibration circuits to the analog-to-digital converter. Each chain is effectively its own “mini-radio,” with small, persistent gain and phase offsets that differ from chain to chain. To capture this in measurement-error modeling, every Rx chain should be assigned its own constant bias, drawn once per simulation.

In FR2 (mmWave) systems each UE Rx beam is formed by combining signals from many antenna elements through phase shifters and hybrid couplers inside a single RF chain. Small mismatches in those beamforming weights, element gains, create beam-specific residuals even when the chain-wide hardware is identical. These residuals are typically stable over the duration of a test run and have statistics that depend on how similar two beams are in direction: beams pointing to nearly the same angle share more of the same hardware path and are therefore more strongly correlated than beams pointing far apart. This means the overall RF error for beam b on chain r and frequency f can be expressed conceptually as a sum of a chain-level bias and a beam-dependent residual.

A simple way to capture this is

E_RF(f,r,b) = B_r + D_r(b),

where B_r is the constant bias for chain r and D_r(b) is a zero-mean truncated Gaussian representing the residual unique to beam b. Correlation among beam residuals can be described by

Corr[D_r(b1), D_r(b2)] = ρ_beam · exp[ -(Δθ(b1,b2)/θ0)^q ],

with Δθ the boresight angle separation, ρ_beam the maximum intra-beam correlation, θ0 an angular scale, and q a shape exponent. In this structure B_r is common to all beams on the chain, while D_r(b) adds beam-specific imperfections whose correlation fades as the beams diverge in direction. The result is a compact, physically meaningful model for RF measurement error that distinguishes chain-level biases from beam-level mismatches in FR2 receivers.

Layered RF-Error Model for Realistic Measurement Imperfections.

RAN4 shall study a three-layer RF-error model to capture frequency, chain, and beam-specific analog front-end imperfections that bias or jitter measured RSRP.

- Frequency dependency:

  - Keep the current truncated-Gaussian error shape but allow different spreads and clipping ranges for FR1 and FR2.

  - Include an optional distance-based correlation so that carriers close in frequency share part of the same bias, while widely separated bands (e.g., FR1 vs FR2) remain independent.

- RF-chain bias:

  - Give each receive chain its own constant gain/phase offset that persists during a test and is independent across chains unless hardware is shared.

- Rx-beam residuals (FR2 focus):

  - Add small, stable beam-specific mismatches within each RF chain, with stronger correlation for beams pointing in similar directions.

Inter-frequency Inter-cell Predictions

A key AI/ML application in NR inter-cell mobility is predicting inter-frequency measurements, which currently require measurement gaps where the UE pauses serving-cell communication to measure other frequencies. These gaps cause service interruptions and increase power consumption. AI/ML can reduce the need for such gaps, improving continuity and UE energy efficiency. By using measurement data from two frequency layers, models can learn cross-layer signal relationships, enabling prediction of one layer’s measurements from the other or determining if a UE on a macro layer can reliably connect to a micro layer. Clustering techniques can further enhance accuracy by combining multi-cell data as input features.

.

Frequency Correlation Coefficient

Channel correlation coefficient between measured and predicted frequency layers that will impact the prediction accuracy for inter-frequency prediction

Issue 2-1-8: Inter-frequency prediction scenario Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

The channel correlation coefficient between measured and predicted frequency layers plays a critical role in determining the accuracy of AI/ML-based inter-frequency RSRP prediction. This coefficient reflects how similar the propagation conditions are between the two frequency layers, which directly impacts model generalization and reliability.

However, the channel correlation is affected by several key factors, including:

- Differences in antenna patterns between frequencies

- Variations in bandwidth

- Changes in line-of-sight (LOS) probability

- Frequency-dependent scaling of large-scale parameters

- Divergences in cluster delays and angles

- Independent shadowing effects

- Presence of frequency-specific blockage

These variations can significantly reduce the correlation between layers, thereby limiting the predictive power of models trained on one frequency to accurately infer RSRP at another. These parameters have been tabulated in R4-2505766 and they are shown below:

| Step | Issue | Impact on Multi-Frequency Modeling |
| --- | --- | --- |
| Step 1 | Shared parameters except antenna/array/center freq/bandwidth | • Antenna pattern differences cause spatial response mismatch • Bandwidth variation affects PSD |
| Step 2 | Uniform propagation conditions (soft LOS may differ by frequency) | • FR2 typically has lower LOS probability than FR1 |
| Step 4 | LSPs same but DS/AS allow frequency scaling | • DS(f)= 10^(μlgDS(f) + σlgDS(f)x) |
| Steps 5-7 | Identical cluster delays/angles for all frequencies | • Conflicts with Clause 7.5 (noted in standard) |
| Step 6 | Independent per-cluster shadowing, frequency-dependent cluster powers | • Allows realistic power variation but no explicit pathloss linkage |
| Steps 8-11 | Small-scale parameters generated independently per frequency | • Breaks physically existing phase correlations |
| Blockage | Shared blocker positions across frequencies | • Underestimates FR2 blockage probability (wavelength-sensitive) |

RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction models

Impact of Cluster Approach

AIML model performance can be enhanced when measurements from a group of cells in one carrier frequency are used as an input for AIML model. The rationale behind this is diverse inputs can help to enhance AIML model performance. Clustering is needed in AI/ML-based inter-frequency measurement prediction because relying on measurements from a single serving cell often provides a limited and potentially biased view of the radio environment. In many deployment scenarios, especially in heterogeneous networks with varying cell sizes and antenna configurations, the signal behavior of a single cell may not fully represent the spatial and frequency-domain correlations present across the network. By incorporating measurements from multiple cells within the same frequency layer, whether serving or neighbor cells, the model gains access to richer and more diverse spatial fingerprints.

Another key benefit of clustering is resilience against measurement anomalies. In practice, a single cell’s measurement may be affected by temporary disturbances, such as beam misalignment, or short-term channel fluctuations that could mislead the prediction model. When the model instead consumes a set of measurements from multiple cells, it can average out or filter these anomalies, allowing it to identify the underlying long-term correlation between frequency layers more accurately. This is particularly important in mobility scenarios where channel conditions change rapidly, and the predictive power of any single cell’s measurement may degrade quickly.

Finally, clustering supports better generalization across deployment environments. AI/ML models trained with diverse multi-cell input are less sensitive to changes in topology, cell power configurations, or beamforming patterns because they learn relationships that are consistent across the network, not just within a single link. For inter-frequency prediction, this means the model can more reliably infer a target frequency’s RSRP, even in areas with sparse direct measurements, by leveraging patterns observed in other, spatially correlated cells.

Study Clustering for AI/ML-based Inter-Frequency Measurement Prediction

RAN4 shall evaluate how clustering multiple cells as joint inputs affects prediction accuracy and measurement overhead.

- Examine how multi-cell clustering provides richer spatial and frequency information, improving robustness when individual cell measurements are noisy or missing.

Generalization Issues

In this section we present an end-to-end simulation blueprint for a RAN4 study aimed at resolving generalization problems in AI/ML-based mobility prediction.

Problem statement and study objective
AI/ML beam-selection and hand-over algorithms will be trained and benchmarked in a single “friendly” lab environment. However, the live network changes band, panel size or mobility pattern, and the prediction accuracy can collapse. RAN4 therefore needs proof that AI/ML prediction can generalize across the full radio diversity operators deploy. The proposed study will compare three different approaches and their underlying models

  - Universal model: one large neural network trained on every dataset it sees, with no specialization.

  - Per-scenario model: a completely separate full-size network for every deployment slice, guaranteeing accuracy but at huge memory and certification cost.

  - Backbone + Experts: the proposal under test, where a single physics-aware backbone is frozen and each slice adds only a small adaptation layer. The goal is to show that Backbone + Experts meets RAN4 KPI thresholds and beats the other two on size, update agility and conformance overhead.

To judge generalization we must move the radio “knobs” that truly shape propagation. This slice universe lets us create controlled yet diverse scenarios. By training on one set of slices and testing on others, we can measure how well an AI/ML model transfers its learning when any single axis or several at once changes.

Defining the slice universe
Generalizations can only be judged if we deliberately change the radio “knobs” that matter. We describe every deployment by a five-axis code:

- Frequency layer (F) – low sub-6 GHz, high sub-6 GHz, low mmWave, high mmWave; four buckets capture two octaves of path-loss change.

- Antenna / beam class (A) – small arrays (≤ 8 TX), medium arrays (9–32 TX) and massive arrays (> 32 TX); this axis drives beam-width and spatial consistency. [ a1, a8]

- UE-speed tier (V) – pedestrian, city-traffic, high-speed rail; speed alters temporal correlation and Doppler. [vlow, vmedim, vhigh]

- Environment (E) – dense-urban street canyon, suburban macro, rural motorway and indoor hotspot; each has distinct scatterer density. [e1, e2, e3,,, ]

- SNR (S): it could be defined based on ranges: Low, medium, high [snr low snr medium snr hign ]

Let’s consider one example where the whole set of slices as defined by the cartesian product is 4(F) × 3(A) × 3(V) × 4 (E) x 3 = 432 theoretical slices. RAN4 will never test all 432, but the taxonomy ensures every real-world cell can be mapped to a canonical slice code.

How to Choose the “training coverage” level (so that we test the generalization later):

### There are two practical strategies for down-selecting slices before we start training:

1 – “Exploratory” (very low budget, ≈ 8 slices)
When you just need a quick sanity check, anchor each axis at its two extremes and cross them:

- Frequency: choose the lowest band we operate and the highest (e.g., 47 GHz).

- Antenna size: take the smallest panel in the network (≤ 8 TX) and the largest massive-MIMO panel (> 32 TX).

- Speed: include a pedestrian trace (< 10 km h⁻¹) and a high-speed-rail trace (> 120 km h⁻¹).

- Environment: pick one outdoor macro setting (e.g., suburban, UMa) and one dense indoor/hotspot (InH).

- SNR:  pick low and high

The cartesian product 2 × 2 × 2 × 2 = 16 slices captures the widest physics gap on every axis with the smallest possible dataset. We can use this dataset when we want a fast test to see if the modelling concept holds.

2 “Balanced” (mid-size, ≈ 40 slices)
Sample slices are data-driven rather than by eyeballing the extreme values:

- Encode every one of the all possible slices as a numeric vector
[ freq_GHz,  log2(#TX),  speed_km h−1,  env_id , SNR]

- Cluster them with K-means (choose K).

- Attach a weight to each slice if available

- Rank clusters by the summed weight of their members and keep the centroids of the top KK clusters.

This way we end up with a modest-sized training set that is (a) evenly spread across the physics space thanks to K-means distance minimization and (b) skewed toward the slices that will actually be more beneficial according to some predefined criteria.

Below is the suggested simulation workflow for addressing the generalization challenge and evaluating different models.

### Option 1: Universal model: single network for every slice (scenario)

In the simulation we first evaluate a naïve baseline by training a single universal model on an aggregated dataset that combines logs from all the chosen training scenarios. To prevent dominant slices from overshadowing rare ones, we apply balanced mini-batching with equal sampling from each slice in every training step. The model is optimized over many epochs using AdamW and an appropriate learning rate around . This simulation produces one checkpoint. Checkpoint refers to a saved snapshot of the trained AI/ML model on the aggregated dataset at a given point during or after training. It contains the learned parameters (weights and biases) of the model, as well as optimizer states if needed.

### Option 2: Many stand-alone models:  one full network per slice

### In this simulation setup, we also evaluate the per-slice training approach, where a separate full model is trained individually on each slice/scenario of the dataset. For each slice scenario, we create an independent training dataset using only the dataset from that specific slice. The model is trained on this single-slice data with standard mini-batching, optimized over many epochs using AdamW with an appropriate learning rate. At the end of training, we save a checkpoint of the model for each slice. These checkpoints are later used for offline KPI evaluation to assess how well models perform when trained exclusively on one slice and tested across others.

### Option 3: Shared backbone + tiny slice-specific experts

In this simulation setup, we also evaluate a parameter-efficient approach using a shared backbone combined with small slice-specific experts. A single backbone model is trained to capture general radio characteristics across all slices. For each individual slice, we attach a lightweight  adapter and fine-tune only this adapter while keeping the backbone frozen. Training follows standard mini-batching over many epochs with AdamW and a appropriate learning rate. After training, we save the common backbone along with the set of slice-specific adapters, which are later used for offline KPI evaluation. This method aims to balance generalization with slice-specific adaptation while keeping the total model size compact.

- Stage A – learn backbone Φ

  - Train on the pooled data exactly like the universal case.

  - Freeze Φ once validation loss stabilizes

- Stage B – fine-tune for all adapters

  - For each slice k: attach a fresh adapter Ψₖ, train just 2-3 epochs on that slice, freeze, export

  - Repeat until all adapters are done.

- Simulation outputs: one backbone file + (number of chosen slices) adapters.

Summary of the model options described:

- Universal model tests raw generalizability with minimal engineering effort.

- Stand-alone models give an upper-bound on accuracy but are heavy.

- Backbone + experts aims for near-upper-bound accuracy at a fraction of the storage and training cost, exactly what we want to quantify before any formal RAN-4 test plans are drafted.

We make the following definitions:

### Seen Slices = Slices used for training, but tested on held-out (unseen) data

- During training, we use part of the data from each of the Seen Slices (e.g., 80% for training).

- For testing, we evaluate the model on the remaining 20% (or any split) of data that was never used in training from those same slices.

- This gives us a valid estimate of in-distribution generalization testing whether the model learned meaningful patterns rather than memorizing the training data.

### Unseen Slices = Slices never used during training, fully reserved for testing

- Unseen Slices are slice-types (combinations of deployment parameters) that are entirely excluded from the training phase.

- Neither the model nor any of its components (universal, stand-alone, or expert) have been exposed to any data samples, patterns, or statistics from these slices during training.

- The model is evaluated on fresh data drawn from these Unseen Slices, representing truly out-of-distribution (OOD) testing.

The NMSE performance on Unseen Slices directly measures the model’s ability to generalize to deployment scenarios it was never trained on. This simulates real-world robustness when a trained model encounters new environments, frequencies, or mobility patterns.

Let:

- J= Index set of Seen Slices (used for training, tested on held-out data). Measures in-distribution performance on known slice types.

- O = Index set of Unseen Slices (never used for training, fully held out). Measures out-of-distribution performance on unknown slice types.

The total number of NMSE test error values is:

Nt=Nseen+Nunseen=J+O

Let u∈RNt be the NMSE row-vector for the universal model; element uj corresponds to test file or slice j.

Let S∈RNseen×Nt  denote the NMSE matrix for the fully stand-alone fleet, so that Sij is the error obtained when stand-alone model i is applied to test slice j

Let E∈RNseen×Nt  be the analogous NMSE matrix for the backbone + expert system.

We can use the following aggregated statistics to judge generalization:

Universal model averages

NMSEseenU=1Ij∈Iuj,   NMSEunseenU=1Oi∈Oui,  and define the generalization gap as:

ΔU=NMSEunseenU-NMSEseenU. (scalar)

Stand-alone diagonal and cross averages

We train Nseen​ separate models one model per Seen Slice

- We evaluate each trained model on all slices in the testing set:

  - The Nseen slices (in-distribution)

  - The Nunseen​ slices (out-of-distribution)

Resulting in a Nseen×(Nseen+Nunseen)  NMSE matrix S∈RNseen×Nt

The diagonal entries Sii indicate the accuracy on Seen Slices (Self-Performance): each model tested on its own slice (e.g., Model 5 on Slice 5). We want low NMSE on the diagonal, showing the model fits well to its trained slice. This validates in-distribution fitting.

Off-diagonal entries ​Sij, i≠j, Cross-Slice Generalization: Model i tested on a different slice j.

- We can analyze the following:

  - If off-diagonal NMSE is high:
→ The stand-alone model overfits its own slice and generalizes poorly.

  - If off-diagonal NMSE is low:
→ Some generalization exists across slices.

The goal of this study is to answer the following question: How many stand-alone models would be needed to “cover” the full set of possible slices with acceptable performance and by coverage we mean low NMSE.

### If we see: Diagonal ≪ Off-Diagonal NMSE (Poor generalization)

- If Diagonal NMSE ≪ Off-Diagonal NMSE, this shows:

  - Each stand-alone model performs well on its own slice

  - But generalizes poorly to other slices (seen or unseen)

- This means:

  - Models are highly slice-specific

  - We may need a large number of specialized models: possibly close to one per slice

  - The stand-alone modeling approach does not generalize efficiently

### If we see: Diagonal ≈ Off-Diagonal NMSE (Good generalization)

- If we observe Diagonal NMSE ≈ Off-Diagonal NMSE, that suggests:

  - Each model can generalize well to slices beyond its training slice

  - We may need fewer models (e.g., grouping slices into clusters)

  - The system could work with shared models across similar slices

### Let’s provide an example with numerical values (assume 144 total slices):

- If we have 16 stand-alone models which fail on most of the 4 unseen slices (high off-diagonal NMSE),
→ We cannot expect the 16 models to cover the other 144 – 16 = 128 slices
→ We would likely need many more models or a better generalization strategy (e.g., Backbone + Experts)

- If some stand-alone models perform well on multiple unseen slices,
→ We might cluster slices based on those generalization patterns
→ Estimate coverage as 1 model per generalizable cluster rather than per slice

### We can use the following rule of thumb Interpretation:

- Diagonal NMSE ≪ Off-Diagonal NMSE
→ We need ~1 model per slice (poor generalization)

- Diagonal NMSE ≈ Off-Diagonal NMSE (for both seen and unseen slices)
→ We can cover multiple slices with fewer models (good generalization)

### What Stand-Alone Models tells us: they tell us how specialized each model must be to achieve good performance on its own slice. However, by testing off-diagonal NMSE, we could see how poorly a slice-specific model performs on other slices. Therefore:

- If off-diagonal NMSE is high, stand-alone models suggest we need one model per slice

- If off-diagonal NMSE is low in clusters, we might cluster slices and reduce the number of models.

However:

- The stand-alone study doesn’t tell us whether you can share a backbone between slices.

- Nor does it inform whether lightweight specialization (experts) is enough for good performance.

If the study with standalone models show poor generalization performance, then we can adopt a better generalization strategy explained below:

### Backbone + Experts Simulation Study:

- The Backbone + Experts approach tests whether a single shared backbone can generalize common features across slices, while a small slice-specific expert handles the specialization.

- We employ the following training procedures:

  - One backbone model on all Seen Slices mixed dataset.

  - One expert per Seen Slice (small parameter set and few iterations).

- Employ the following test procedures:

  - Test how well the backbone + expert combination works on:

    - The Seen slices (with their own expert)

    - The Unseen slices (with a random expert or no expert possibly fallback)

- If Backbone + Experts generalize well:
→ We may only need a single backbone and a small set of experts for similar slice groups.
→ This reduces total models dramatically (e.g., 1 backbone + ~10 experts instead of ~100 full models).

- If Backbone + Experts fail on unseen slices:
→ The radio domain doesn’t support generalization well.
→ We may still need many specialized models (but at least now we know).

### Therefore, the bottom line and conclusions from above discussion are:

- Stand-alone models = Worst case upper bound on model count (likely 1 per slice if no generalization).

- Backbone + Experts = Test if we can collapse models to a small number of shared structures.

Based on the above analysis we have the following proposal:

Study Model Generalization Across Deployment Scenarios for AI/ML-based Mobility

RAN4 shall evaluate how different training and deployment strategies affect AI/ML model robustness when faced with diverse network conditions.

- Compare three approaches: a universal model trained on combined multi-scenario data, stand-alone models trained per scenario, and a shared backbone with lightweight scenario-specific experts.

- Test each approach on both Seen (training) and Unseen (new) deployment scenarios to measure generalization and stability.

- Identify whether mobility prediction requires many specialized models or if a shared architecture with minimal specialization is sufficient.

- Provide guidance on model deployment and standardization to balance performance, complexity, and conformance testing effort.

Impacts from observation and prediction windows

Following agreement was reached in RAN4#114 meeting:

Issue 2-1-9: Impacts from observation and prediction windows Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

In temporal domain AI/ML prediction, the relationship between the observation window (OW) length, prediction window (PW) length, UE speed, and measurement reporting repetition time (MRRT) can significantly affect prediction accuracy requirements. The number and type of measurements collected within the OW directly influence the AI/ML model’s ability to learn and forecast. However, the ability to precisely determine the number of usable measurement samples within the OW is limited by several factors.

For Case A (no measurement reduction), the model inputs may be either L1 measurements (sub-cases 1 and 3) or L3 measurements (sub-case 2). While the number of L3 measurement samples in the OW can be derived from the configured measurement periodicity, the number of L1 samples depends on UE-specific parameters such as L1 sampling period and filter coefficients, which are proprietary and unknown to RAN4. Even in the L3 case, the subset of L3 samples actually used by the AI/ML model may remain UE-proprietary. As a result, RAN4 cannot reliably assess the number of measurements performed during the OW, nor quantify their direct effect on prediction accuracy.

This limitation means that while OW, PW, MRRT, and UE speed are important parameters to study, RAN4’s evaluation should focus on observable and standardized factors rather than attempting to account for proprietary measurement sample counts inside the OW.

In case A temporal prediction, OW/PW length, UE speed, and MRRT affect accuracy, but the exact number of measurements in the OW is UE-proprietary. RAN4 should therefore base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts.

In temporal domain prediction case B, the UE may reduce the number of actual measurements by replacing them with predicted values, with the reduction level controlled by the MRRT parameter. When combined with variations in UE speed, OW, and PW settings, the total number of possible test configurations can grow significantly, making exhaustive testing impractical. To manage this complexity, RAN4 should agree on a limited set of pre-selected reference scenarios for OW, PW, MRRT, and UE speed to ensure consistency and comparability across evaluations.

RAN4 is invited to agree on a set of pre-selected reference testing scenarios for temporal domain prediction case B, covering representative combinations of OW, PW, MRRT, and UE speed. This approach will limit the exponential growth of possible test configurations.

In temporal domain prediction case B, achieving a specific Measurement Reduction Rate in the Temporal domain (MRRT) can be done through different skipping patterns, even when the target reduction percentage is the same. For example, a 50% MRRT could be realized by replacing a continuous block of measurement slots with predictions within the Prediction Window (PW), or by alternating measured and skipped slots throughout the Observation Window (OW). While both patterns meet the MRRT target, they have different impacts on the distribution of real measurements, potentially influencing prediction accuracy and robustness.

As discussed in RAN2, skipping patterns are applied at the L1 sample level and depend on the UE’s proprietary L1 filtering implementation. This raises the question of whether the UE should indicate its supported skipping patterns to the network (NW). If such capability information is shared, the NW could configure MRRT explicitly by selecting a specific skipping pattern from those supported by the UE. If not shared, the MRRT configuration would be applied without visibility into the underlying measurement distribution. If the UE does not tell the network (or the test equipment) which skipping pattern it is using, the MRRT setting alone (e.g., “reduce measurements by 50%”) doesn’t reveal when and which measurement slots are actually skipped.

For RAN4 testing, two approaches emerge. Option 1 keeps the skipping pattern fully proprietary, with the Test Equipment (TE) only setting the MRRT target for the Device Under Test (DUT) without knowing exactly which measurements are skipped. While simpler, this lacks robustness and creates opportunities for the UE to pass tests without faithfully implementing the intended measurement reduction behavior. Option 2 allows the UE to inform the NW of supported skipping patterns, enabling the TE to explicitly select the pattern for testing. This adds transparency and control, ensuring the test evaluates the intended conditions, but requires additional signaling and coordination.

RAN4 is invited to study whether MRRT configuration for temporal domain prediction should include visibility into the UE’s measurement skipping pattern. Two options are proposed: (1) Keep the pattern UE proprietary, with the TE applying only the MRRT value and verifying KPI compliance; or (2) Have the UE report supported patterns so the TE can explicitly configure one during testing.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied, we particularly addressed the following topics:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Measurement Error Model

- Generalization issues

- Relative accuracy

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

- Relative RSRP accuracy evaluates how well the predicted difference between two cells matches the ground-truth difference, providing a measure of consistency between cell comparisons. While a small relative-accuracy error suggests that the prediction and measurement are mutually reliable, it does not reveal which cell is actually stronger. This creates a risk when the two values are taken at different times (t₁ for one cell and t₂ for the other): channel changes, mobility, or fading can cause the relative ranking of cells to change between these moments. As a result, even with perfect relative accuracy, the network could incorrectly conclude that one cell is stronger and make a sub-optimal handover decision.

Alternative 1, where the FR1 ground truth is derived at the TE from transmitted or received power (legacy approach), is not well suited for AI/ML mobility scenarios. In fading CDL channels, rapid signal fluctuations make precise time alignment between measured and predicted samples difficult, leading to potential inaccuracies in ground truth. Furthermore, the TE lacks knowledge of the UE’s proprietary L1 filtering parameters, such as filtering coefficients and sample counts, which further limits its ability to replicate the UE’s measurement context.

Alternative 2, where the FR1 ground truth is derived from UE-reported measurements under defined conditions, is better suited for AIML Mobility because channel variations and L1 filtering effects are inherently accounted for by the UE when producing the ground truth measurements

The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbor prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side.

- RAN4 is invited to adopt a clear time-indexed definition of relative RSRP accuracy for intra-frequency temporal prediction. At time t, cell 1 predicts its Layer-3 RSRP at t + Δt1, while cell 2 provides a reported (measured or predicted) RSRP at t + Δt2. The metric compares the predicted power gap to the true power gap at those future times and is expressed as ( P_pred1(t + Δt1) - P_rep2(t + Δt2)) -( P_true1(t + Δt1) - P_true2(t + Δt2)).  No requirements between Δt1, Δt2

Clarify Beam-level Relative RSRP Accuracy KPI

RAN4 shall explicitly define the beam-level relative RSRP accuracy metric, covering both the prediction domain and the valid reference-beam relationship, to enable consistent and interoperable testing.

- Specify whether the KPI applies to temporal prediction (future-time beam RSRP) or frequency-domain inference (other-layer RSRP), or provide separate formulas for each.

- Define the reference-beam relationship: intra-cell (best vs. secondary beam) or inter-cell (best vs. neighbor beam), or adopt distinct KPI sub-modes.

- Provide the exact formula (Pred B1 – Pred/Reported B2) – (GT B1 – GT B2) and state how measured vs. predicted values are used in each case.

New Definition of Relative Inter-Frequency L3-RSRP Accuracy

RAN4 shall define relative inter-frequency accuracy through two complementary cases to ensure consistent evaluation of cross-frequency prediction.

- Case A – Time-domain, cross-frequency: Cell 1 on f₁ predicts its L3-RSRP at t + Δt₁ while Cell 2 on f₂ (same FR) provides a reported value at t + Δt₂ (measured or predicted); the KPI compares predicted vs. true inter-cell power gaps to assess future-time signal differences.

- Case B – Frequency-domain, cross-cell: Cell 1 on f₁ is predicted across frequency using measurements from other cells, while Cell 2 on f₂ provides a reported value (predicted across frequency from other cells or measured); the KPI compares predicted vs. true inter-cell power gaps for cross-frequency inference.

- Explicitly state the mathematical KPI as (Pred f1 – Pred/Reported f2) – (GT f1 – GT f2) for both cases.

Anti-cheating strategy for Alt-2 testing (Twin-cell single-run)

RAN4 shall adopt a twin-cell test method to guarantee that reported prediction accuracy reflects true model performance rather than post-run adjustments.

- Create two parallel cells with identical fading but a hidden, fixed or time-varying power offset known only to the test equipment.

- Require the UE to predict Cell A’s L3-RSRP for future times while simultaneously measuring Cell B’s RSRP as ground truth.

- Enforce time-stamped prediction submission before measurements exist, preventing back-filling.

- Because prediction and measurement occur simultaneously on two synchronized cells, any RSRP gap directly indicates genuine prediction error, not fading drift.

Layered RF-Error Model for Realistic Measurement Imperfections.

RAN4 shall study a three-layer RF-error model to capture frequency, chain, and beam-specific analog front-end imperfections that corrupts measured RSRP.

- Frequency dependency:

  - Keep the current truncated-Gaussian error shape but allow different spreads and clipping ranges for FR1 and FR2.

  - Include an optional distance-based correlation so that carriers close in frequency share part of the same bias, while widely separated bands (e.g., FR1 vs FR2) remain independent.

- RF-chain bias:

  - Give each receive chain its own constant gain/phase offset that persists during a test and is independent across chains unless hardware is shared.

- Rx-beam residuals (FR2 focus):

  - Add small, stable beam-specific mismatches within each RF chain, with stronger correlation for beams pointing in similar directions.

RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction models

Study Clustering for AI/ML-based Inter-Frequency Measurement Prediction

RAN4 shall evaluate how clustering multiple cells as joint inputs affects prediction accuracy and measurement overhead.

- Examine how multi-cell clustering provides richer spatial and frequency information, improving robustness when individual cell measurements are noisy or missing.

Study Model Generalization Across Deployment Scenarios for AI/ML-based Mobility

RAN4 shall evaluate how different training and deployment strategies affect AI/ML model robustness when faced with diverse network conditions.

- Compare three approaches: a universal model trained on combined multi-scenario data, stand-alone models trained per scenario, and a shared backbone with lightweight scenario-specific experts.

- Test each approach on both Seen (training) and Unseen (new) deployment scenarios to measure generalization and stability.

- Identify whether mobility prediction requires many specialized models or if a shared architecture with minimal specialization is sufficient.

- Provide guidance on model deployment and standardization to balance performance, complexity, and conformance testing effort.

In case A temporal prediction, OW/PW length, UE speed, and MRRT affect accuracy, but the exact number of measurements in the OW is UE-proprietary. RAN4 should therefore base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts.

RAN4 is invited to agree on a set of pre-selected reference testing scenarios for temporal domain prediction case B, covering representative combinations of OW, PW, MRRT, and UE speed. This approach will limit the exponential growth of possible test configurations.

RAN4 is invited to study whether MRRT configuration for temporal domain prediction should include visibility into the UE’s measurement skipping pattern. Two options are proposed: (1) Keep the pattern UE proprietary, with the TE applying only the MRRT value and verifying KPI compliance; or (2) Have the UE report supported patterns so the TE can explicitly configure one during testing.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- R4-2504901, WF on AI/ML Mobility, Nokia, RAN4#114bis, Wuhan, China, 7th – 11 April, 2025.