3GPP TSG-RAN WG4 Meeting #116bis                                    	                      R4-2513594

# Prague, Czech Republic, October 13 – October 17, 2025

Agenda item:	6.1.2

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

After finalizing the discussion on the maximum number of MIMO layers for 6Rx handheld UEs, in this paper we propose the scope of the 6Rx Demodulation Performance and CSI Reporting Requirements.

# UE Capability and Release Independence Aspects

Companies have discussed in the past how to specify applicability rules for the core requirements phase. More specifically, how to indicate that a UE support 6 MIMO layers with 6Rx and how to capture in the specifications that only FWA devices are supporting this feature. During last RAN Plenary it was agreed that RAN2 specifications would recognize, in writing, this outcome of the discussion.

Observation #1: RAN Plenary has confirmed that 6 MIMO layers is a FWA feature, which gets explicitly captured in the text of RAN2 specifications.

In addition, also during the last RAN Plenary, it was discussed the issue of Release Independence. As per the 3GPP specification 38.307, Requirements on User Equipments (UEs) supporting a Release-Independent Frequency Band, is the spec for the release independence issues and maintained by RAN4. By its name, the release independence should be primarily used for frequency band and band combinations.

Observation #2: TS 38.307 is primarily used to define release-independent band and band combinations. The large majority of the release independent features are spectrum-related, on topics like NR operating frequency bands, channel bandwidths, carrier aggregation configurations, and power classes, among others.

In terms of spectrum related features, there is a plethora of use cases that comprise the large majority of TS 38.307. Among others, we can find:

- NR Operating Frequency Bands, which include power classes and RF requirements.

- Supported Channel Bandwidths, which are specified per duplex mode (FDD, TDD, SDL, SUL)

- Carrier Aggregation (CA) Configurations, which cover both intra-band and inter-band CA.

- Power Classes, which details requirements related to UE power levels for different bands and modes.

On the other hand, there are some non-spectrum related release-independent features, which are generic baseband requirements for the already specified features in legacy releases based on RAN4 consensus, such as the following release independent performance requirements for features already specified in NR Rel-15:

- Rel-15 RF requirements for 4Rx UEs

- Rel-15 Transparent Tx Diversity

- Rel-15 UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference

- Rel-17 PDSCH absolute physical layer throughput requirements with link adaptation

- Rel-17 RF requirements for 8Rx UE

- Rel-17 Demodulation requirements for 8Rx UE

The main principle is that the newly defined requirements for legacy features can be release independence. In other words, previous non-spectrum related release independence cases are focused on the new requirements only without additional signalling or new features included.

Proposal #1: RAN4 to confirm 6Rx UE as a Rel-19 feature only, and do not consider release independence.

For the rest of this work item, defining the new PDSCH requirements is most important. Based on previous experience during Rel-18 8Rx discussions, we think only a single PDSCH test per case is needed. Additionally, a FWA device should only need to pass a single test case with 6 MIMO layers. If this test passes, the FWA UE should be allowed to skip the 4 MIMO layers test case since all new relevant functionality, e.g. layer mapping and 6Rx functionality, would be already tested.

  - Observation #3: For a 6Rx FWA device, a performance requirement that focuses on 6 MIMO layers exercises the same new funcitonality that a test focusing on 4 MIMO layers.

  - Proposal #2: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will only apply for FWA devices, while the 4 MIMO layers test case is reserved for handheld UEs.

  - Proposal #3: Discuss the case where if a device passed the 6Rx 4 MIMO layers test case, the UE should be allowed to skip one or multiple 6Rx and/or 4Rx 4 MIMO layers test cases.

# PDSCH Requirements

With respect to specific simulation parameters, the following table consolidates all simulation assumptions agreed after the past RAN4#115. We highlight in yellow those items that still need to be decided.

| Test case type |  | Only single carrier test |
| --- | --- | --- |
| TDD pattern |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  | 40 MHz / 30 kHz |
| Propagation channel |  | 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  | 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| TRS configuration |  | Symbol#{5, 9} |
| PT-RS |  | Not configured |
| DM-RS configuration |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3 |
| Overhead |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2) = (2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  | 8 |
| HARQ ACK/NACK bundling |  | Not configured |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  | SNR@70% max TP |

From the table above, there’s still disagreement on propagation scenarios and antenna correlation models. For the 6 MIMO layers requirements, since this is a FWA test, we suggested reflecting this aspect in the test definition. Therefore, a proper propagation channel should be TDLA30-10, which provides reasonable frequency selectivity and minimises doppler spread. Since there’s no mobility, the network doesn’t need an additional DMRS symbol due to the low doppler spread (which only accounts for surrounding scattering but not UE mobility). Configuring 2+2 for 6 layers isn’t ideal, but RRC controls reconfiguring DMRS positions, which is slow, and higher rank transmissions are likely mostly in low mobility scenarios. We compromised and accepted a 2+2 DMRS configuration, which has the same throughput performance as a 2+0 configuration. Since FWA devices have a large form factor compared to handheld UEs, ULA Low antenna correlation model is reasonable.

Proposal #4: For 6 MIMO layers, consider Rel-15 DMRS with a 2+2 configuration. Do not consider Rel-18 DMRS.

For the 4 MIMO layers test, since it’s targeted for a handheld UE with limited space and relatively small form factor, parameters should be different. Because of this, a proper propagation channel should be TDLA30-10, since it provides reasonable frequency selectivity while allowing a somewhat higher doppler spread. Since most of the gains of 6Rx vs 4Rx for handheld devices are related to diversity gain, we believe that a relatively low mobility scenario should be targetted, since we still care more for the high throughput conditions. On the other hand, since handheld UEs indeed present form factor constraints, we believe a good approach would be to consider a variation of the existing MediumB antenna correlation model. However, as discussed during the feasibility study before, BS transmitted correlation in MediumB seems quite pessimistic. Hence, we believe a good compromise for testing purposes is to consider Alpha = 0.0 while Beta = 0.07179. Some companies may have concerns that previously used Alpha = 0.3 provides a too high correlation at the Tx side, which might be excessive if compared to existing products in the field. On the other hand, we consider Alpha = Beta = 0.0 too optimistic for a reasonable UE implementation

Observation #4: For 4 MIMO layers, it has been agreed to consider TDLA30-10 as the propagation channel.

Observation #5: Antenna correlation in handheld UEs are expected to be greater than zero, for which assuming Alpha = Beta = 0.0 might be unrealistic and overly restrictive.

Proposal #5: For 4 MIMO layers, define one single test case corresponding to a handheld UE scenario, considering ULA with Alpha = 0.0 and Beta = 0.07179 for the antenna correlation model.

# CSI Reporting Requirements

Finally, we propose to base the CSI requirements discussion on the previous experience with the Rel-18 8Rx work item. For CQI reporting requirements, we realized that for having a more consistent TS 38.101-4, we should define 6Rx CQI requirements.

Proposal #6: RAN4 to define CQI reporting requirements for 6Rx UEs on STATIC channel.

In addition, we are now open to discuss RI reporting requirements, even though we did not define such requirements in Rel-18 for 8Rx.

Proposal #7: RAN4 continue discussing RI requirements for 6Rx UEs.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: RAN Plenary has confirmed that 6 MIMO layers is a FWA feature, which gets explicitly captured in the text of RAN2 specifications.

Observation #2: TS 38.307 is primarily used to define release-independent band and band combinations. The large majority of the release independent features are spectrum-related, on topics like NR operating frequency bands, channel bandwidths, carrier aggregation configurations, and power classes, among others.

Proposal #1: RAN4 to confirm 6Rx UE as a Rel-19 feature only, and do not consider release independence.

  - Observation #3: For a 6Rx FWA device, a performance requirement that focuses on 6 MIMO layers exercises the same new funcitonality that a test focusing on 4 MIMO layers.

  - Proposal #2: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will only apply for FWA devices, while the 4 MIMO layers test case is reserved for handheld UEs.

  - Proposal #3: Discuss the case where if a device passed the 6Rx 4 MIMO layers test case, the UE should be allowed to skip one or multiple 6Rx and/or 4Rx 4 MIMO layers test cases.

Proposal #4: For 6 MIMO layers, consider Rel-15 DMRS with a 2+2 configuration. Do not consider Rel-18 DMRS.

Observation #4: For 4 MIMO layers, it has been agreed to consider TDLA30-10 as the propagation channel.

Observation #5: Antenna correlation in handheld UEs are expected to be greater than zero, for which assuming Alpha = Beta = 0.0 might be unrealistic and overly restrictive.

Proposal #5: For 4 MIMO layers, define one single test case corresponding to a handheld UE scenario, considering ULA with Alpha = 0.0 and Beta = 0.07179 for the antenna correlation model.

Proposal #6: RAN4 to define CQI reporting requirements for 6Rx UEs on STATIC channel.

Proposal #7: RAN4 continue discussing RI requirements for 6Rx UEs.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024