# 3GPP TSG-RAN WG4 Meeting #116bis                               R4-2513596

# Prague, Czech Republic, October 13 – October 17, 2025

Agenda item:	6.1.2

Source:	Apple

Title:	On Demodulation Performance Requirements for LP-WUS

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

# Background

RAN4 RRM has concluded the Core Part of the analysis and has moved into the Performance Part. RAN4 RF session continued working on their technical report. On RAN1 side, signal design has been completed on UE operation on both Idle/Inactive and Connected Mode. However, from LP-WUS detection performance, we believe that .

Observation 1: LP-WUS detection is revelant for both Idle/Inactive and Connected Mode, but signal design and its performance is fundamentally the same.

For the LP-WUR, there are two types of receivers considered: an OOK receiver and a OFDM receiver. From LP-WUS design point of view, RAN1 has specified OOK (both OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK on-duration symbols, and with the possibility of this OFDM sequence to carry information the information as well.

Observation 2: For LP-WUS detection, the LP-WUR receiver can be of OOK type and of OFDM type.

The OOK receiver type performs envelop detection to decode the informaton carried over the OOK ON and OFF symbols. This receiver has lower power consumption but also lower link-level performance. It’s not sensitive to frequency error and it’s moderately sensitive to timing errors. See Figure 1.

Figure 1: OOK Receiver - homodyne/zero-IF architecture with baseband envelope-detection based LP-WUR

The OFDM receiver has higher complexity since it may perform time-domain or frequency-domain correlation with known sequences, decoding the information carried in the overlay sequences. This receiver has higher power consumption but also a better link-level performance. It can be sensitive to both time and frequency errors, but may reuse PSS/SSS for synchronization. See Figure 2.

Figure 2: OFDM Receiver - IQ based LP-WUR

In terms of the OOK waveform, RAN1 has designed based on OOK with Manchester Encoding as the baseline waveform for LP-WUS and LP-SS. The OOK-based receiver can be used to detect the OOK symbols, while the OFDM receiver can detect also the OFDM overlaid sequence. DFT-s-OFDM is used to generate OOK waveform to allow the reuse of the gNB hardware for generating this waveform while multiplexing this newly generated signal with legacy NR signals.

Observation 3: New OOK modulation and new OFDM overlaid sequence detection has been specified for LP-WUS detection, which are novel in the context of TS 38.101-4.

In terms of modulation, the parameter M can be 1, 2, or 4, representing the number of OOK symbols contained within a single OFDM symbol. Regarding the information payload, an LP-WUS can carry up to 5 information bits. These bits are mapped to one of 32 possible codepoints, each corresponding to a group of UEs. These codepoints indicate which UEs should begin PDCCH or Paging monitoring at a specific Paging Occasion (PO). This codepoint-based wake-up signaling mechanism is used for LP-WUS in both idle/inactive and connected modes.

In terms of coding, for # of information bits >=3 bits, Reed-Muller (RM) code is used with rate matching to match the number of OFDM symbols available. For more details, see TS 38.212 Section 5.3.3 where RM code is specified for Uplink Control Information (UCI). For 1 and 2 information bits, the procedure is given in the same Section 5.3.3.

Observation 4: For OOK envelope detection, LP-WUS coding aspects have both an information payload dimension and a code rate dimension.

In terms of the so-called Overlaid OFDM Sequences, these are specified to be on top of the OOK ON symbols, i.e. the ON duration of the OOK pattern transmitted for the LP-WUS. These overlaid sequences may or may not carry actual information. The receiver can perform sequence correlation in either time or frequency domain, and it is expected that an OFDM receiver can detect early the LP-WUS compared to an OOK receiver, depending on the number of candidate overlay sequences for each OOK ON symbol.

When using overlaid OFDM sequences, several details are important. First, the length of the overlaid OFDM sequences is given by 12 * X / M for all M values, where X = 11 is the number of PRBs for the LP-WUS, meaning the sequence length is 132, 66 and 33 for M = 1, 2 and 4, respectively. In addition, overlaid OFDM sequences use Zaddoff-Chu (ZC) sequences with cyclic extension. BZC is given by the largest prime number such that BZC < LZC, with the corresponding allowed root values and cyclic extensions, which are in turn obtained from RRC signalling. In addition, the maximum number of candidate overlaid sequences per OOK ON chip for one cell is 16 for M=1; 8 for M=2; and 4 for M=4. If the overlaid sequence does not carry information, a single overlaid sequence is on each OOK ‘ON’ symbol. Otherwise, the raw information bits are used to determine the overlaid OFDM sequences for each OOK ON chip. This is

- Assume L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

- Assume N is the number of raw information bits. If N / log2 L is not an integer, bit 0 as MSB is used for padding.

- The raw information bits are divided into K segments from MSB to LSB, where K = ceil(N / log2 L).

In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially. See Figure 3 below.

Figure 3: Overlay sequence to bit mapping in OOK waveform

Observation 5: For OFDM detection, LP-WUS can be transmitted in several ways based on the configured M-factor, which determines a maximum number of candidate overlaid sequences per OOK ON chip.

In general, it can be observed that several new aspects are introduced with this new LP-WUS waveform. Hence, RAN4 needs to discuss if and how to determine the relevant demodulation requirements based on the multiple novel aspects discussed above.

# Discussion

In terms of concrete parameters, we first need to recognize the different variations of the LP-WUR. During the last RAN4#116 meeting, it was agreed that we will consider both receivers and waveforms.

Observation 6: During RAN4#116 it was agreed to consider both OOK and OFDM receivers, and both waveforms OOK-1 and OOK-4.

In terms of the frequency range being considered for demodulation requirements, we believe we should take priority with FR1. We don’t see strong traction of deploying this feature, at least in Rel-19, on frequencies other than FR1. In addition, operators have strong interest in deploying the feature in a restricted number of frequencies, which justified even more that FR2 should be deprioritized at least from requirements point of view. RAN4 can decide once FR1 requirements are fully in place whether more time should be allocated to work on the FR2 side.

Observation 7: There is stronger interest in deploying the LP-WUS feature on a limited number of frequencies, and seems more cost effective that frequencies are in FR1.

Proposal 1: Prioritize FR1 requirements. RAN4 to continue with FR2 requirements after meaningful progress granted for FR1.

In terms of propagation related characteristics, antenna configuration and channels are relevant. We believe that the antenna configuration should be assumed as 1Tx/1Rx since this should be in any case a low-complexity feature for both BS and UE. In terms of propagation, both TDLA and TDLC channels could be taken into consideration since there are valid operating points. However, the expected mobility of a UE operating with this feature is assumed to be low. Hence, a good approach would be to define requirements based only on TDLA30-10.

Observation 8: For best LP-WUS/WUR operation, expectations are that UE should operate in rather low mobility conditions.

Proposal 2: For FR1, define TDLA30-10 as propagation channel.

Proposal 3: For FR2, define TDLA30-75 as propagation channel instead of the previously agreed TDLA30-10 in RAN4#116 such that sufficient doppler effect is considered.

In addition, RAN1 has defined for the OFDM LP-WUR receiver the maximum number of candidate overlaid sequences. The longer the chip duration due to M, the larger the maximum number of candidate overlaid sequences, and the number of roots. For mapping between a sequence index cm and root value (qcm) & cyclic shift (CS) value (ncs, cm).

qcm=qi ,   i=floor(cmP),  P=NseqNroot

ncs, cm=ncs, k,     where k=mod(cm, P)

where cyclic shift value ncs, kis determined by
ncs, k=k*floor BZC P, where k=0,…, P-1

At least for FR1, RAN1 design supports a maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4, with a maximum of 2 roots for each M value, and different roots can be picked for different M values.

Observation 9: For the OFDM LP-WUR, RAN1 has defined the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, with 16 candidates for M=1, 8 candidates for M=2 and 4 candidates for M=4, with corresponding mapping between roots and cyclic shifts.

Proposal 4: For the OFDM LP-WUR, consider M=2,4 for FR1. RAN4 to discuss on specific overlaid sequence configuration details.

The OOK LP-WUR receiver has a clear coding pattern based on the number of information OOK bits. For 1 and 2 information bits, coding is as described in the first row of TS 38.212, Section 5.3.3.1 and Section 5.3.3.2, respectively. For more than 3 information bits, Reed Muller coding is applied.

Observation 10: RAN4#116bis defined 12 or 16 coded (after rate-matching) bits for 2 information bits, and 32 coded bits for 5 information bits.

Proposal 5: Define OOK LP-WUR requirements for 12 or 16 coded bits for 2 information bits and 32 coded bits for 5 information bits. Consider M=4 as priority, and M=2 as second priority, down-selecting two test from these possible combinations based on companies results.

Proposal 6: Define separate LP-WUS demodulation requirements for envelope detection LP-WUR and sequence detection LP-WUR.

To perform agreed test based on the parameters discuss above, we need to discuss again any concern about the testability of the new LP-WUS signal. Like PBCH, LP-WUS lacks a direct mechanism to determine its detection rate. However, we can indirectly detect LP-WUS by configuring an appropriate PDCCH detection condition such that at the target SNR for LP-WUS guarantees quasi-perfect detection rate in PDCCH. We can clearly tell if a LP-WUS decoding instance was triggered by the UE by simply detecting the associated PDCCH, since the MR monitored the configured PDCCH windows directly.

Observation 11: LP-WUS miss-detection rate can be evaluated indirectly via the associated PDCCH detection rate.

Proposal 7: RAN4 to focus on defining demodulation requirements and its associated parameters since testability is clear.

Finally, another aspect that has being brought during this work item is the LP-WUS false alarm rate (FAR), but we prefer to leave this up to UE implementation. First, FAR is not a new concept, since it already happens in PDCCH detection, and it’s not a phenomenon that can be tested by the network or by TE vendor. Second, it’s up to UE implementation to minimize FAR in order to maximize the power saving gains of this feature.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: LP-WUS detection is revelant for both Idle/Inactive and Connected Mode, but signal design and its performance is fundamentally the same.

Observation 2: For LP-WUS detection, the LP-WUR receiver can be of OOK type and of OFDM type.

Observation 3: New OOK modulation and new OFDM overlaid sequence detection has been specified for LP-WUS detection, which are novel in the context of TS 38.101-4.

Observation 4: For OOK envelope detection, LP-WUS coding aspects have both an information payload dimension and a code rate dimension.

Observation 5: For OFDM detection, LP-WUS can be transmitted in several ways based on the configured M-factor, which determines a maximum number of candidate overlaid sequences per OOK ON chip.

Observation 6: During RAN4#116 it was agreed to consider both OOK and OFDM receivers, and both waveforms OOK-1 and OOK-4.

Observation 7: There is stronger interest in deploying the LP-WUS feature on a limited number of frequencies, and seems more cost effective that frequencies are in FR1.

Proposal 1: Prioritize FR1 study. RAN4 to continue with FR2 requirements after meaningful progress granted for FR1.

Observation 8: For best LP-WUS/WUR operation, expectations are that UE should operate in rather low mobility conditions.

Proposal 2: For FR1, define TDLA30-10 as propagation channel.

Proposal 3: For FR2, define TDLA30-75 as propagation channel instead of the previously agreed TDLA30-10 in RAN4#116 such that sufficient doppler effect is considered.

Observation 9: For the OFDM LP-WUR, RAN1 has defined the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, with 16 candidates for M=1, 8 candidates for M=2 and 4 candidates for M=4, with corresponding mapping between roots and cyclic shifts.

Proposal 4: For the OFDM LP-WUR, consider M=2,4 for FR1. RAN4 to discuss on specific overlaid sequence configuration details.

Observation 10: RAN4#116bis defined 12 or 16 coded (after rate-matching) bits for 2 information bits, and 32 coded bits for 5 information bits.

Proposal 5: Define OOK LP-WUR requirements for 12 or 16 coded bits for 2 information bits and 32 coded bits for 5 information bits. Consider M=4 as priority, and M=2 as second priority, down-selecting two test from these possible combinations based on companies results.

Proposal 6: Define separate LP-WUS demodulation requirements for envelope detection LP-WUR and sequence detection LP-WUR.

Observation 11: LP-WUS miss-detection rate can be evaluated indirectly via the associated PDCCH detection rate.

Proposal 7: RAN4 to focus on defining demodulation requirements and its associated parameters since testability is clear.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024