# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513603

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.20.4

Source:	Apple

Title:	On UE Demodulation Requirements for NTN Phase 3

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The UE demodulation requirements for Rel-19 NTN Phase 3 WI were discussed in RAN4#116 and way forward [1] was agreed. In this contribution we present views on open issues and initial simulation results for the agreed test cases for defining requirements.

# 2. Discussion

In RAN4#116 the test cases were agreed for requirements for RedCap and eRedCap for NTN. The open issues were related to FRC for some of the test cases.

Issue 2-1-2: FRC for HD-FDD RedCap demodulation

Way forward:

- Reuse “R.PDSCH.1-1.2 HD-FDD” in TS38.101-4 for MCS13/52PRB test.

- FFS whether to define a new HD-FDD reference channel for MCS4/52PRB tests.

  - Note: R.PDSCH.1-1.1 HD-FDD in TS 38.101-3 configures 18 DM-RS REs. MediaTek proposes to configure 12 DM-RS REs (1+1) instead of 18 REs (1+1+1).

Issue 2-2-3: FRC for eRedCap demodulation

Way forward:

- Reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth

- FFS for other test cases.

It is unclear what re-use of FRC from TS 38.101-4 means. For all requirements specified in 38.101-5 an existing or new FRC would be needed in 38.101-5. For HD-FDD requirements in R17 RedCap and R18 eRedCap the same test requirement as FDD was used with FRC for HD-FDD mode. The same would follow in NTN requirements for RedCap and eRedCap. All the test cases for HD-FDD would need a new FRC in 38.101-5.

- It is unclear what reusing FRC from 38.101-4 means.

- All requirements defined in 38.101-5 would need a corresponding FRC in 38.101-5.

- HD-FDD test cases in 38.101-5 would need a new FRC based on test configuration.

- Define new FRCs for HD-FDD test cases applicable to RedCap and eRedCap.

# 3. Simulation Results

For the test cases agreed in RAN4#116 in [1] initial alignment results are provided.

RedCap 1Tx-1Rx, 10MHz/15kHz, FDD/HD-FDD

Table 1: Simulation results for RedCap with 1RX

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 | 2.2 |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 | 0.8 |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 | 8.3 |

eRedCap 1Tx-1Rx/2RX, 10MHz/15kHz, FDD/HD-FDD

Without reduced baseband bandwidth capability

Table 2: Simulation Results for eRedCap - without reduced BW capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 2.2 |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | 8.3 |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | Reuse from Rel-17 |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | 4.8 |

With reduced baseband bandwidth capability

Table 3: Simulation Results for eRedCap - with reduced BW capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | 2.5 |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | 7.7 |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | -1.6 |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | 4.3 |

# 3. Conclusion

In this paper, we present our views on UE demodulation requirements for NTN Phase 3 WI and initial simulation results. Our observations and proposals are captured below:

- It is unclear what reusing FRC from 38.101-4 means.

- All requirements defined in 38.101-5 would need a corresponding FRC in 38.101-5.

- HD-FDD test cases in 38.101-5 would need a new FRC based on test configuration.

- Define new FRCs for HD-FDD test cases applicable to RedCap and eRedCap.

# Reference

- R4-2512644, “Way Forward for [116][331] NR_NTN_Ph3_demod”, Ericsson