# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513605

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	4.8.2

Source:	Apple

Title:	On RRM core requirement for R19 LB CA

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_LBCA_Sw [1] have been agreed in RAN4#116, and the open issues have been summarized in the topic summary for [116][206] NR_LBCA_Sw [2]. Even though the core part of WI has been completed, we still need to address some remaining issues in the maintenance stage.

In this contribution, we continue the discussion for RRM requirement of LB CA via switching.

# SDL SCC related RRM requirement

##### 2.1 Activated SDL SCell L3 measurement

Issue 1-3-1:  Requirement design of activated SDL SCell L3 measurement

This issue has been concluded in last meeting [1].

#### 4.2 Deactivated SDL SCell L3 measurement

Issue 1-3-2: Interruption requirement design of deactivated SDL SCell measurement

This issue has been concluded in last meeting [1].

#### 4.3 SDL SCell activation/deactivation

Issue 1-3-3: interruption or scheduling restriction during SDL SCell activation/deactivation

This issue has been concluded in last meeting [1].

Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA

- [Agreement]: - FFS this issue in maintenance stage.

It’s still FFS for CQI and L1-RSRP reporting during the SDL SCell activation. In legacy SCell activation requirement, UE is required to report OOR and lowest L1-RSRP right after the MAC CE decoding.

However, such CQI or L1-RSRP reporting is on PCell, and when UE needs to report CQI/L1-RSRP on PCell, UE cannot stay on SCell for DL synchronization if the SSB or SMTC is colliding with the CQI/L1-RSRP reporting occasion. Like in figure 3,

Figure 1. the CQI/L1-RSRP on PCell colliding with the SSB/SMTC for SDL SCell activation

It’s unclear how UE can handle the CQI/L1-RSRP reporting during the SCell activation procedure, and our view is to prioritize the SSB/SMTC measurement over the OOR or lowest L1-RSRP reporting to complete the SDL SCell activation as soon as possible. However, the SMTC and CQI reporting or L1-RSRP reporting resource is configured by network, network can simply avoid such colliding case. Thus, we think the colliding case shall be treated as an error case since there is no benefit to do so. In addition, UE only needs to switch to SCC when SMTC window or RS occasion comes, and therefore UE can stay on PCC for these UL reporting outside the SMTC window or RS occasions on SCC, i.e., no CC switching and interruption is needed for CQI/L1-RSRP reporting.

Proposal 1: If CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA.

Issue 1-3-5: Activation of Switching Pattern

[Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

Regarding the application time for switching pattern after the SCell activation command, we identified one extra issue to be clarified in RRC based direct SCell activation. In current RRC based SCell activation requirement, following time margin was considered:

It very clearly specified that the activation time starts from the timing when UE sends the RRCReconfigurationComplete message. However, the pattern application in the previous agreement is counted from the end of RRC processing time. Network can only assume the SCell addition is completed when it receives RRCReconfigurationComplete message, and it’s very important to make sure UE and network/UE have the common understanding on the states for carrier switching between FDD and SDL. For example, network may assume the switching pattern can be applied after n+(TRRC_Process + TLBCA)/slot-length but UE may not be able to add this SCell and cannot send RRCReconfigurationComplete back. We think it would be easier for UE and network to start the SCell related behavior after the RRC completion, i.e., for RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + T1 + TLBCA)/slot-length, where T1 is delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message.

Figure 1. Legacy direct SCell activation starts from RRC reconfiguration completion

Proposal 2: for RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + T1 + TLBCA)/slot-length, where T1 is delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message.

Regarding the TLBCA time margin for the switching pattern application, we think it might not need that long time like 4ms, since in last meeting RAN4 agreed to apply switching pattern during SCell activation procedure. In our view, for SCell addition, UE can load the RRC configured switching pattern to the MAC layer as the other SCell configurations in advance, and then when UE receives the SCell activation command, the switching pattern will be applied directly to PHY layer. The time margin for the switching pattern application can be reduced to 1ms.

Proposal 3: TLBCA time margin for the switching pattern application is 1ms (Option 2) for both MAC CE and RRC based SCell activation.

Regarding when UE can perform carrier switching during the SCell activation for LB CA, we didn’t see any complexity for UE to perform switching immediately after switching pattern application as long as the switching time FDD-to-SDL and SDL-to-FDD can be contained in the residual time in the current SDL duration based on the switching pattern. As shown as the following figure, even though UE completes pattern application in middle of a SDL duration, UE shall be able to switch to the SDL SCC immediately as long as the switching time FDD-to-SDL and SDL-to-FDD can be contained in the residual time of this duration. Comparing with waiting for the next available duration, it can also help UE/NW to complete the SCell activation as early as possible.

Figure 2. UE can perform the switching immediately after application as long as switching time can be contained in the residual time of duration.

In addition, for RRC based SCell activation, we also assume that UE shall perform this carrier switching after RRCReconfigurationComplete message, since the legacy SCell activation procedure (e.g., SCell sync, T/F tracking and etc) starts from RRCReconfigurationComplete message.

Thus, we propose:

Proposal 4:

- For MAC CE based SCell activation in LB CA via switching, according to the switching pattern,

  - UE switches to SCell immediately after n+(THARQ+3ms+TLBCA)/slot-length.

- For RRC based direct SCell activation, according to the switching pattern,

  - UE switches to SCell immediately after n+(TRRC_Process+T1+TLBCA)/slot-length, where T1 is delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message.

Issue 1-3-5a: Deactivation of Switching Pattern

[Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Similar as issue 1-3-5, the application only needs to consider how to enable or disable the switching pattern at UE, and therefore 1ms margin is sufficient. Regarding when UE performs switching, in order to maximize the FDD throughput for both UE and network, UE shall perform the switching immediately after the application time. Here the application time is for UE to disable the switching pattern for the SDL SCC. To simplify the UE implementation, we also believe UE can perform the SDL-FDD switching at the end of the slot.

Proposal 5:

- The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length).

  - TLBCA =1ms

- If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after the n+(THARQ+3ms+ TLBCA)/slot-length. The carrier switching, if applicable, shall be placed at the end of a slot.

#### 4.4 Others for SDL SCell

Issue 1-3-9: BWP switching for SDL SCell

The first slot for new active BWP operation in BWP switching requirement needs to be clarified for the LB CA case. As shown in the following figure, if active BWP switching is triggered on FDD PCC, however, the BWP switching processing time (1ms or 3ms) is longer than the residual time within the current PCC duration time according to the switching pattern, then the first slot for new active BWP operation on PCC may not be the one at n+TBWP-swtiching. There might be 3 different cases for UE implementation when such case happens:

Case 1 and 2:

- For BWP switching on FDD PCC, if active BWP switching is triggered but the residual time of the PCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP from first DL/UL slot in following FDD PCC duration after the BWP switching processing time.

- For BWP switching on SDL SCC, if active BWP switching is triggered but the residual time of the SCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP from first DL slot in following SDL SCC duration after the BWP switching processing time.

Case 3:

- For BWP switching on FDD PCC, if active BWP switching is triggered but the residual time of the PCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP after the BWP switching processing time from first DL/UL slot in following FDD PCC duration.

- For BWP switching on SDL SCC, if active BWP switching is triggered but the residual time of the SCC duration based on RRC configured switching pattern is not sufficient for BWP switching processing, UE will start operation on the new BWP after the BWP switching processing time from first DL slot in following SDL SCC duration.

Case 1

Case 2

Case 3

Figure 3. example for active BWP switching in LB CA

In addition, since the FDD-SDL carrier switching happens in middle of BWP switching processing, it’s unclear that if the active BWP switching delay shall be extended or not. In our view, no matter which case is considered for implementation, it would be very complicated for UE to coordinate BWP switching colliding with the carrier switching, and such case has never been considered for other CA scenario before. On the other hand, the active BWP switching is a known operation to the network since anyway network needs to know when it can schedule UE on the new active BWP, and therefore to simplify UE and network behavior, RAN4 can assume that NW shall avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching and no requirement is needed to consider such colliding case.

Proposal 6: RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case.

Issue 1-3-10: others for SDL SCell

- Proposal 1 (Ericsson): RAN4 to further discuss CSSF inside MG for the LB CA. - Proposal 2 (Apple): the CSSF inside MG can be FFS (contribution-driven) in the maintenance stage

Regarding the MG application, we identified one possible issue for SDL SCell measurement with MG. The existing CSSF for measurement inside MG is as following,

The intra-frequency MO without MG shall be counted in the CSSF inside MG only when it meet above highlighted conditions, however, the existing fully overlapped or partially overlapped conditions may not be accurate when LB CA via switching is used. Like in the figure 3, the MO on PCC is intra-frequency MO without MG, and the SMTCs on the PCC is not fully overlapped with MG for inter-frequency measurement, i.e., it’s partially overlapped case if only considering the SMTC configuration and MG configurations. However, effectively, UE cannot perform measurement in the SMTC of PCC when it’s colliding with the SDL SCC staying duration (in which UE switched to stay on SDL SCC and cannot receive anything from PCC). In this case, PCC SMTC and MG are fully overlapped effectively; and SDL SCC SMTC and MG are fully non-overlapped effectively

Figure 4. fully overlapped and partially overlapped conditions for CSSF in LB CA via switching

Thus, in LB CA via switching, the layers or MOs counted in the CSSF inside MG shall be based on the effective SMTCs of that layer or MO. The effective SMTC means:

(1) the SMTCs of PCC outside the duration time of staying on SDL SCC are treated as effective SMTCs of PCC; and

(2) the SMTCs of SDL SCC inside the duration time of staying on SDL SCC are treated as effective SMTCs of SCC

Figure 5. effective SMTCs to decide overlapping condition for CSSF inside MG

In summary, the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

Proposal 7: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

SDL SCell activation/deactivation

Proposal 1: If CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA.

Proposal 2: for RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + T1 + TLBCA)/slot-length, where T1 is delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message.

Proposal 3: TLBCA time margin for the switching pattern application is 1ms (Option 2) for both MAC CE and RRC based SCell activation.

Proposal 4:

- For MAC CE based SCell activation in LB CA via switching, according to the switching pattern,

  - UE switches to SCell immediately after n+(THARQ+3ms+TLBCA)/slot-length.

- For RRC based direct SCell activation, according to the switching pattern,

  - UE switches to SCell immediately after n+(TRRC_Process+T1+TLBCA)/slot-length, where T1 is delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCReconfigurationComplete message.

Proposal 5:

- The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length).

  - TLBCA =1ms

- If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after the n+(THARQ+3ms+ TLBCA)/slot-length. The carrier switching, if applicable, shall be placed at the end of a slot.

Others for SDL SCell

Proposal 6: RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case.

Proposal 7: the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

# References

[1] R4-2512119, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #116

[2] R4-2509049, Topic summary for [116][206] NR_LBCA_Sw, Apple, RAN4#116