# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513609

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	4.25.3

Source:	Apple

Title:	On R19 NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#116, and the open issues of coverage enhancement have been summarized in the topic summary for [116][227] NR_NTN_Ph3_Part2 [2].

In this contribution, we continue the discussion on the RRM requirements for R19 NTN DL coverage enhancement.

# Topic #1: SSB periodicity larger than 20 ms

[Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6)

RAN4 to further discuss the following in RAN4#116-bis: - For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant: - The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE. - Note: The SMTC selection rule may be further discussed and specified or left to UE implementation. Agreement (Ad-hoc): - RRC Idle/Inactive mode RRM requirements for more than 4 SMTCs are applicable to the earth-moving cell and earth-fixed deployments.

In last RAN2#131 meeting, followings were agreed in LS (R2-2506298):

## Agreements in #131: For downlink coverage enhancement, RAN2 further made the following agreements related to SMTC enhancement: - We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. - In connected mode the NW will only configure up to 4 STMC. - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

Firstly, we need to differentiate the cases between connected mode and IDLE/inactive mode. For RRC connected mode, network is aware of which serving cell or footprint UE is located in, and therefore the RRC configuration of SMTC shall consider the rough UE location (cell level). In RAN2 #131 meeting, the UE assisted SMTC configuration in connected mode has been agreed.

However, in IDLE/Inactive mode, network is unaware of any serving cell information for a particular UE. RAN2 agreed that network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. That can really help UE to understand which SMTC(s) can be used for cell searching at a specific location, and additionally, in RAN2 #131 meeting, it was agreed that “RAN2 reconfirms the understanding that if the NW configures more than 4 SMTCs the UE in idle/inactive the UE selects up to 4 SMTCs (or less, depending on the UE capability) based on location and only considers them for measurements”.

Thus, we assume UE can select up to 4 SMTCs associated with the closest reference locations for measurement in IDLE/Inactive mode.

Proposal 1: For RRC Idle/Inactive mode RRM requirements, if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement.

# Topic #2: Satellite Beam Hopping (e.g. Cell DTX/DRX)

In last RAN1 FL Summary #4: NR-NTN downlink coverage enhancements (R1-2501728), following issue was discussed:

However, no conclusion has been achieved in last RAN1 meeting. In the current TN FR1 scheduling restriction for L1 or L3 measurement, it has two main kinds:

- TDD based scheduling restriction because UE cannot do UL transmission when UE is operation on DL for neighbor cell measurement.

- Other scheduling restriction applies because UE cannot do e.g., mixed numerology Rx or Tx when UE is performing SSB or CSI-RS measurement.

R17 NTN has new scheduling restriction if UE does’t support parallelMeasurementWithoutRestriction, when UE is performing SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

The above scheduling restriction happens during the SSB measurement occasions of the target cells, e.g., UE is doing SSB measurement but cannot receive the data/control or transmit data/control/RACH to the serving satellite. However, in this special case for DL coverage enhancement with Tx beam hopping, even on other time period outside the configured SSB occasions, NW may be still not possible to schedule or receive any channel/signal to or from UE, as shown in following figure 1. To the example UE in cell 2, outside the cell 2 SSB occasions, before T2 and after T2+40ms, NW hopped Tx beams to other cells (cell 1/3/4), and therefore serving satellite cannot transmit/receive anything to or from UE in cell 2. Therefore, we need to reconsider the scheduling restriction applicability in this scenario of DL coverage enhancement.

Figure 1. cell 2 UE(s) may not transmit or receive signaling from satellite when beam hopped away

If the RO periodicity configuration from serving cell is not greater than the SSB periodicity (less than 160ms in the example figure 2), the scheduling restriction applies to UE RACH and RACH related channels transmission outside the time period of serving Tx beam availability, time period of serving Tx beam availability means, e.g., the time period between the beginning of slot containing available SSB (i.e., T2 in example figure 2) and (original SSB periodicity + T2). Thus, the scheduling restriction for RACH could be: If DL coverage enhancement or satellite beam hopping is enabled, the UE is not expected to transmit PRACH on symbols outside the serving satellite Tx/Rx beam availability durations.

Figure 2. example for RO in SSB periodicity

Proposal 2: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# Conclusion

In this contribution, we discuss the RRM requirement for Non-Terrestrial Networks (NTN) NR Phase 3.

Proposal 1: For RRC Idle/Inactive mode RRM requirements, if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement.

Proposal 2: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# References

[1] R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, RAN4#116

[2] R4-2509070, Topic summary for [116][227] NR_NTN_Ph3_Part2, Qualcomm, RAN4#116