3GPP TSG-RAN WG4 Meeting#116bis		R4-2513641

Prague, Czech, Oct 13th -17th, 2025

Agenda item:	7.8.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE RF requirements for R20 LB-LB CA switching

Document for:	Discussion

Introduction

In RAN-P#109 meeting, a new WID [1] was approved. In this contribution, we want to provide some initial views on UE RF requirements for LB-LB CA switching.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Discussion

UE RF requirements

Switching delay

According to the new WID, CA_n12A-n29A can be as an example to be analysed in this WI. In Figure1, it is observed

Figure1: CA_n12A-n29A

that there is only 1MHz gap between n29 DL and n12 DL. Therefore, a wide filter is needed to support DL CA. Based on the WI, case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2; case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. Switching between case 1 and case 3 to enable low band carrier aggregation for the dual downlink scenario between FDD band and SDL band. Similarly, with Rel-19 LB-LB CA switching, switching delay also depends on UE architecture implementation.

UE reference architecture

Figure1: UE reference architecture for CA_n12A-n29A.

Based on UE reference architecture, there are common antenna, RF switching and mixer2 for n12A_Tx and DL CA_n12A-n29A. Therefore, the switching delay between case1 and case3 is the same as Rel-19 switching delay for LB-LB CA.

Proposal 1: Figure1 can be UE architecture assumption in R20 LB-LB CA switching. Reuse the switching periods (35us, 70us and 140us) for switching between Case1 and Case3 as an optional UE capability in TR 38.768.

Switching period location

■ FDD to DL CA switch

According to the WID, we need to reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching. For the FDD to DL CA switch, the switching gap ends at end of slot on“switch from” carrier(FDD). Figure 2 shows the case with TTA > 0 µs. The figure illustrates the applicable transient periods, switching period (35𝜇s, 70𝜇s, and 140𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

Proposal 2: the switching period location can refer to the figure2 for FDD to DL CA switch.

■ DL CA to FDD switch

According to the WID, we need to reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching. For the DL CA to FDD switch, the switching gap ends at end of slot on“switch from” carrier(DL CA). Figure 3 shows the case with TTA > 0 µs. The figure illustrates the applicable transient periods,

Proposal 3: the switching period location can refer to the figure3 for DL CA to FDD switch.

UL time mask

For the UL time mask, similar with R19 LB-LB CA switching, FDD band is the only Tx, the switching period and interruption in the end slot in FDD band for Tx. We can reuse the time mask which is defined in TR 38.768. Figure 4 below illustrates the ON/OFF time mask for low NR band carrier aggregation via switching.

Figure4: ON/OFF time mask for NR UL transmission for DL CA via switching in the UL

Proposal 4: UL time mask R19 LB-LB CA switching can be reuses in R20 LB-LB CA switching.

Fall back behavior

When the UE supports the DL CA via switching, which means UE have the possibility to support the low band CA via switching with non-CA. Thus, R19 UE capability of supportedLowBandSwitching-r19 can be as one of fallback configurations for the UE supports R20 DL CA via switching capability. In our understanding, this fallback behavior needs to be further discussed.

Proposal 5: Fall back behavior for R20 DL CA via switching needs to be further discussed.

RX requirements

Base on the TR 38.768, the note in chapter 5.7 mentions that when introducing higher order CA with LB CA switching as one of fallback configurations and/or different device type of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) RAN4 should discuss ΔRIB,c requirements. Based on the UE architecture above, there is more insertion loss produced by the switching for DL CA. From our point of view, ΔRIB,c requirements need to discussed. In addition, there is no band combination for CA_n12_n29A in TS 38.101-1, REFSENS also needs to be discussed.

Proposal 6: REFSENS and ΔRIB,c requirements are all need to be discussed.

Conclusions

In this paper, we provide our initial views on UE RF requirements for R20 LB-LB CA, we have made the following proposals.

Proposal 1: Figure1 can be UE architecture assumption in R20 LB-LB CA switching. Reuse the switching periods (35us, 70us and 140us) for switching between Case1 and Case3 as an optional UE capability in TR 38.768.

Proposal 2: the switching period location can refer to the figure2 for FDD to DL CA switch.

Proposal 3: the switching period location can refer to the figure3 for DL CA to FDD switch.

Proposal 4: UL time mask R19 LB-LB CA switching can be reuses in R20 LB-LB CA switching.

Proposal 5: Fall back behavior for R20 DL CA via switching needs to be further discussed.

Proposal 6: REFSENS and ΔRIB,c requirements are all need to be discussed.

- Reference

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching. TSG-RAN WG 109 meeting

- TR 38.768