3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513742

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Verification of RB&BW extension framework for 2Tx PC2/1.5 and R18 boosting

Source: 	Skyworks Solutions, Inc.

Agenda Item:	4.6.1.2	Power domain enhancement for NR single carrier and NR intra-band UL CA for PC2 and PC3	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#116, the MPR reduction based on RB/BW extension feature was finalized and captured in Big CR [2]. At this time, the feature was restricted to PC2 and PC2 1Tx as indicated in the WI. Moreover, there were no evaluations available for the 2Tx PC2 and 2Tx PC1.5 cases. In this contribution, we provide measured verifications for the 2Tx PC2 and 2Tx PC1.5 cases to make the MPR reduction based on RB/BW extension applicable to those cases too. The same measurements are also used to gauge the applicability to the R18 power boosting option.

# Discussion

## 2.1 Verification of extension ratio principles for 2Tx PC2 and 2Tx PC1.5.

To verify the proposed extension ratio principles for 1Tx PC2 and PC1.5,on top of the 1Tx PC3 and PC2 measurements provided in [1], we have measured the same waveforms in a 2Tx PC2 and PC1.5 setup for OOB emissions (ACLR/SEM/Spurious emissions for the worst-case proposed extensions ratios (asymmetric extension on the lower frequency side as this has the lowest guard band) for 5, 20, 50 and 100MHz channel bandwidths.

The raw analysis from [1] for 1Tx PC3 and 1Tx PC1 is copied in annex section 5.1 and 5.2 respectively. The raw analysis of the new measurements for 2Tx PC2 is presented in Annex section 5.3 and the analysis of the 2Tx PC1.5 is presented in Annex section 5.4.

To facilitate the observations, Table 1 below provides the worst-case margin for all the measured extended cases.

Table 1: ACLR and SEM (low and high) margin for new inners based on RB/BW extension

Observations for PC2 2Tx and PC1.5 2Tx:

- The margin is positive for all cases showing the RB/BW extension framework works for PC2 2Tx and PC1.5 2Tx as it was already shown in [1] for Pc3 1tx and PC2 1Tx.

- In general, the margin is higher for:

  - ½ extension versus ¼ extension

  - Smaller CBW

  - Lower power class (it is partly due to the fact that the difference between outer and inner MPR difference gets larger for higher power class and 2Tx versus 1Tx).

- The tighter case is for the ¼, ¼ case.

Proposal on modification to enable PC2 and PC1.5 2Tx:

- RB/BW extension framework should be enabled for PC2 2Tx and PC1.5 2Tx:

  - PC3 and PC2 restriction highlighted in yellow below in following 38.101-1 clause 6.2.2 is modified to include PC1.5 and 1Tx/2Tx as highlighted in yellow below:

If all of the following conditions are simultaneously met:

-	The UE is a PC3 or PC2 PC3, PC2 or PC1.5 UE supporting 1Tx or 2Tx operating in any channel bandwidth except 3MHz

-	The UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19

-	The uplink modulation order is either QPSK or 16QAM

-	When no A-MPR is applicable

  - 38.101-1 clause 6.2D.2 text below Table 6.2D.2-5 is modified for PC1.5 edge allocation as highlighted in yellow below.

Inner, outer and edge allocations are as defined in section 6.2.2 except for PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes.

When a UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19, an Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost edge of the channel LCRB ≤ 4 RBs with Rext_low = 0, or the one for which the RB(s) is (are) allocated at the uppermost edge of the channel LCRB ≤ 4 RBs with Rext_high = 0.

## 2.1 Verification of extension ratio principles for 2Tx PC2 and 2Tx PC1.5.

To verify the proposed extension

To evaluate the possibility too boost the “new inner” according to the R18 feature, as we did not have time to measure the boosted the power level, we can look at the margins to the QPSK DFT-s-OFDM 1Tx PC3, 1Tx PC2 and 2Tx PC2 OOB emissions in Table 1 and apply the following criteria:

- It is safe to consider that cases that >3dB margin can be boosted by 1dB (PC3) and >2dB margin by 0.5dB (PC2)

- It should be feasible to boost the PC3 cases that have 2dB to 3dB margin by 1 dB

- It should be feasible to boost the PC2 cases that have 1dB to 3dB margin by 0.5 dB

- R18 power boosting does not apply to PC1.5

- R18 power boosting applies to QPSK inner DFT-s-OFDM with 1/0.5dB boosting for PC3/PC2 (we do not have data for Pi/2 BPSK.

Observations on R18 boosting options:

- For 5MHz PC3 and PC2: The margin seems sufficient except for:

  - SEMlow for ¼ low extension for 1Tx PC3 and 1Tx PC2

- For 20MHz PC3 and PC2: The margin seems sufficient

- For 50MHz PC3 and PC2: The margin seems sufficient except for:

  - ACLRlow ¼ low extension for 1Tx PC3 and

  - SEMlow ½ and ¼ low extension for 1Tx PC2

- For 100MHz PC3 and PC2: The margin seems sufficient except for:

  - ACLRlow ¼ low extension for 1Tx PC3 and

  - SEMlow ½ and ¼ low extension for 1Tx PC2

  - Some ACLRhigh and SEMhigh marginal cases for 2Tx PC2.

Proposal on enabling R18 power boosting: Although our data shows margin for new inners to enable boosting, further verification is needed for the few PC3 and PC2 cases with less than 3dB and 1.5dB margin respectively.

## 2.3 Applicability to small channel BW

One of the reasons to use the initial CBW to define the ACLR and SEM measurement BW in the extended cases is to make sure the spurious emission region of the extended SEM guaranty that the ITU -30dBm/MHz at 250% BW is met by coinciding with the spurious emission region.

This means that: extended region+CBW+5MHz ≤ 2*CBW => CBW ≥ extended region+5MHz. This is tabulated in Table 2 for the lowest channel bandwidths.

Table 2: Minimum channel bandwidth to meet ITU 250% criteria

| CBW [MHz] |  | 3 | 5 | 7 | 10 |
| --- | --- | --- | --- | --- | --- |
| Extension | 1/4 | 5.75 | 6.25 | 6.75 | 7.5 |
|  | 1/2 | 6.5 | 7.5 | 8.5 | 10 |

Observations:

- It is only possible to meet the 250% criteria for ½ and ¼ extensions at CBW ≥ 10MHz

- For 7MHz it is only possible to meet the 250% criteria for ¼ extensions

- It should be noted that the new 6MHz CBW does meet the criteria for both ¼ and ½ extensions like 3MHz and 5MHz.

- It is safe to remove 3MHz and 7MHz as the CBW have been created to fit a specific operator spectrum thus they are unlikely to be extended.

- However, it is still possible that at least the 5MHz CBW would meet the criteria as the SEM at least guaranties -25dBm/MHz and our measurements do not show that this is the limiting factor for the SEM. In any case, a specific test could be added to verify this.

Proposal on min CBW:

- On top of 3MHz, 7MHz is excluded from the RB/BW extension framework. 38.101-1 clause 6.2.2 is modified to exclude 7MHz CBW

- Further checks for the 250% BW criteria can be performed for 5MHz CBW and if necessary additional testing can be performed for this CBW.

# Conclusion

In this contribution, we verified that the principles of extension ratios also apply to 2Tx PC2 and 2Tx PC1.5 and also evaluated the feasibility to enable R18 power boosting.

Proposal on modification to enable PC2 and PC1.5 2Tx:

- RB/BW extension framework should be enabled for PC2 2Tx and PC1.5 2Tx:

  - PC3 and PC2 restriction highlighted in yellow below in following 38.101-1 clause 6.2.2 is modified to include PC1.5 and 1Tx/2Tx as highlighted in yellow below:

If all of the following conditions are simultaneously met:

-	The UE is a PC3 or PC2 PC3, PC2 or PC1.5 UE supporting 1Tx or 2Tx operating in any channel bandwidth except 3MHz

-	The UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19

-	The uplink modulation order is either QPSK or 16QAM

-	When no A-MPR is applicable

  - 38.101-1 clause 6.2D.2 text below Table 6.2D.2-5 is modified for PC1.5 edge allocation as highlighted in yellow below.

Inner, outer and edge allocations are as defined in section 6.2.2 except for PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes.

When a UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19, an Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost edge of the channel LCRB ≤ 4 RBs with Rext_low = 0, or the one for which the RB(s) is (are) allocated at the uppermost edge of the channel LCRB ≤ 4 RBs with Rext_high = 0.

Proposal on enabling R18 power boosting: Although our data shows margin for new inners to enable boosting, further verification is needed for the few PC3 and PC2 cases with less than 3dB and 1.5dB margin respectively.

Proposal on min CBW:

- On top of 3MHz, 7MHz is excluded from the RB/BW extension framework. 38.101-1 clause 6.2.2 is modified to exclude 7MHz CBW

- Further checks for the 250% BW criteria can be performed for 5MHz CBW and if necessary additional testing can be performed for this CBW.

# References

[1] R4-2509943 Proposal on OOB range and RB extension principles for MPR reduction, Skyworks Solutions Inc., RAN4#116

[2] R4-2511761 Big CR for TS 38.101-1: Introduction of MPR reduction, Huawei, HiSilicon, vivo, ZTE, Nokia, China Telecom, CATT, xiaomi, Skyworks Solutions Inc., OPPO, AT&T, Apple, LG Electronics, Qualcomm, MediaTek, Murata, RAN4#116

# Annex: Raw measurement data for 2Tx PC2 and PC1.5

The Tables Ax provides the RAW measured verification data per the Table A0 template for ACLR and SEM+Spurious emissions margins for:

- PC3 1Tx in chapter 5.1 and PC2 1Tx in chapter 5.2

- The two cases above are copied from [1] but tables have been corrected as some columns were swapped, this does not change the earlier conclusions

- PC2 2Tx in chapter 5.3 and PC1.5 2Tx in chapter 5.4

- Channel bandwidth / SCS of: 5MHz/15kHz, 20MHz/15kHz, 50MHz/15kHz and 100MHz/30kHz

- CP-OFDM and DFT-s-OFDM for various allocations with their Inner/Outer definition as per the extension ratios. Allocations on the right side have been used most often as they have the lowest distance to where the emissions apply.

- Pass/fail for inner/outer

- Baseline extension ratio {0,0} applying MPR according to inner, outer definition

- Optional extension ratio {X,Y} with X and Y = 0, ½, ¼ applying MPR according to inner only

Table A 0: template for tables in Annex

| CBW/SCS |  |  | Channel Bandwidth and SCS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform | CP/DF |  | Waveform type (QPSK CP-OFDM/DFT-s-OFDM) |  |  |  |  |  |  |
|  | Alloc |  | Allocation LCRB/RB/RBstart |  |  |  |  |  |  |
| Pout w MPR |  |  | Measured output power (with MPR for {0,0} with inner MPR for others) |  |  |  |  |  |  |
| Extension {0,0} | I/O |  | Inner (I) or outer (O) tag as extension ratio |  |  |  |  |  |  |
|  | OK/KO |  | OK | inner pass = expected | OK+ | Outer pass = better | KO | outer fail= expected |  |
|  | ACLR | low | Lower ACLR margin |  |  |  |  |  | fail |
|  |  | high | Higher ACLR margin |  |  |  |  |  | marginal |
|  | SEM+ SPUR | low | Lower SEM+Spurious emissons margin |  |  |  |  |  | Small margin |
|  |  | high | Higher SEM+Spurious emissons margin |  |  |  |  |  | Large margin |

## 5.1 PC3 1Tx measurements for 5, 20, 50 and 100MHz CBW

Table A 1: 5MHz CBW with 15kHz SCS PC3 1Tx verification versus extension ratios

Table A 2: 20MHz CBW with 15kHz SCS PC3 1Tx verification versus extension ratios

Table A 3: 50MHz CBW with 15kHz SCS PC3 1Tx verification versus extension ratios

Table A 4: 100MHz CBW with 30kHz SCS PC3 1Tx verification versus extension ratios

## 5.2 PC2 1Tx measurements for 5, 20, 50 and 100MHz CBW

Table A 5: 5MHz CBW with 15kHz SCS PC2 1Tx verification versus extension ratios

Table A 6: 20MHz CBW with 15kHz SCS PC2 1Tx verification versus extension ratios

Table A 7: 50MHz CBW with 15kHz SCS PC2 1Tx verification versus extension ratios

Table A 8: 100MHz CBW with 30kHz SCS PC2 1Tx verification versus extension ratios

## 5.3 PC2 2Tx measurements for 5, 20, 50 and 100MHz CBW

Table A 9: 5MHz CBW with 15kHz SCS PC2 2Tx verification versus extension ratios

Table A 10: 20MHz CBW with 15kHz SCS PC2 2Tx verification versus extension ratios

Table A 11: 50MHz CBW with 15kHz SCS PC2 2Tx verification versus extension ratios

Table A 12: 100MHz CBW with 30kHz SCS PC2 2Tx verification versus extension ratios

## 5.4 PC1.5 2Tx measurements for 5, 20, 50 and 100MHz CBW

Table A 13: 5MHz CBW with 15kHz SCS PC1.5 2Tx verification versus extension ratios

Table A 14: 20MHz CBW with 15kHz SCS PC1.5 2Tx verification versus extension ratios

Table A 15: 50MHz CBW with 15kHz SCS PC1.5 2Tx verification versus extension ratios

Table A 16: 100MHz CBW with 30kHz SCS PC1.5 2Tx verification versus extension ratios