3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513792

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Inputs to 6MHz channel bandwidth

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.4 6MHz [NR_UE_RF_Ph5-Core]

Document for:	Approval

# 1	Background

In RAN#109, a new 5GA Release 20 WI [1] was agreed on FR1 enhancements and includes the introduction of 6MHz channel bandwidth. In this contribution, we provide our preliminary views on feasibility.

# Discussion

## 2.1 HPUE objectives in R20 new WI: UE RF enhancements for NR FR1

The new Release 20 WI UE RF enhancements for NR FR1 [1] introduces 6MHz channel bandwidth. The corresponding RAN4 objectives are copied below in italics.

6MHz channel bandwidth

- Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4]

  - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements (OTA and conducted) for UL and DL

- Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed

    - Additional Maximum Power Reduction (A-MPR)

NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

In this contribution, we provide our preliminary input on NRB (spectrum utilisation) and required work for NS.

## 2.2 NRB for 6MHz

Since 5MHz and 7MHz CBW support 25RB and 35RBs at 15kHz SCS respectively, it is logical to use 30RB for 6MHz. This number of RB is applicable to both CP-OFDM and DFT-OFDM as with 5MHz (35RB is only applicable to CP-OFDM for 7MHz).

Based on this, the 6MHz REFSENS and RSD requirements can be derived for the 5MHz and 10MHz CBW specifications as shown in Table 1.

Table 1: 6MHz receiver requirements derived from 5MHz and 10MHz

| CBW | NRB | Band | ideal REFSENS [dBm] | REFSENS [dBm] | UL config [#RB] |  | 1Tx RSD [dB] | 2Tx RSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5MHz | 25 | n5 | NA | -98.0 | 25 |  | 0.5 | 1.1 |
|  |  | n12 | NA | -97.0 | 20 |  | NA | NA |
|  |  | n29 | NA | -97.0 | NA |  | NA | NA |
|  |  | n85 | NA | -97.0 | 20 |  | 0.6 | 1.2 |
| 10MHz | 52 | n5 | -94.8 | -94.8 | 25 |  | 0.7 | 1.2 |
|  |  | n12 | -93.8 | -93.8 | 20 |  | NA | NA |
|  |  | n29 | -93.8 | -93.8 | NA |  | NA | NA |
|  |  | n85 | -93.8 | -93.8 | 20 |  | 1 | 1.4 |
| comment |  |  | No de-sense |  |  | <30 | Slight increase |  |
| 6MHz | 30 | n5 | NA | -97.2 | 25 |  | 0.6 | 1.1 |
|  |  | n12 | NA | -96.2 | 20 |  | NA | NA |
|  |  | n29 | NA | -96.2 | NA |  | NA | NA |
|  |  | n85 | NA | -96.2 | 20 |  | 0.8 | 1.3 |

Proposal on 6MHz NRB:

- NRB is 30RB for 15kHz SCS, NRB is not specified for higher SCS

- SU is 90% for both CP-OFDM and DFT-s-OFDM

- Minimum guard band is 292.5 kHz

- 6MHz REFSENS per band requirements are specified as proposed in the table below:

| Band | REFSENS | UL config | 1Tx RSD | 2Tx RSD |
| --- | --- | --- | --- | --- |
| n5 | -97.2dBm | 25RB | 0.6dB | 1.1dB |
| n12 | -96.2 dBm | 20RB | NA | NA |
| n29 | -96.2 dBm | NA | NA | NA |
| n85 | -96.2 dBm | 20RB | 0.8dB | 1.3dB |

## 2.3 A-MPR studies required for 6MHz

Since n5, n8, n12 and n85 are example bands and PC2 is specified for Bands n5 and n85:

- 6MHz PC2 A-MPR for NS_14, NS_15 should be evaluated for n5

- 6MHz PC3 A-MPR for NS_06 should be evaluated for n12

- 6MHz PC3 and PC2 A-MPR for NS_06 should be evaluated for n85.

Proposal on 6MHz A-MPR evaluation:

- 6MHz PC2 A-MPR for NS_14, NS_15 should be evaluated for n5

- 6MHz PC3 A-MPR for NS_06 should be evaluated for n12

- 6MHz PC3 and PC2 A-MPR for NS_06 should be evaluated for n85.

# Conclusion

In this contribution, we provide our preliminary views on 6MHz SU, REFSENS and A-MPR evaluations.

Proposal on 6MHz NRB:

- NRB is 30RB for 15kHz SCS, NRB is not specified for higher SCS

- SU is 90% for both CP-OFDM and DFT-s-OFDM

- Minimum guard band is 292.5 kHz

- 6MHz REFSENS per band requirements are specified as proposed in the table below:

| Band | REFSENS | UL config | 1Tx RSD | 2Tx RSD |
| --- | --- | --- | --- | --- |
| n5 | -97.2 | 25 | 0.6 | 1.1 |
| n12 | -96.2 | 20 | NA | NA |
| n29 | -96.2 | NA | NA | NA |
| n85 | -96.2 | 20 | 0.8 | 1.3 |

Proposal on 6MHz A-MPR evaluation:

- 6MHz PC2 A-MPR for NS_14, NS_15 should be evaluated for n5

- 6MHz PC3 A-MPR for NS_06 should be evaluated for n12

- 6MHz PC3 and PC2 A-MPR for NS_06 should be evaluated for n85.

# References

[1] RP-252901 New WID: UE RF enhancements for NR FR1, Phase 5, vivo, RAN#109