3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513825

Prague, Czech Republic, October 13th – October 17th, 2025

Source :	Iridium, Cambridge Consultants (CCL)

Title:	Demodulation performance requirements for IoT NTN TDD

Agenda item:	6.21.3

Document for:	Discussion

Introduction

In RAN#106 revised WID RP-243293 was approved [1] to introduce IoT NTN TDD mode.

The Objectives of the WID, as per [1]:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

At RAN#109, the core part of this WID was completed, and the following performance related items remain outstanding:

- RRM performance requirements, as needed [RAN4]

- Necessary SAN and UE demodulation requirements [RAN4]

- SAN conformance testing requirements [RAN4]

In this contribution, we present simulations results for the necessary SAN and UE demodulation performance requirements.

Discussion

NPRACH Demodulation Performance (SAN)

The demodulation performance requirements for NPRACH in SAN are based on the propagation conditions specified in the Annex of 36.108. The performance criterion for NPRACH is that the false detection must be less than or equal to 0.1%, for the specified SNR value.

For NB-IoT FDD mode, the NPRACH performance requirements from 36.108 are captured here for ease of reference.

Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 13.3 | 13.7 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 9.8 | 9.6 |
| 1 | 2 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 6.0 | 6.1 |
| 1 | 2 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 1.6 | 1.7 |

We have performed simulation runs using both the NTN-TDLA100-1 and NTN-TDLC5-1 channel models to determine the SNR required for a 99% successful detection rate. Rather than specifying performance requirements based on two receiver antennas, we propose including results with two distinct channel models, as the TDLC model was utilised in RAN1 during the study phase of for IoT NTN TDD.

RAN4 to consider updating the NPRACH demodulation performance requirements for IoT NTN TDD mode, by adding the Table 8.5.3.2.1-2 as follows:

Table 8.5.3.2.1-2: NPRACH missed detection requirements for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 5.8 | 6.0 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 1.5 | 2.1 |
| 1 | 1 | 8 | 8 | NTN-TDLC5-1 | 200 Hz | 2.2 | 2.4 |
| 1 | 1 | 16 | 16 | NTN-TDLC5-1 | 200 Hz | -0.5 | -0.3 |

NPUSCH Demodulation Performance (SAN)

NPUSCH format-1

The demodulation performance requirements for NPUSCH format-1 in SAN are defined using fixed reference channels and the propagation conditions, as specified in the Annex in 36.108. The minimum performance requirement for NPUSCH is achieving at least 70% throughput at a given SNR. This throughput includes MAC layer procedures such as HARQ retransmissions.

To manage simulation complexity within the available time constraints, certain simplifying assumptions were made. Notably, the NPDCCH configuration used for NPUSCH performance evaluation assumes an ideal NPDCCH channel. Based on this assumption, a NPUSCH BLER of 30% is expected to closely correspond to achieving 70% of the maximum throughput using fixed reference channel.

For NB-IoT FDD mode, the NPUSCH performance requirements from 36.108 are captured here for ease of reference.

Table 8.5.1.1.1-2: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -3.2 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -4.0 |
| 1 | 2 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.1 |
| 1 | 2 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | -7.8 |

For NB-IoT FDD performance evaluation, the FRC configuration A7-2 utilises a Transport Block Size (TBS) of 136 bits. In defining the NPUSCH REFSENSE requirement for IoT NTN TDD, we introduced a new NPUSCH FRC configuration, A14-3, which features a transmit duration of 8 ms and a TBS of 16 bits. Both FRC configurations are summarised here for ease of reference

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Table A.14-1 FRC parameters for reference sensitivity

| Reference channel | A14-1 for FDD operation | A14-2 for FDD operation | A14-3 for TDD operation |
| --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 |
| Number of tone | 1 | 1 | 1 |
| Diversity | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 |
| Number of NPUSCH repetition | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 | 16 |
| Allocated resource unit | 2 | 2 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 |
| Total symbols per resource unit | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |

We conducted simulations using both FRC configurations A7.2 and A14.3 with a 15 kHz subcarrier spacing to determine the SNR required to achieve a 30% BLER across various repetition values. The results, obtained using one transmit, and one receive antenna, are presented in Table 1 and Table 2. All simulations are performed with a frequency offset of 8 Hz.

| Propagation Model | Num of Subcarriers | Transmit Duration (ms) | Repetiton | SNR (dB) |
| --- | --- | --- | --- | --- |
| NTN TDLA100-1 | 1 | 8 | 1 | 1.8 |
| NTN TDLC5-1 | 1 | 8 | 1 | -1.2 |
| NTN TDLA100-1 | 1 | 16 | 2 | -1.1 |
| NTN TDLC5-1 | 1 | 16 | 2 | -4.1 |
| NTN TDLA100-1 | 1 | 32 | 4 | -4.3 |
| NTN TDLC5-1 | 1 | 32 | 4 | -6.7 |
| NTN TDLA100-1 | 12 | 1 | 1 | 11.2 |
| NTN TDLC5-1 | 12 | 1 | 1 | 7.3 |
| NTN TDLA100-1 | 12 | 2 | 2 | 7.8 |
| NTN TDLC5-1 | 12 | 2 | 2 | 5.2 |
| NTN TDLA100-1 | 12 | 4 | 4 | 5.7 |
| NTN TDLC5-1 | 12 | 4 | 4 | 2.4 |

Table 1 NPUSCH format-1 performance with A14-3 FRC, SCS=15 kHz, BLER= 30%

| Propagation Model | Num of Subcarriers | Transmit Duration (ms) | Repetiton | SNR (dB) |
| --- | --- | --- | --- | --- |
| NTN TDLA100-1 | 1 | 8 | 1 | 12.1 |
| NTN TDLC5-1 | 1 | 8 | 1 | 7.8 |
| NTN TDLA100-1 | 1 | 16 | 2 | 6.5 |
| NTN TDLC5-1 | 1 | 16 | 2 | 2.3 |
| NTN TDLA100-1 | 1 | 32 | 4 | 1.6 |
| NTN TDLC5-1 | 1 | 32 | 4 | 0.1 |
| NTN TDLA100-1 | 12 | 1 | 1 | 17.7 |
| NTN TDLC5-1 | 12 | 1 | 1 | 14.4 |
| NTN TDLA100-1 | 12 | 2 | 2 | 14.1 |
| NTN TDLC5-1 | 12 | 2 | 2 | 10.6 |
| NTN TDLA100-1 | 12 | 4 | 4 | 11.3 |
| NTN TDLC5-1 | 12 | 4 | 4 | 7.9 |
| NTN TDLA100-1 | 12 | 8 | 8 | 8.8 |
| NTN TDLC5-1 | 12 | 8 | 8 | 5.3 |
| NTN TDLA100-1 | 12 | 16 | 16 | 5.8 |
| NTN TDLC5-1 | 12 | 16 | 16 | 2.7 |

Table 2 NPUSCH format-1 performance with A7.2 FRC, SCS=15 kHz, BLER=30%

RAN4 to consider updating the NPUSCH format-1 demodulation performance for IoT NTN TDD mode, to include Table 8.5.1.1.1-3, based on one of the following three options:

Option 1 (A7-2):

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 5.8 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 2.7 |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A7-2 | 8 Hz | 32 | 4 | 70% | 1.6 |
| 1 | 1 | 15KHz | 1 | NTN-TDLC5-1 | A7-2 | 8 Hz | 32 | 4 | 70% | 0.1 |

Option 2 (A14.3):

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A14-3 | 8 Hz | 4 | 4 | 70% | 5.7 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A14-3 | 8 Hz | 4 | 4 | 70% | 2.4 |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -1.1 |
| 1 | 1 | 15KHz | 1 | NTN-TDLC5-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -4.1 |

Option 3 (A7.2 for 12 SC and A14.3 for 1SC):

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 5.8 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 2.7 |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -1.1 |
| 1 | 1 | 15KHz | 1 | NTN-TDLC5-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -4.1 |

NPUSCH format-2

The demodulation performance requirement for NPUSCH Format-2 is defined in terms of the probability of DTX being incorrectly detected as ACK, or a missed ACK detection. This probability must be equal to or less than 1%.

The missed ACK detection rate shall not exceed 1% at the specified SNR. For NB-IoT FDD, the relevant SNR values are specified in 36.108 and are reproduced here for ease of reference.

Table 8.5.2.2.1-2: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | 5.1 |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 32 | 16 | -3.3 |

For DTX ACK false detection, the existing requirement of the probability of false detection must be equal to or less than 1% can also be applied to IoT NTN TDD mode. This requirement applies across all SNR values.

For IoT NTN TDD mode, we conducted simulations of NPUSCH Format-2 transmissions using only ACK signalling to identify the SNR point at which the error rate does not exceed 1%. The results of these simulations are presented in Table 3.

| Propagation Model | Freq Offset (Hz) | Transmit Duration (ms) | Repetiton | SNR (dB) |
| --- | --- | --- | --- | --- |
| NTN TDLA100-1 | 1 | 8 | 4 | 5.8 |
| NTN TDLA100-1 | 1 | 16 | 8 | -2.1 |

Table 3 NPUSCH format-2 missed ACK detection with error <= 1%

The existing performance requirement for NPUSCH format-2 transmissions related to the probability of false ACK detection from DTX shall be equal to or less than 1% can also be applied to IoT NTN TDD mode.

RAN4 to consider updating the NPUSCH format-2 missed ACK detection performance for IoT NTN TDD mode, by adding Table 8.5.2.2.1-3, as follows:

Table 8.5.2.2.1-3: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 16 | 8 | -2.1 |

NPDSCH Demodulation Performance (UE)

The demodulation performance requirements for NPDSCH format-1 in UE are defined using fixed reference channels and the propagation conditions, as specified in the Annex in 36.102. The minimum performance requirement for NPDSCH is achieving at least 70% throughput at a given SNR. This throughput includes MAC layer procedures such as HARQ retransmissions.

To manage simulation complexity within the available time constraints, certain simplifying assumptions were made. Notably, the NPDCCH configuration used for NPDSCH performance evaluation assumes an ideal NPDCCH channel. Based on this assumption, a NPDSCH BLER of 30% is expected to closely correspond to achieving 70% of the maximum throughput using fixed reference channel.

The demodulation performance for UE in NB-IoT FDD mode is specified in 36.102 and copied here for ease of reference.

Table 8.3.1.1.1.1-2: Minimum performance for NPDSCH under Standalone Operations and In-band Operations with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

We have run simulations using the existing NPDSCH FRC and channel models to determine the SNR point that yields a maximum BLER of 30%. In addition to a maximum Doppler frequency of 100 Hz, we have also considered 160 Hz offset caused by oscillator error (0.1 ppm). The simulation results for two different repetition values are presented below.

| Carrier Type | Reference Channel | Propagation Model | Repetition | SNR (dB) |
| --- | --- | --- | --- | --- |
| Anchor | R.NB.1.FDD | NTN TDLC5-200 | 4 | -1.3 |
| Non-Anchor | R.NB.2.FDD | NTN-TDLA100-10 | 4 | -1.1 |
| Anchor | R.NB.1.FDD | NTN TDLC5-200 | 8 | -3.7 |
| Non-Anchor | R.NB.2.FDD | NTN-TDLA100-10 | 8 | -4.3 |

RAN4 to consider updating the NPDSCH performance requirement for IoT NTN TDD mode by adding Table 8.3.1.1.1.1-3, as below:

Table 8.3.1.1.1.1-3: Minimum performance for NPDSCH under Standalone Operations and In-band Operations with 1 NRS port for IoT NTN TDD

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 4 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -1.3 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 4 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -1.1 | NB1, NB2 |

Conclusion

In this contribution, we have presented our views on the UE and the SAN demodulation performance requirements. In this contribution, we have evaluated the last remaining RRM core requirement topics from RAN4#116 and provided our views. Our observations and proposals are listed below:

- Proposal 1: RAN4 to consider updating the NPRACH demodulation performance requirements for IoT NTN TDD mode, by adding the Table 8.5.3.2.1-2 as follows:

Table 8.5.3.2.1-2: NPRACH missed detection requirements for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 5.8 | 6.0 |
| 1 | 1 | 16 | 16 | NTN-TDLA100-1 | 200 Hz | 1.5 | 2.1 |
| 1 | 1 | 8 | 8 | NTN-TDLC5-1 | 200 Hz | 2.2 | 2.4 |
| 1 | 1 | 16 | 16 | NTN-TDLC5-1 | 200 Hz | -0.5 | -0.3 |

- Proposal 2: RAN4 to consider updating the NPUSCH format-1 demodulation performance for IoT NTN TDD mode, to include Table 8.5.1.1.1-3, based on one of the following three options:

Option 1 (A7-2):

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 5.8 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 2.7 |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A7-2 | 8 Hz | 32 | 4 | 70% | 1.6 |
| 1 | 1 | 15KHz | 1 | NTN-TDLC5-1 | A7-2 | 8 Hz | 32 | 4 | 70% | 0.1 |

Option 2 (A14.3):

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A14-3 | 8 Hz | 4 | 4 | 70% | 5.7 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A14-3 | 8 Hz | 4 | 4 | 70% | 2.4 |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -1.1 |
| 1 | 1 | 15KHz | 1 | NTN-TDLC5-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -4.1 |

Option 3 (A7.2 for 12 SC and A14.3 for 1SC):

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 5.8 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 8 Hz | 16 | 16 | 70% | 2.7 |
| 1 | 1 | 15KHz | 1 | NTN-TDLA100-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -1.1 |
| 1 | 1 | 15KHz | 1 | NTN-TDLC5-1 | A14-3 | 8 Hz | 16 | 2 | 70% | -4.1 |

- Proposal 3: The existing performance requirement for NPUSCH format-2 transmissions related to the probability of false ACK detection from DTX shall be equal to or less than 1% can also be applied to IoT NTN TDD mode.

- Proposal 4: RAN4 to consider updating the NPUSCH format-2 missed ACK detection performance for IoT NTN TDD mode, by adding Table 8.5.2.2.1-3, as follows:

Table 8.5.2.2.1-3: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx for IoT NTN TDD

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 Hz | 16 | 8 | -2.1 |

- Proposal 5: RAN4 to consider updating the NPDSCH performance requirement for IoT NTN TDD mode by adding Table 8.3.1.1.1.1-3, as below:

Table 8.3.1.1.1.1-3: Minimum performance for NPDSCH under Standalone Operations and In-band Operations with 1 NRS port for IoT NTN TDD

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 4 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -1.3 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 4 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -1.1 | NB1, NB2 |

- References

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

- RP-252329, “Status Report – WID Introduction of IoT NTN TDD mode”, Iridium Satellite LLC, RAN#109, Beijing, China, 15th – 18th Sep 2025

- RP-252407, “RAN4 CRs to IoT_NTN_TDD”, CR pack, RAN#109, Beijing, China, 15th – 18th Sep 2025

- RP-252935, “Revised WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #109, Beijing, China, 15th – 18th Sep 2025