3GPP RAN WG4 Meeting #116bis		R4-2513850

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	4.6.1.2

Source:	Apple

Title:	On maintenance for Rel-19 power domain enhancement

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

RAN4 completed the Rel-19 task on power domain enhancement by agreeing on the big CR [1] during the e-mail approval phase. There are three items we would like to discuss during the maintenance phase.

# 2  Discussion

##### 2.1 Maintenance on missing edge RB allocation for Intra-band contiguous CA

The Big CR [1] introduces the support of MPR enhancement for intra-band contiguous CA by introducing a new paragraph in clause 6.2A.2.1 which points to the single CC MPR tables.

### 6.2A.2 UE maximum output power reduction for CA #### 6.2A.2.1 UE maximum output power reduction for Intra-band contiguous CA For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.2A.1.1-1 with contiguous RB allocation is specified in Table 6.2A.2.1-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth class C when the signalling is indicated for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1b for power class 2 CA bandwidth classes B and C with TxD supported. For UE indicating fr1-MPRenhancement-r19 supported and if single CC is activated for intra-band contiguous CA, the allowed MPR is specified in Table 6.2.2-1 for PC3 CA bandwidth classes B and C, Table 6.2D.2-1 for PC2 CA bandwidth classes B and C when TxD is indicated, Table 6.2.2-2 for PC2 CA bandwidth classes B and C when TxD is absent.

While the single CC MPR is applicable the Inner/outer RB allocations are still calculated according to clause 6.2A.2.1. Since edge allocations do not exist for intra-band CA MPR the edge allocation is in fact not defined. One could argue that if the single CC MPR table refers to edge RB allocation, then the edge RB definition of the specific clause (either 6.2.2 or 6.2D.2) is applicable. But we would like to check understanding of companies. If there is different understanding, then this aspect should be clarified in TS 38.101-1.

The option of referencing to the single CC MPR tables is one approach it might also be considered to directly refer to the single CC clauses itself (either 6.2.2 or 6.2D.2)

Observation 1: For intra-band CA MPR scenarios with single CC UL the single CC MPR is specified by simply referencing the MPR tables. However, intra-band CA calculation for inner/outer still applies. Since intra-band CA does not specify edge RB allocations this raises the question whether edge RB definitions from specific clauses (6.2.2 or 6.2D.2) should be directly referenced or not.

Observation 2: The option of referencing to the single CC MPR tables is one approach it might also be considered to directly refer to the single CC clauses itself (either 6.2.2 or 6.2D.2)

Proposal 1: RAN4 should discuss whether applicability of edge RB allocations should be directly specified to avoid confusion and whether the approach of directly referencing to the requirements of clauses 6.2.2 and 6.2D.2 would be a better option.

##### 2.2 Single Carrier MPR enhancement and A-MPR

One result of RAN4 discussion was that any channel which is subject to A-MPR is not subject to single carrier MPR enhancement [2]. The reason for introducing this limitation has been explained in [3]. The Big CR [1] features the A-MPR aspect by stating “When no A-MPR is applicable” then the enhancement can be applicable. While this statement reflects RAN4 understanding it might be slightly confusing to the reader as it is not explicitly specified that it refers to the channel. Therefore, we propose to add the channel context as shown below:

If all of the following conditions are simultaneously met: - The UE is a PC3 or PC2 UE operating in any channel bandwidth except 3MHz - The UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19 - The uplink modulation order is either QPSK or 16QAM - When no A-MPR is applicable to the configured channel

Observation 3: One result of RAN4 discussion was that any channel which is subject to A-MPR is not subject to single carrier MPR enhancement . The reason for introducing this limitation has been explained in. The Big CR features the A-MPR aspect as by stating “When no A-MPR is applicable” then the enhancement can be applicable. While this statement reflects RAN4 understanding it might be slightly confusion to the reader as it is not explicitly specified that it refers to the channel.

Proposal 2: Add the channel context to the A-MPR condition.

##### 2.3 Single Carrier MPR enhancement and Rel-18 Power Boost

At the last RAN4 meeting a proposal was made to enable simultaneous use of Rel-19 single CC enhancement with the legacy Rel-18 power boost. In order to enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified. From our point of view the simultaneous use of Rel-19 and Rel-18 feature for single CC are fine if the UE indicates that it can support both features simultaneous.

Observation 4: To enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified to use the extended RBs.

Proposal 3: If RAN4 decides to enable simultaneous use of Rel-19 MPR enhancement and Rel-18 power boost feature then there needs to be an indication for the UE that it can support both features simultaneous.

# 4	Conclusions

This contribution discusses maintenance topics for single CC and intra-band CA. The following proposals and observations are made:

Observation 1: For intra-band CA MPR scenarios with single CC UL the single CC MPR is specified by simply referencing the MPR tables. However, intra-band CA calculation for inner/outer still applies. Since intra-band CA does not specify edge RB allocations this raises the question whether edge RB definitions from specific clauses (6.2.2 or 6.2D.2) should be directly referenced or not.

Observation 2: The option of referencing to the single CC MPR tables is one approach it might also be considered to directly refer to the single CC clauses itself (either 6.2.2 or 6.2D.2)

Proposal 1: RAN4 should discuss whether applicability of edge RB allocations should be directly specified to avoid confusion and whether the approach of directly referencing to the requirements of clauses 6.2.2 and 6.2D.2 would be a better option.

Observation 3: One result of RAN4 discussion was that any channel which is subject to A-MPR is not subject to single carrier MPR enhancement . The reason for introducing this limitation has been explained in. The Big CR features the A-MPR aspect as by stating “When no A-MPR is applicable” then the enhancement can be applicable. While this statement reflects RAN4 understanding it might be slightly confusion to the reader as it is not explicitly specified that it refers to the channel.

Proposal 2: Add the channel context to the A-MPR condition.

If all of the following conditions are simultaneously met: - The UE is a PC3 or PC2 UE operating in any channel bandwidth except 3MHz - The UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19 - The uplink modulation order is either QPSK or 16QAM - When no A-MPR is applicable to the configured channel

Observation 4: To enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified to use the extended RBs.

Proposal 3: If RAN4 decides to enable simultaneous use of Rel-19 MPR enhancement and Rel-18 power boost feature then there needs to be an indication for the UE that it can support both features simultaneous.

# 4	References

- R4-2511761, “Draft big CR to TS 38.101-1 on MPR reduction”, Huawei, 3GPP TSG-RAN WG4 Meeting #116

- R4-2505099, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114bis

- R4-2503189, “On Rel-19 power domain enhancement for FR1”, Apple, 3GPP TSG-RAN WG4 Meeting #114bis