3GPP RAN WG4 Meeting #116bis		R4-2513851

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	4.6.1.1

Source:	Apple

Title:	On HPUE band combinations handling after HPUE MSD LUT introduction

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

HPUE MSD look-up tables (LUTs) have been introduced in Rel-19 with the intention to ease the new HPUE CA/DC combination introducing process and simplify the RAN4 specifications. With HPUE MSD LUTs, the existing HPUE (P2 and PC1.5) applicability notes in band combination configuration tables in clause 5 of RAN4 UE RF specifications [1,2] in principle are no longer needed which have been known linked to several issues on HPUE support for band combinations [3] and also led to a RAN task to mitigate part of the issues. In addition, RAN4 should start moving away from the legacy HPUE basket approach after the HPUE MSD LUTs are introduced as the existing process is rather inefficient. In last RAN4 meeting, with the anticipation of the HPUE MSD LUTs readiness, a draft CR to 38.101-1 to streamline the procedure of HPUE introduction was proposed [4] which however was postponed as RAN4 seemed not ready to move away from the existing HPUE band combination introduction process (at least in Rel-19) despite the HPUE LUTs are already developed. It was also agreed in the approved WF [5] that all existing HPUE baskets are recommended extended to December if there are combinations left to be completed in the WID Excel and proponents shall contribute via draftCR for the cases covered by MSD LUTs. However, this process in our view is essentially redundant since the MSD can already be referred to the LUTs and there is no need to further add them to the legacy MSD tables. On the other hand, the HPUE MSD LUTs are supposed to resolve the issue on disenabled HPUE support for band combinations without the HPUE applicability notes in clause 5 which is still left open even after the LUT introduction.

In this contribution, we sharing our views on the HPUE band combinations handling after HPUE MSD LUT introduction and propose a solution in Rel-19 specifications to resolve the issue on disenabled HPUE support for band combinations without the HPUE applicability notes in clause 5.

# Discussion

One of the main reasons the PC2 and PC1.5 applicability notes in band combination configuration tables of the RAN4 UE RF specifications [1,2] are necessitated is to ensure the corresponding MSD requirements are specified, in particular, for the impacted cross DL bands without UL configured. On the other hand, despite many band combinations are not subject to MSD issue, to maintain the specifications consistency on indicating HPUE support for band combinations, the PC2 or PC1.5 applicability notes still need to be added to those band combinations via formal CR process. While RAN4 may have seemed accustomed to the process of adding the PC2 and PC1.5 applicability notes and MSD requirements for the band combinations intended to support HPUE, some deficiency was also identified in the current HPUE introduction process which has been alluded in [3] and also led to a RAN task to mitigate part of the issues which is for DL CA combinations with single UL configured only. To resolve all the issues due to the lack of HPUE MSD requirements hindering the HPUE band combinations support, RAN4 made a great effort in Rel-19 to introduce HPUE MSD LUTs which have been completed in last RAN4 meeting [6].

Observation 1: One of the main reasons the PC2 and PC1.5 applicability notes in band combination configuration tables of the RAN4 UE RF specifications are necessitated is to ensure the corresponding MSD requirements are specified.

Observation 2: RAN task on HPUE support for band combinations without HPUE applicability notes only mitigated the issue for DL CA combinations with single UL configured.

Observation 3: The introduction of HPUE MSD LUTs is intended to resolve all the issues due to the lack of HPUE MSD requirements which has been hindering the HPUE support for band combinations without the HPUE applicability notes in clause 5.

With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications [1,2] in principle are no longer needed. In addition, RAN4 should start moving away from the legacy HPUE basket approach after the HPUE MSD LUTs are introduced as the existing process is rather inefficient. In last RAN4 meeting, with the anticipation of the HPUE MSD LUTs readiness, a draft CR to 38.101-1 to streamline the procedure of HPUE introduction was proposed [4] which however was postponed as RAN4 seemed not ready to move away from the existing HPUE band combination introduction process (at least in Rel-19) despite the HPUE LUTs are already developed. It was also agreed in the approved WF [5] that all existing HPUE baskets are recommended extended to December if there are combinations left to be completed in the WID Excel and proponents shall contribute via draftCR for the cases covered by MSD LUTs. However, this process in our view is essentially redundant since the MSD can already be referred to the LUTs and there is no need to further add them to the legacy MSD tables. On the other hand, the HPUE MSD LUTs are supposed to resolve the disenabled HPUE support issue for band combinations without the HPUE applicability notes in clause 5 which is still left open even after the LUTs introduction.

Observation 4: With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications in principle are no longer needed.

Observation 5: RAN4 seemed not ready to move away from the existing HPUE band combination introduction process (at least in Rel-19) despite the HPUE LUTs are already developed.

Observation 6: With HPUE MSD LUTs specified, the current HPUE band combination introduction process is essentially redundant since the MSD can already be referred to the LUTs and there is no need to further add them to the legacy MSD tables.

Observation 7: The HPUE MSD LUTs are supposed to resolve the disenabled HPUE support issue for band combinations without the HPUE applicability notes in clause 5 which is still left open even after the LUTs introduction.

Based on above observations and assessment, we would like to make the following proposals on HPUE band combinations handling for Rel-19 and Rel-20 to resolve the aforementioned issue on disenabled HPUE support for band combinations without the HPUE applicability notes in clause 5 of the RAN4 UE RF specifications and simplify the HPUE band combinations introducing process going forward.

Proposal 1a: Add two general notes below Table 5.5A.3.1-1o (last CA configuration table for inter-band CA with 2 bands):

NOTE 17: For single uplink carrier with up to 2Tx antenna connectors without NOTE 8, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2.1-1 or Table 6.2D.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination.

NOTE 18: For single uplink carrier with up to 2Tx antenna connectors without NOTE 9, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination.

Proposal 1b: Remove the texts in clause 7.3A.1 introduced via RAN task for CA combinations with single uplink as they are no longer needed after the HPUE MSD LUT introduction.

Proposal 2: Add two general notes below Table 5.5A.3.2-1c (last CA configuration table for inter-band CA with 3 bands):

NOTE 15: For a 2-band UL configuration without NOTE 7 or NOTE 13, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2A.1.3-1 or Table 6.2H.3.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination.

NOTE 16: For a 2-band UL configuration without NOTE 14, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2H.3.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination.

For Proposal 1a/1b and Proposal 2, we also prepare a companion draft CR [7] in this meeting for consideration.

Proposal 3: For the extended Rel-19 HPUE basket WIDs, proponents are encouraged to only introduce new 2UL/2DL combinations and not to capture MSD requirements in legacy MSD tables if they are covered by the HPUE MSD LUTs nor to add HPUE applicability notes in clause 5 to reduce the RAN4 workloads.

Proposal 4: Remove HPUE applicability notes in band combination configuration tables in clause 5 in Rel-20 RAN4 specifications.

Proposal 5: Do not continue HPUE basket WIDs for band combinations in Rel-20. The 2-band HPUE support can be requested in generic 2UL/2DL basket WIDs.

# 3	Conclusion

In this contribution, we sharing our views on the HPUE band combinations handling after HPUE MSD LUT introduction and propose a solution in Rel-19 specifications to resolve the issue on disenabled HPUE support for band combinations without the HPUE applicability notes in clause 5.

Observation 1: One of the main reasons the PC2 and PC1.5 applicability notes in band combination configuration tables of the RAN4 UE RF specifications are necessitated is to ensure the corresponding MSD requirements are specified.

Observation 2: RAN task on HPUE support for band combinations without HPUE applicability notes only mitigated the issue for DL CA combinations with single UL configured.

Observation 3: The introduction of HPUE MSD LUTs is intended to resolve all the issues due to the lack of HPUE MSD requirements which has been hindering the HPUE support for band combinations without the HPUE applicability notes in clause 5.

Observation 4: With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications in principle are no longer needed.

Observation 5: RAN4 seemed not ready to move away from the existing HPUE band combination introduction process (at least in Rel-19) despite the HPUE LUTs are already developed.

Observation 6: With HPUE MSD LUTs specified, the current HPUE band combination introduction process is essentially redundant since the MSD can already be referred to the LUTs and there is no need to further add them to the legacy MSD tables.

Observation 7: The HPUE MSD LUTs are supposed to resolve the disenabled HPUE support issue for band combinations without the HPUE applicability notes in clause 5 which is still left open even after the LUTs introduction.

Proposal 1a: Add two general notes below Table 5.5A.3.1-1o (last CA configuration table for inter-band CA with 2 bands):

NOTE 17: For single uplink carrier with up to 2Tx antenna connectors without NOTE 8, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2.1-1 or Table 6.2D.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination.

NOTE 18: For single uplink carrier with up to 2Tx antenna connectors without NOTE 9, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination.

Proposal 1b: Remove the texts in clause 7.3A.1 introduced via RAN task for CA combinations with single uplink as they are no longer needed after the HPUE MSD LUT introduction.

Proposal 2: Add two general notes below Table 5.5A.3.2-1c (last CA configuration table for inter-band CA with 3 bands):

NOTE 15: For a 2-band UL configuration without NOTE 7 or NOTE 13, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2A.1.3-1 or Table 6.2H.3.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination.

NOTE 16: For a 2-band UL configuration without NOTE 14, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2H.3.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination.

Proposal 3: For the extended Rel-19 HPUE basket WIDs, proponents are encouraged to only introduce new 2UL/2DL combinations and not to capture MSD requirements in legacy MSD tables if they are covered by the HPUE MSD LUTs nor to add HPUE applicability notes in clause 5 to reduce the RAN4 workloads.

Proposal 4: Remove HPUE applicability notes in band combination configuration tables in clause 5 in Rel-20 RAN4 specifications.

Proposal 5: Do not continue HPUE basket WIDs for band combinations in Rel-20. The 2-band HPUE support can be requested in generic 2UL/2DL basket WIDs.

# 4	References

3GPP TS 38.101-1 V19.3.0 (2025-09)

3GPP TS 38.101-3 V19.3.0 (2025-09)

R4-2400191 “On PC2 and PC1.5 indications in BC configuration tables”, Apple, 3GPP RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024

R4-2510045 “Draft CR for 38.101-1 to streamline the procedure of HPUE introduction”, Samsung, 3GPP RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

R4-2511803 “WF for the extended Rel-19 band combination baskets”, Nokia, 3GPP RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

R4-2511752 “Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification”, Skyworks Solutions Inc., et al, 3GPP RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

R4-2513852 “Draft CR for 38.101-1 on enabling HPUE support for band combinations without HPUE applicability notes in clause 5”, Apple, 3GPP RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13th – 17th, 2025