3GPP TSG-RAN WG4 Meeting # 116Bis		R4-2513854

Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	4.8.1

Source:	Apple

Title:	On RMCs definition for Rel-19 low NR band CA via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #116, Bengaluru, India, August 25 – 29, 2025), discussions were held on several UE RF aspects of low NR band carrier aggregation via switching (NR_LBCA_Sw) work item. As captured in the approved topic WF [1], the main remaining aspect to be further discussed is the definition of reference measurement channels (RMCs) for UE performance testing.

- For downlink (DL) RMCs, 4 options were considered based on the submitted contributions.

- For uplink (UL) performance testing, it was agreed to further discuss if UL RMCs are needed.

Highlighted below is the summary of the meeting WF on the RMCs definition:

Issue 1: Downlink reference measurement channels

Agreement:

-	RAN4 can continue to discuss how to specify the DL RMC for this feature as part of the Performance phase of the work item

-	The following options for the RMC have been proposed:

-	Option 1: 10ms periodicity with switching period placed in slots 4 and 8

-    Option 2: Increase period of RMC to 40 slots and use the LBCA-Switching Pattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’

-	Option 3: Bitmap (0000000000, 1111111111, 0000000000, 1111111111)

-	Option 4: other options are not precluded

Issue 2: Uplink reference measurement channels

Agreement:

-	RAN4 can further discuss whether an UL RMC is needed for this feature

In this discussion paper, we will share our views on some of the issues highlighted above.

# Discussion

  - RMCs for REFSENS performance measurements

In Rel-19, low NR band aggregation via switching was introduced to address challenges that exist for LB-LB inter-band carrier aggregation when traditional aggregation techniques (single antenna with multiplexers) are used. Only CA combinations involving an SDL band and a FDD band were considered. RAN4 has recently completed the definition of the core UE RF requirements. For the UE RF performance evaluation phase, the introduction of new DL RMCs was considered in the previous meeting based on contributions from three companies. No agreement was reached on the DL RMC to be used, and the debate was postponed to the upcoming meetings.

    - Option 2

To verify the effects of the switching period on the UE REFSENS for low NR band aggregation via switching feature, one of the DL RMC options considered in the previous meeting was  based on having a 40-slot (4 frames) RMC  periodicity and using the LBCA-switching pattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’ [2]. A total of 18 switching events occur during that 4-frame periodicity.

DL measurement channels are defined in clause A.3 of the TS38.101-1 3GPP specifications. As can be seen in clause A.3.2 (DL measurement channels for FDD), the RMCs for single carrier and for carrier aggregation are based on 1 frame (10 slots).

Observation #1: DL measurement channels for both single-carrier and carrier aggregation as defined in clause A.3.2 are based on one NR frame (10 slots).

For consistency and to easily compare the effects of switching on the UE REFSENS performance, we recommend using 1 NR frame (10 slots) to define DL RMCs for low NR ban aggregation via switching.

Proposal #1: For consistency with the current specifications and to easily compared the UE REFSENS performance with and without low NR band aggregation via switching, we propose using 1 frame (10 slots) as the periodicity of the DL RMCs.

    - Option 3

Option 3 was based on a proposal defining DL RMC using a 40ms periodicity with bitmap (switching pattern: 0000000000, 1111111111, 0000000000, 111111111) [3]. The number of switches were undefined. For HRAQ-ACK timing, i.e., from the end of PDSCH to the beginning of HARQ-ACK, the fallback DCI assumes maximum 8 slots (as captured in clause 9.2.3 of TS38.213). So, if UE stays in SDL for too long, it may not be able to receive fallback DCI scheduled PDSCH at the beginning of SDL due to fact that HARQ-ACK cannot be scheduled.

Observation #2: With the switching pattern “0000000000, 1111111111, 0000000000, 111111111” as proposed in option 2, the UE may not be able to receive fallback DCI scheduled PDSCH at the beginning of SDL due to fact that HARQ-ACK cannot be scheduled (Fallback DCI assumes a maximum of 8 slots as captured in clause 9.2.3 of TS38.213).

    - Option 1

Based on the observations above, we propose using Option 1 as a candidate for DL RMCs definition [4]. It is based on 1 frame (10 slots) with two switching events occurring in one frame.

Proposal #2: We prefer  using Option 1 which is based on 10ms (10 slots) periodicity as a candidate for DL RMC for  low NR band aggregation via switching.

DL RMC for evaluating the UE DL performance for low NR band aggregation via switching:

Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 3 | 5, 10, 15, 20 (Note 5) | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| For Slot 4 | Bits | 504 | 1064 | 2600 | 4064 | 5576 | 7088 | 8648 | 11784 | 14808 |
| Transport block CRC | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| For Slot 4 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,5,6,7,8 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,9 | Bits | 3240 | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| For Slot 4 | Bits | 2520 | 4200 | 8736 | 13272 | 17808 | 22344 | 26880 | 36288 | 45360 |
| Max. Throughput averaged over 1 frame | Mbps | 0.3456 | 0.6080 | 1.2704 | 1.9424 | 2.6312 | 3.3224 | 3.8600 | 5.4048 | 6.7080 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |  |

Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, SDL, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 3 | 5, 10, 15, 20 (Note 5) | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3,4,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 5,6,7 | Bits | 984 | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| For Slot 8 |  | 504 | 1064 | 2600 | 4064 | 5576 | 7088 | 8648 | 11784 | 14808 |
| Transport block CRC | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3,4,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 5,6,7 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| For Slot 8 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3,4,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 5,6,7 | Bits | 3240 | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| For Slots 8 | Bits | 2520 | 4200 | 8736 | 13272 | 17808 | 22344 | 26880 | 36288 | 45360 |
| Max. Throughput averaged over 1 frame | Mbps | 0.3456 | 0.6080 | 1.2704 | 1.9424 | 2.6312 | 3.3224 | 3.8600 | 5.4048 | 6.7080 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |  |

  - RMCs for UL performance measurements

For issue 2 of the previous meeting WF, it was agreed to further discuss if UL RMC is needed for low NR band aggregation via switching. Testing UE ON/OFF UL performance when it is subjected to switching is similar to testing the ON/OFF performance for the UL Tx switching feature.

Observation #3: Testing UE ON/OFF UL performance when the UE is subjected to switching is similar to testing the ON/OFF performance for the UL Tx switching feature.

However, there is no UL RMC dedicated specifically for UL TX switching. As captured in Clause 6.3A.3.1 of the TS38.521 shown below, the UL RMC used for UL Tx switching are as specified in Annexes A.2 and A.3. These are the same RMCs used for all the other UL tests. Therefore, we do not need new UL RMC to test the UL performance for low NR band aggregation via switching.

Proposal #3: New UL RMCs are not needed to evaluate the UE UL performance for low NR band aggregation via switching. The existing RMCs defined in Annexes A.2 and A.3 can be used to test the UL performance, just as it is the case for evaluating UL Tx switching.

Transmit ON/OFF time mask testing procedure for UL Tx switching

# Conclusions

Observation #1: DL measurement channels for both single-carrier and carrier aggregation as defined in clause A.3.2 are based on one NR frame (10 slots).

For consistency and to easily compare the effects of switching on the UE REFSENS performance, we recommend using 1 NR frame (10 slots) to define DL RMCs for low NR ban aggregation via switching.

Proposal #1: For consistency with the current specifications and to easily compared the UE REFSENS performance with and without LBCA via switching, we propose using 1 NR frame (10 slots) to define DL RMCs for low NR ban aggregation via switching.

Observation #2: With the switching pattern “0000000000, 1111111111, 0000000000, 111111111” as proposed in option 2, the UE may not be able to receive fallback DCI scheduled PDSCH at the beginning of SDL due to fact that HARQ-ACK cannot be scheduled (Fallback DCI assumes a maximum of 8 slots as captured in clause 9.2.3 of TS38.213).

Proposal #2: We prefer  using Option 1 which is based on 10ms (10 slots) periodicity as a candidate for DL RMC for  for low NR band aggregation via switching.

Observation #3: Testing UE ON/OFF UL performance when the UE is subjected to switching is similar to testing the ON/OFF performance for the UL Tx switching feature.

Proposal #3: New UL RMCs are not needed to evaluate the UE UL performance for low NR band aggregation via switching. The existing RMCs defined in Annexes A.2 and A.3 can be used to test the UL performance, just as it is the case for evaluating UL Tx switching

# 4	References

R4-2511862, “WF on UE RF requirements for LB CA via switching”, by Apple (topic moderator),  3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, August 25-29, 2025

R4-2509246, “Discussion on LB-LB CA via switching”, by CATT, 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, August 25-29, 2025

R4-2509821, “View on LB CA with switching”, by Xiaomi, 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, August 25-29, 2025

R4-2509965, “TP to TR38.768 on low NR band aggregation via switching”, by Apple, 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, August 25-29, 2025