3GPP TSG-RAN WG4 Meeting #116bis							R4-2513864

Prague, Czech Republic, 13th to 18th October 2025

Agenda Item:	7.7.2

Source:	Ericsson

Title:	On General aspects and DL co-existence considerations for UAV

Document for:	Discussion

# 1	Introduction

During RAN#109, a RAN4 WI [1] was agreed on Enhanced NR requirements for UAV operations to address issues related to UL throughput and UL coverage with higher transmit power. The UAVs will be served by ground based UAV BSs by adjusting the tilt angle upwards. It has been highlighted in the WID to take reference from NR ATG study [2], which supported two ATG UE antenna types – omni-directional antenna and antenna array. The objectives for core part from the WID [1], has been highlighted below –

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

This contribution highlights how UAV differs from ATG and the need to also consider downlink co-existence scenarios.

# 2	Discussion

## 2.1	UAV vs ATG

The UAV and ATG systems fundamentally differ in deployment assumptions, coverage geometry and antenna assumptions.

The UAV networks will target low-altitude UEs (up to 0.6 km ~1 km), with ISDs ~2-4 km and cell ranges which are constrained by vertical beam patterns, leading to interference limited operation as the UAVs maintain strong LoS to multiple TN/ UAV BSs simultaneously.  In contrast, the ATG systems are dimensioned at altitudes of 3-10 km with ISDs up to 100 km. It is therefore path-loss limited rather than interference limited, and the key challenge is to close the link budget over long ranges since the ATG UE is typically mounted externally to avoid fuselage penetration losses.

UAV networks are interference limited as compared to ATG systems which are path-loss limited, and the key challenge is to close the link budget over long ranges.

The application scenarios stated in the WID include logistics, travel, tourism, control and supervision, medical rescue etc. Further clarification is needed whether altitude stated for support up to 0.6 km ~1 km is a reasonable range considering the targeted application scenarios.

RAN4 should clarify further with the assumption of coverage height - up to 0.6 km ~1 km

For the ATG network, a single ATG BS with a single cell was assumed. The aircraft were assumed to fly in a straight line from the minimum distance to the maximum horizontal distance from the ATG BS in the horizontal boresight of the ATG sector. It is not clear from WID what shall be the assumption of number of UAV UEs per cell and what shall be the route of UAV UEs – whether they follow specific route same as ATG UEs.

UAV system performance also depends on UE density, while ATG systems are more sensitive to aircraft trajectory and long distance link closure.

For UAV scenarios, there is also a dependency on the assumed UE antenna characteristics. If the UAV is equipped with an antenna array capable of beamforming or directional gain, interference towards terrestrial systems can be reduced, leading to less severe coexistence issues. However, to ensure robustness in coexistence evaluations, it is reasonable to consider a worst-case assumption where the UAV UE employs an omnidirectional antenna, since this maximizes the potential interference in all directions and removes any reliance on specific antenna implementation.

If the UAV UE has directional/beamforming antennas, it can reduce interference to other networks. However, for robustness, coexistence evaluations should assume an omnidirectional UE antenna as a worst case.

It is worth noting that an omnidirectional antenna on a UAV would actually be a downward sphere, as the UAV BSs are on the ground.

## 2.2	Downlink co-existence considerations

In the Rel-15 Study on Enhanced LTE support for Aerial Vehicles documented in TR 36.777 [3], the deployment assumptions were considering both aerial UEs and terrestrial UEs in a single network with IoT (Interference over Thermal) as a performance metric among others. It was observed from the results that aerial UEs will receive more interference in the DL as the aerial UEs experience LoS conditions to multiple cells. It should be noted that the assumptions for this study can be taken as a good starting point, however all may not be relevant considering the current study.

The co-existence scenarios concerning this study shall assume two separate networks. It is not clear from the WID whether downlink scenarios will be studied for co-existence evaluations. The UL scenarios have been discussed in our companion paper [4]. However, to ensure robust performance for the whole UAV system, it is important to analyse the performance impact on both DL and UL cases.

### 2.2.1	TN downlink to UAV downlink co-existence

This case looks at interference from Terrestrial BS DL transmissions into UAV network DL receivers. Here, a UAV UE receives its DL signal from a UAV BS, but at the same time is exposed to strong interfering signals from TN BSs transmitting on the same or adjacent channels. Unlike TN UEs (which are shielded by clutter, buildings and terrains), UAVs at altitudes will have LoS to multiple TN BSs simultaneously even at altitudes up to 1 Km. UAV UEs may accumulate interference from many non-serving TN BSs. It may be that the most problematic case need not be the TN BSs that are directly under the UAV UEs as the antenna is pointing downwards. However, considering LoS conditions TN BSs which are far away at the horizon from UAV point of view could contribute to accumulated interference.

In DL case when UAV UEs are victim, UAV UEs may accumulate interference due to LoS to multiple TN BSs simultaneously even at altitudes up to 1 Km.

Considering LoS conditions TN BSs which are far away at the horizon from UAV point of view could contribute to accumulated interference.

In the Rel-18 ATG study [2], Ericsson provided additional simulation results to study the impact of ATG UE height distribution on the TN DL. The common assumption to derive the ATG UE ACS was with uniform ATG UE distribution between 3-10 km altitude. However, from the additional results it was observed that the ATG UE suffers higher degradation than the acceptable 5% during take-off and landing i.e. at 3 km as compared to two other cases of uniform distribution and fixed 10 km. It was not concerning as aircrafts are usually at altitudes close to 10km or higher when cruising.

It was observed that ATG UEs suffered higher degradation than acceptable 5% at fixed 3 kms height than in uniform distribution or fixed 10 kms height.

In order to make the UAV system robust, it is essential to study the impact of TN networks on the UAV networks as UAV UEs are more exposed to aggregate interference due to LoS to multiple TN BSs at relatively low altitudes as well as when TN BS are at horizon from UAV point of view.

Consider the TN DL to UAV DL co-existence analysis, where UAV UEs are victims due to aggregated interference from multiple TN BSs in co-existence evaluations when TN BSs are underneath UAV UE and TN BSs are far away at horizon from UAV point of view.

### 2.2.2	UAV downlink to TN downlink co-existence

This case looks at interference from UAV BS DL transmissions into TN network DL receivers. Here, TN UEs are intended to receive DL signals from their serving TN BSs, but they may be exposed to interfering UAV BS transmissions operating on the same or adjacent channels. It is reasonable to assume that the interference would be less severe as compared to TN BSs, as the UAV BSs antenna will be pointing upwards.

For the UAV DL to TN DL co-existence analysis, the interference from the UAV BSs to the TN UEs would be less severe as compared to TN BSs, as the UAV BSs antenna will be pointing upwards.

# Conclusion

In the previous sections we made the following observations:

Observation 1	UAV networks are interference limited as compared to ATG systems which are path-loss limited, and the key challenge is to close the link budget over long ranges.

Observation 2	RAN4 should clarify further with the assumption of coverage height - up to 0.6 km ~1 km

Observation 3	UAV system performance also depends on UE density, while ATG systems are more sensitive to aircraft trajectory and long distance link closure.

Observation 4	If the UAV UE has directional/beamforming antennas, it can reduce interference to other networks. However, for robustness, coexistence evaluations should assume an omnidirectional UE antenna as a worst case.

Observation 5	It is worth noting that an omnidirectional antenna on a UAV would actually be a downward sphere, as the UAV BSs are on the ground.

Observation 6	In DL case when UAV UEs are victim, UAV UEs may accumulate interference due to LoS to multiple TN BSs simultaneously even at altitudes up to 1 Km.

Observation 7	Considering LoS conditions TN BSs which are far away at the horizon from UAV point of view could contribute to accumulated interference.

Observation 8	It was observed that ATG UEs suffered higher degradation than acceptable 5% at fixed 3 kms height than in uniform distribution or fixed 10 kms height.

Observation 9	For the UAV DL to TN DL co-existence analysis, the interference from the UAV BSs to the TN UEs would be less severe as compared to TN BSs, as the UAV BSs antenna will be pointing upwards.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider the TN DL to UAV DL co-existence analysis, where UAV UEs are victims due to aggregated interference from multiple TN BSs in co-existence evaluations when TN BSs are underneath UAV UE and TN BSs are far away at horizon from UAV point of view.

# References

[1]	RP-252908, “Enhancement of NR RF and RRM requirements for UAV”, 3GPP RAN#109, September 2025

[2]	3GPP TR 38.876 v18.2.0, “Air-to-ground network for NR”, July 2024

[3]	3GPP TR 26.777 v15.0.0, “Study on Enhanced LTE support for Aerial Vehicles”, December 2017

[4]	R4-2513865, “On Uplink co-existence considerations for UAV”, RAN4#116bis, Ericsson