3GPP TSG RAN WG4 Meeting #116-bis	R4-2513909

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			6.19.3.2

Source:	vivo

Title:	Discussion on RRM performance requirements of DL coverage

Document for:	Discussion

- Introduction

In the previous meeting, companies provided preliminary analysis on performance requirements and RRM test case design for NTN in NR Phase 3, which has been captured in the latest WF [1]. In this paper, we will present our further analysis RRM performance requirements of DL coverage.

- Discussion

For downlink coverage enhancements, the discussion on the core part primarily focused on analyzing the potential impact of SSB period extension on RRM requirements.

For the case of SSB Ês/Iot < -8dB and an unknown NR cell in the following RRC re-establishment requirement, if the SMTC is 160ms, the corresponding delay would be quite long (e.g., Tidentify_inter_NR = 22 x 160 = 3520ms when GEO is considered). In such cases, the re-establishment timer may have already expired if the timer duration is configured to be relatively short.

Table 6.2C.1.2.1-2a: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) and SMTC configuration | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 and SMTC is configured | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 or SMTC is not configured | FR1 | N/A | k * MAX(800 ms, 22 x Tsearch,i) Note2 |
| NOTE 1: K_satellite is defined in clause 9.3C.4. NOTE 2: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

In real-world scenarios, it is not quite typical for Ês/Iot to degrade below -8dB while SMTC is not configured, requiring several seconds for cell search for re-establishment instead of directly entering idle mode to perform cell selection. Therefore, we prefer not to define related test cases for this scenario.

Proposal 1: RAN4 not define test case for the case of SSB Ês/Iot < -8dB and an unknown NR cell defined in RRC Re-establishment

- Summary

Proposal 1: RAN4 not define test case for the case of SSB Ês/Iot < -8dB and an unknown NR cell defined in RRC Re-establishment

- References

[1] R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT