TSG-RAN Working Group 4 (Radio) meeting #116-bis	R4-2513944

Prague, EU, 13th to 17th October 2025

Source:	Ericsson

Title:	Draft LS to ECC PT on parameters for AAS BS operating in bands below 1 GHz

Agenda item:	10.2

Document for:	Approval

# Introduction

RAN4 have received an LS from ECC PT1 with specific technical questions related to operation and modelling AAS BS below 1 GHz.

In the past ECC PT1 has conducted compatibility studies considering AAS BS operating in the 1800 MHz band and other mobile bands at higher frequencies. The characteristics of AAS BSs in those bands are well known. For operation below 1 GHz ECC PT1 currently does not have information of technical characteristics for AAS BS.

The current versions of TS 37.105 and TS 38.104 contain BS RF requirements for AAS BS allowing for operation below 1 GHz. With this in mind ECC PT1 has identified following questions:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band.

In this contribution we provide technical background relevant for AAS BS operation below 1 GHz draft LS response. At the end of the contribution a draft LS is attached to stimulate further discussion on how to proceed.

# Discussion

In this part of the contribution, we will discuss the status in RAN4 with respect to the questions provided by ECC PT1.

  - Question 1: Unwanted emission characteristics

Requirement levels for unwanted emissions for AAS BS operating within FR1 is defined in the MSR specification TS 37.105, clause 6.6 clause 9.7 and the NR specification TS 38.104, clause 6.6 and clause 9.7.

Unwanted emission including spectral and spatial characteristics for AAS BS operating in 700 MHz, 800 MHz and 900 MHz bands is not available in RAN4. Currently, information related to AAS operation is known for bands above 1 GHz since AAS BS have been deployed in networks for many years. For AAS BS above 1 GHz relevant simulations models are established. To collect characteristics a measurement campaign needs to be initiated. Currently RAN4 has no plans to conduct such measurement campaign.

Observation 1: The RAN4 AAS BS requirements are applicable for all bands. However, RAN4 currently does not have recent information regarding relevant spatial unwanted emission for AAS BS operating below 1 GHz. If needed and AAS sub-GHz products are deployed, RAN4 needs to initiate a study to collect measurement data for unwanted emission relevant for AAS BS operating below 1 GHz.

  - Question 2: Requirement applicability

The current requirement levels for unwanted emission were originally derived for non-AAS operating around 2 GHz as part of UTRA technical background. In addition, requirement for AAS BS was defined in Rel-13/15 by moving requirements to the OTA domain maintaining equivalent protection with non-AAS requirement levels.

When new bands are introduced RAN4 typically evaluate adjacent channel co-existence and define proper requirement levels for (ACLR and ACS for both BS and UE).

Based on request RAN4 also is conducting re-evaluation of other requirement levels, e.g. transmitter spurious emission for band n104 in Rel-19.

It is worth to point out that the background assumptions used for UTRA may differ in terms of BS antenna size and deployment parameters (ISD, grid-shift, etc.).

Considering AAS BS operating below 1 GHz it would be reasonable to initiate a re-evaluation short study to check requirement levels for BS AAS ACLR and ACS and transmitter spurious emissions based on relevant parameters assumptions. Currently, such activities are not planned in Rel-19 and Rel-20 timeframe. One approach could be to consider including it in the scope of the RAN4 BS RF 6G study.

Observation 2: Requirement levels for unwanted emissions in TS 38.104 and TS 37.105 are derived based on legacy non-AAS requirement levels. Before we provide response to ECC PT1 there may be a need to re-evaluate and check that requirements are sufficient to guarantee adjacent channel co-existence and protection of other 3GPP bands used in the same geographical area.

  - Question 3: Relevant simulation assumptions

For AAS BS operating below 1 GHz it is reasonable to assume that the antenna size will be larger than the antenna originally assumed for non-AAS (dual polarized single column passive antenna). Therefore, it would be reasonable to establish relevant antenna assumptions including array size and sub-array size for AAS BS below 1 GHz.

For out-of-band evaluation RAN4 need to establish relevant simulation assumption of how the array correlation roll-off moving away from the carrier edge. The discussion has just started regarding modelling out-of-carrier characteristics for band n104 (6125 to 7125 MHz).

It would be reasonable to consider array sizes for AAS BS operating below 1 GHz to assume the number of transceivers to be in the range 8 < N < 32. In the table below array sizes previously communicated to ITU-R WP 5D is collected together with an example configuration for the frequency range 700 to 900 MHz.

Table 2.3-1: Overview of AAS BS array configurations

| Frequency range | Transceiver configuration | Array configuration (Msubx1, MxN) | Gain (dBi) | Reference |
| --- | --- | --- | --- | --- |
| 700 to 900 MHz | 16 TX, 16 RX | [5x1, 2x4] | 23 | Not established |
| 1710 to 4990 MHz | 64 TX, 64 RX | 3x1, 4x8 | 26 | TR 38.922, clause 4.4.1.2 |
| 7125 to 8400 MHz | 256 TX, 256 RX | 3x1, 8x16 | 32 | TR 38.922, clause 5.4.1.2 |
| 14800 to 15350 MHz | 768 TX, 768 RX | 4x1, 16x24 | 38 | TR 38.922, clause 6.5.1.2 |

Observation 3: RAN4 need to establish AAS array parameters for the frequency range below 1 GHz, following structure used for previous frequency ranges.

Regarding the out-of-carrier modelling assumptions RAN4 need to agree to parameters accounting for array de-correlation roll-off, as indicated in TR 38.922, clause 7.3.2.1.2.

Since the array factor gain relies on coherent combining of signals it is not expected that the array factor will operate outside the carrier with full strength in situations outside the carrier where the emission is created as the sum of correlated intermodulation distortion and uncorrelated wide band noise. The Inter-Modulation Distortion (IMD) is coming from non-linear transmitter. The characteristics are typically categorized as distortions due third order (IM3), fifth order (IM5), seven order (IM7), etc. IMD. It is known that the order of IMD will determine the spectral distribution of the distortion. The IMD will also depend on the wanted signal carrier bandwidth, B. The IMD response typically reduces moving away from the edge of the wanted signal.

For the wanted carrier (within the interval -0.5B≤fc≤0.5B, fc is the carrier centre frequency) full array factor is expected which means that the correlation factor is equal to 1. Moving away from the carrier edge (fc>0.5B) the correlation will drop gradually due to the composition of the emission outside the carrier. The emission will be created as the sum of correlated noise due to intermodulation products and non-correlated noise due to wideband transmitter noise. The relation to the induvial noise levels will determine the out-of-carrier correlation properties in the adjacent emission regions. At a certain frequency offset to the carrier edge the emission noise will be dominated by wideband noise which means that the correlation is equal to 0.

Therefore, it would be appropriate to define a model for the array correlation factor to account for carrier bandwidth and decaying correlation moving away from the carrier edge.

To properly model the AAS base station array antenna gain pattern outside the carrier the correlation factor needs to be modelled as a function of frequency offset to the carrier centre frequency. To capture fundamental aspects of the array antenna gain drop outside the wanted signal a parameterized piece-wise linear roll-off model can be adopted for the correlation factor, as visualized in Figure 2.3-1.

Figure 2.3-1: Array correlation factor roll-off model

The roll-off is modelled to reduce stepwise moving away from the centre frequency, also the characteristics is assumed to be symmetrical around the wanted carrier. Close to the carrier the correlation is expected to be close to 1, while moving towards +/-1.5B reducing to r2. At f1 the correlation has reduced even more to r1. At f0 the correlation has reduced to 0, which corresponds to that the array factor gain in 0 dB.

For this model to be use-full parameter values for B, f0, f1, r1 and r2 needs to be determined.

Using the approach above the antenna model can be extended to capture array antenna gain outside the wanted carrier as:

AAθ,φ,f=Asubθ,φ+10log101+ρfm=1Mn=1Nwm,nvm,n2-1	(Eq. 2.3-1)

Observe that the extended array antenna model only captures effect due to array signal correlation and not effects due to antenna element detuning effects of mismatch effects in transmission lines and Radio Distribution Network (RDN).

Observation 4: RAN4 need to establish relevant parameters for correlation roll-off outside the carrier for the frequency range 700 to 900 MHz (including adjacent channels, harmonics and other spectral regions).

The discussion to define correlation parameters values have been ongoing since the start of AAS in Rel-12 without any consensus and corresponding agreements. It is thus clear that RAN4 cannot directly answer the given questions without further discussion.

# Conclusion

Analysing the request for information from ECC PT1 and RAN4 status we made the following observations:

Observation 1: The RAN4 AAS BS requirements are applicable for all bands. However, RAN4 currently do not have recent information regarding relevant spatial unwanted emission for AAS BS operating below 1 GHz. If needed and AAS sub-GHz products are deployed, RAN4 need to initiate a study to collect measurement data for unwanted emission relevant for AAS BS operating below 1 GHz.

Observation 2: Requirement levels for unwanted emissions in TS 38.104 and TS 37.105 are derived based on legacy non-AAS requirement levels. Before we provide response to ECC PT1 there may be a need to re-evaluate and check that requirements are sufficient to guarantee adjacent channel co-existence and protection of other 3GPP bands used in the same geographical area.

Observation 3: RAN4 need to establish AAS array parameters for the frequency range below 1 GHz, following structure used for previous frequency ranges.

Observation 4: RAN4 need to establish relevant parameters for correlation roll-off outside the carrier for the frequency range 700 to 900 MHz (including adjacent channels, harmonics and other spectral regions).

It is obvious that RAN4 need to do work before answering questions from ECC PT1. To speed up interaction between the groups it is essential to ask RAN for mandate to send information directly to ECC PT1 according to approved time schedule.

Proposal: Use the information in this contribution and attached draft LS to further discuss on how to proceed in RAN4.

At the end of this contribution, we have attached a draft LS to RAN asking for mandate for direct communication to ECC PT1 to speed up communication when answers in RAN4 have been established.

# References

[1]	R4-2513058, “Parameters for AAS BS operating in bands below 1 GHz”, ECC PT1

DRAFT LS:

TSG-RAN Working Group 4 (Radio) meeting #116-bis	R4-xxyyzzz

Prague, EU, 13th to 17th October 2025

Title:	Draft LS to ECC PT1 on parameters for AAS BS operating in bands below 1 GHz

Response to:	LS R4-2513058 on parameters for AAS BS operating in bands below 1 GHz

Release:	-

Work Item:	-

Source:	RAN WG4

To:	TSG RAN

Cc:	-

Contact person:	Torbjörn Elfström

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

-

# 1	Overall description

RAN WG4 received the incoming LS from ECC PT1 on parameters for AAS BS operating in bands below 1 GHz and would like to thank for the opportunity to give input on this topic. In the LS, “ECC PT1 is seeking technical and operational characteristics of AAS BS used for terrestrial IMT systems that would operate in frequency bands of 700 to 900 MHz.

RAN WG4 has made an assessment of the work needed to complete the task of responding and also held first brief technical discussions.

For the three questions, RAN WG4 identified the following estimated completion dates for the work:

•	Simulation parameters (in-band):	Estimated date for completion: May 2026 (RAN WG4#119)

•	Re-evaluation of unwanted emission requirements:	Estimated date for completion: August 2026 (RAN WG4#120)

•	Unwanted emission characteristics (out-of-band):		Estimated date for completion: November 2026 (RAN WG4#121)

Note that for the initial response, if TSG RAN wishes any partial technical input made available by RAN WG4#119 in May to be sent to ECC PT1 should give a mandate for RAN WG4 to respond directly to ECC PT1. It should also be noted that even November 2026 is a challenging RAN WG4 deadline and RAN WG4 may need more time to complete work needed to answer all questions.

# 2	Actions

To TSG RAN

ACTION: 	RAN WG4 asks TSG RAN to use the planning and timeline information given above in the RAN work planning and for a response LS to ECC PT1, noting that:

- If TSG RAN wishes any partial technical input made available by RAN WG4#119 in May to be sent to ECC PT1, TSG RAN should give a mandate for RAN WG4 to respond directly to ECC PT1.

- Information can be submitted in steps to ECC PT1, which could be beneficial for their work progress.

- It is the view of RAN WG4 that even November 2026 is a challenging RAN WG4 deadline.

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN4 Meeting #117		17 – 21 November 2025		Dallas, US

TSG RAN4 Meeting #118		9 – 13 February 2026		Gothenburg, EU

TSG RAN4 Meeting #118-bis	13 – 17 April 2026			EU

TSG RAN4 Meeting #119		18 – 22 May 2026			China

TSG RAN4 Meeting #120		24 – 28 August 2026			Maastricht, EU

TSG RAN4 Meeting #120-bis	12 – 16 October 2026		Korea

TSG RAN4 Meeting #121		16 – 20 November 2026		Calgary, Canada