3GPP TSG-RAN WG4 Meeting #116bis	R4-2513948

Prague, Czech Republic, October 13th-17th, 2025

Source:	Nokia

Title:	Discussion on General Aspects for Two-sided AI/ML Models

Agenda item:	7.9.2

Document for:	Discussion

Introduction

As quoted below, the RAN4#116 WF states the scalability as part of the topics to be discussed in the WI.

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the scalability alternatives

In this contribution, we share our views on scalability and generalization aspects for two-sided CSI compression.

Discussion

Scalability aspects for two-sided AI/ML models

The scalability concept has not been discussed yet in RAN4, at least in the context of UE-sided AI/ML use cases. However, it was discussed and evaluated in RAN1 for CSI compression. According to the RAN1 agreement stated below, the scalability study for CSI compression considers parameters like Tx port number, CSI payload size, bandwidth and subband size. These parameters affect the format and size of the PMI data.

RAN1#120 Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

Other parameters that affect the format and size of the compressed representation of the PMI, like latent vector dimension and quantization scheme, can be also considered as part of the scalability parameters. We propose that RAN4 initiates the discussion on scalability in AI/ML-based CSI compression by identifying the parameters that are relevant for scalability discussions.

RAN4 to consider the following parameters for scalability testing in AI/ML-based CSI compression:

- Parameters affecting the format and size of the PMI data

  - Antenna port configuration (number of array rows, number of array columns, polarization)

  - CSI feedback payload size

  - Bandwidth configuration (number of subbands, relative location of subbands)

- Parameters affecting the format and size of the compressed representation of the PMI

  - Latent vector dimension

  - Quantization scheme

One more aspect in RAN1 scalability study is the scalability solutions. According to the summary of performance results for CSI compression in [1], appropriate scalability solutions, like truncation/padding, adaptive quantization granularities, and adaptation layer in the AI/ML model, may be needed to scale the dimensions of the AI/ML model when the training dataset includes data samples subject to configuration#A which has different input/output dimension than configuration#B used for inference.

Scalability solutions, like truncation/padding, adaptive quantization granularities, and adaptation layer in the AI/ML model, are considered by RAN1 to scale the dimensions of the AI/ML model when training and inference assume different input/output dimensions.

Generalization aspects for two-sided AI/ML models

The RAN1 generalization evaluations for CSI compression considered the following 3 cases [1]:

- Generalization Case 1: the AI/ML model is trained with a dataset subject to a certain scenario#B/configuration#B and applied for inference with the same scenario#B/configuration#B.

- Generalization Case 2: the AI/ML model is trained with a dataset from a different scenario#A/configuration#A and applied for inference with scenario#B/configuration#B.

- Generalization Case 3: the training dataset is constructed with data samples subject to more than one scenario/configuration (evaluations studied up to four scenarios/configurations) including scenario#B/configuration#B and the AI/ML model is applied for inference with scenario#B/configuration#B.

Examples of deployment scenarios considered in the generalization evaluations include InH, Umi, and UMa, while the configurations include carrier frequencies (e.g., 3.5/4GHz, 2GHz), UE distributions (e.g., indoor, outdoor), and TxRU mappings (e.g., [2,8,2], [4,4,2]). Final observations in [1] show that, for generalization Case 2, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others. For generalization Case 3, generalized performance of the AI/ML model can be achieved.

According to RAN1 generalization evaluations for CSI compression, a generalized performance of the AI/ML models can be achieved when training the models on mixed datasets.

From RAN4 perspective, and as this is the beginning of the WI, the method for creating the mixed dataset is not agreed yet (using samples from different companies assuming a single scenario/configuration or using samples from different companies assuming different scenarios/configurations). While waiting for that to be agreed, RAN4 can initiate the discussion on generalization testing in AI/ML-based CSI compression by identifying the relevant parameters. We believe that these parameters should be the ones that may affect the statistical properties of the PMI data, but do not affect the format and size of the PMI data array or the format and size of the compressed representation of the PMI, like deployment scenario and carrier frequency.

RAN4 to identify the relevant parameters for generalization testing in AI/ML-based CSI compression (e.g., carrier frequency, deployment scenario).

Conclusion

The following proposals and observations were made in this contribution:

Proposal 1: RAN4 to consider the following parameters for scalability testing in AI/ML-based CSI compression:

	Parameters affecting the format and size of the PMI data

o	Antenna port configuration (number of array rows, number of array columns, polarization)

o	CSI feedback payload size

o	Bandwidth configuration (number of subbands, relative location of subbands)

	Parameters affecting the format and size of the compressed representation of the PMI

o	Latent vector dimension

o	Quantization scheme

Observation 1: Scalability solutions, like truncation/padding, adaptive quantization granularities, and adaptation layer in the AI/ML model, are considered by RAN1 to scale the dimensions of the AI/ML model when training and inference assume different input/output dimensions.

Observation 2: According to RAN1 generalization evaluations for CSI compression, a generalized performance of the AI/ML models can be achieved when training the models on mixed datasets.

Proposal 2: RAN4 to identify the relevant parameters for generalization testing in AI/ML-based CSI compression (e.g., carrier frequency, deployment scenario).

- References

- TR 38.843 V19.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 19)”.