3GPP TSG-RAN WG4 Meeting #116bis	R4-2513949

Prague, Czech Republic, October 13th-17th, 2025

Source:	Nokia

Title:	Discussion on Interoperability for Two-sided AI/ML Models

Agenda item:	7.9.3

Document for:	Discussion

Introduction

In RAN#109, the revised WID of Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was agreed [1]. The following scope was captured for RAN4 in this WID.

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In the RAN4#116 meeting, the following WF was made for the WI discussions. Among the topics listed below, we handle the scalability aspects in the general aspects paper [R4-2513948] and the performance metrics in the requirements paper [R4-2513950]. In this contribution, we share our views on the remaining topics, including assumptions on models and datasets, and relation to RAN1 inter-vendor training collaboration directions.

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also, may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

Discussion

Assumptions for test decoder and reference encoder structures

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on assumptions for the test decoder and reference encoder structures - Also, may need to consider how many test decoders and encoders

During the SI phase, RAN4 simulations on the feasibility study of the different test decoder options relied mainly on the CNN-based model structure for encoders and decoders, although other transformer- and MLP-based model structures were considered in extended simulation activities. From the RAN1 perspective, the transformer-based model structure was the one used during the SI phase for studying inter-vendor collaboration and scalability issues. By comparing the transformer-based model structures used by RAN4 and RAN1, and depicted in the Annex, we can observe the similarity between the two model structures.

The transformer-based model structures used in RAN1 and RAN4 during the SI phase are similar.

One question that needs to be answered in RAN4 to decide about the model structures to assume for the WI is whether to adopt CNN-based models (same as in the SI) or transformer-based models (same as RAN1). To answer this question, we use the following agreement made in RAN4#116 meeting regarding the criteria for selecting the encoder/decoder model.

Criteria for selecting the encoder/decoder model: Agreements: - Achievable performance, complexity and robustness in different conditions need to be taken into account in selecting the model. - Whether a single or multiple test decoders is needed will need more elaboration during the WI phase - For option 3 the reference encoder should have been joint trained with the test decoder.

Performance. Table 1 reports the results comparing the CNN reference model used in RAN4 to the reference transformer model used in RAN1 in terms of SGCS. Nokia’s dataset is used for both training and testing, and Rel-16 eType II codebook is used as the baseline for SGCS gain calculation (SGCS=0.689). In addition to the higher SGCS gain shown in Table 1, and according to RAN1 evaluations, the maximum throughput gain obtained with CNN-based models over Rel-16 eType II codebook can reach up to 5%, while it can reach up to 20% with transformer-based models.

Table 1: SGCS performance comparison of CNN-based and transformer-based models (Rel-16 eType II codebook SGCS=0.689).

| Model | Number of parameters (enc+dec) | SGCS (with 2-bit scalar quantization) | SGCS gain over Rel-16 eType II codebook |
| --- | --- | --- | --- |
| CNN [RAN4 reference model] | 1.9M | 0.698 | 1.3% |
| Transformer [RAN1 reference model] | 2.4M | 0.724 | 5.1% |

Transformer-based models outperform CNN-based models in terms of SGCS and throughput gains over Rel-16 eType II codebook.

Complexity. Transformer-based models can have higher complexity than CNN-based models, e.g., higher number of model parameters as depicted in Table 1. However, techniques like model pruning and knowledge distillation can be utilized to lower the complexity of transformer-based models.

Techniques like model pruning and knowledge distillation can be utilized to lower the complexity of transformer-based models.

Generalization and scalability. According to RAN1 evaluations [2], generalized performance of the transformer-based models can be achieved, especially when the training dataset is constructed with data samples subject to more than one scenario/configuration including the inference scenario/configuration (generalization Case 3). For scalability, appropriate solutions, like truncation/padding, adaptive quantization granularities, and adaptation layer in the model architecture, may need to be performed to scale the dimensions of the transformer-based models when the training dataset includes data samples subject to a configuration which has different input/output dimension than the inference configuration.

Generalized and scalable performance of the transformer-based models can be achieved with some design considerations, like using a mixed dataset for training and adopting appropriate scalability solutions.

Taking the above analysis into account, we propose to adopt the transformer-based model structure in the RAN4 normative work on AI/ML-based CSI compression.

RAN4 to adopt the transformer-based model structure for the test decoder and the corresponding reference encoder.

Regarding the number of test decoder models to consider in RAN4, we believe that it depends mainly on the configurations that RAN4 will test for AI/ML-based CSI compression and the generalization/scalability capabilities of the test decoder model that RAN4 will select. For instance, let us consider the case where RAN4 agrees to use the transformer-based model for the test decoder and corresponding reference encoder and to define test cases for several configurations of the number of antenna ports. In this case, if simulations prove that the performance of the agreed transformer-based model does not degrade for the different configurations of the number of antenna ports (i.e., scalable performance is achieved), then RAN4 may consider only one test decoder model.

RAN4 to decide about the number of test decoder models based on the test configurations to be identified as well as the generalization/scalability capabilities of the models to be agreed for the normative work.

Dataset and channel model assumptions

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the dataset assumptions and dataset - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

The first assumption about the dataset to capture in RAN4 specifications is related to the format of the dataset (i.e., eigenvectors or raw channel data). We believe that this point is closely related to the assumptions on the format of the input to the encoder and the latent space. The format of the reference dataset to capture in RAN4 specifications should be selected in a way that works for all possible formats of the input to the encoder and the latent space. One way to cover all these options is to specify the dataset in terms of raw channel data accompanied with a clear agreed procedure of how to get channel eigenvectors from the raw channel data. This way is equivalent to specifying eigenvectors but without excluding the raw channel data option.

RAN4 can capture the reference dataset used for training the test decoder in the format of raw channel data accompanied with a clear agreed procedure of how to generate channel eigenvectors from the raw channel data.

Another assumption about the dataset to capture in RAN4 specifications is related to the method for generating the mixed dataset. In the SI, RAN4 generated the mixed dataset assuming a single configuration and samples from multiple companies. From the other side, in RAN1, companies generated mixed datasets assuming multiple configurations, especially for generalization evaluations. If RAN4 reuses the same method as in the SI (i.e., mixed dataset from different companies), this may lead to the creation of several datasets for generalization purposes. The other case, where RAN4 generates the mixed dataset considering several configurations, will raise the question of which company’s dataset to select and may add extra work to do from RAN4 perspective. Hence, one solution to generate the mixed dataset is to create it considering both factors, i.e., a limited number of agreed configurations and samples from different companies. This dataset can be used for both performance testing and generalization/scalability testing. The procedure for creating the mixed dataset using the different datasets to be shared by companies can be agreed also.

RAN4 to utilize the following steps for generating the mixed dataset:

- 1. Select a limited number of agreed configurations for data generation

- 2. Share datasets generated using the configurations selected in Step (1)

- 3. Create one mixed dataset using the datasets shared in Step (2)

The other topic mentioned in the RAN4#116 WF is the channel model for requirements. TDL channel model is used for defining performance requirements in legacy PMI reporting and will be also used for defining performance requirements for CSI prediction. Hence, we believe that the TDL channel model can also be used for defining the AI/ML-based CSI compression performance requirements.

RAN4 to consider the TDL channel model for defining the AI/ML-based CSI compression performance requirements.

Means to capture models and datasets in 3GPP

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Develop a means to capture model structures and model parameters in 3GPP

To define requirements for the encoder, RAN4 needs to specify a test decoder, the corresponding reference encoder, and a dataset. Links to specified locations of the models and the dataset can be used in RAN4 specifications to refer to the models and the dataset used for defining the requirements.

The models for the test decoder and the reference encoder and the dataset can be captured in RAN4 specifications using links to specified locations.

Relation to RAN1 inter-vendor training collaboration directions

In the RAN4#116 WF, there was a question on “whether the decoder/encoder for RAN4 are the same as for option C”.

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model

We believe that it is up to RAN1 to use the input/feedback from RAN4 for the directions that they are considering for specification (i.e., Direction C and/or Direction A with its two sub-options). We do not see a meaningful outcome from the comparison at this stage. RAN4 can focus on specifying the test decoder, corresponding reference encoder, and dataset for defining requirements for the encoder.

RAN4 to focus on specifying the test decoder, corresponding reference encoder, and dataset needed for defining the requirements for the encoder. No need for the comparison between RAN4 test decoder options and RAN1 inter-vendor training collaboration directions at this stage.

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: The transformer-based model structures used in RAN1 and RAN4 during the SI phase are similar.

Observation 2: Transformer-based models outperform CNN-based models in terms of SGCS and throughput gains over Rel-16 eType II codebook.

Observation 3: Techniques like model pruning and knowledge distillation can be utilized to lower the complexity of transformer-based models.

Observation 4: Generalized and scalable performance of the transformer-based models can be achieved with some design considerations, like using a mixed dataset for training and adopting appropriate scalability solutions.

Proposal 1: RAN4 to adopt the transformer-based model structure for the test decoder and the corresponding reference encoder.

Proposal 2: RAN4 to decide about the number of test decoder models based on the test configurations to be identified as well as the generalization/scalability capabilities of the models to be agreed for the normative work.

Proposal 3: RAN4 can capture the reference dataset used for training the test decoder in the format of raw channel data accompanied with a clear agreed procedure of how to generate channel eigenvectors from the raw channel data.

Proposal 4: RAN4 to utilize the following steps for generating the mixed dataset:

1. Select a limited number of agreed configurations for data generation

2. Share datasets generated using the configurations selected in Step (1)

3. Create one mixed dataset using the datasets shared in Step (2)

Proposal 5: RAN4 to consider the TDL channel model for defining the AI/ML-based CSI compression performance requirements.

Proposal 6: The models for the test decoder and the reference encoder and the dataset can be captured in RAN4 specifications using links to specified locations.

Proposal 7: RAN4 to focus on specifying the test decoder, corresponding reference encoder, and dataset needed for defining the requirements for the encoder. No need for the comparison between RAN4 test decoder options and RAN1 inter-vendor training collaboration directions at this stage.

- References

- RP-252445, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#109, September 15-18, 2025, Beijing, China.

- TR 38.843 V19.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 19)”.

- Annex

Transformer-based model structure used in RAN4

Transformer-based model structure used in RAN1

RAN1#120 Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment. Decoder description: The decoder has a mirroring design as the encoder. Details are to be discussed.