3GPP TSG-RAN WG4 Meeting #116bis	R4-2513950

Prague, Czech Republic, October 13th-17th, 2025

Source:	Nokia

Title:	Discussion on Core Requirements for Two-sided AI/ML Models

Agenda item:	7.9.4

Document for:	Discussion

Introduction

In RAN#109, the revised WID of Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was agreed [1]. The following scope was captured for RAN4 in this WID.

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we share our views on the core requirements for two-sided CSI compression, including performance requirements and LCM requirements.

Discussion

Test metric(s) for performance requirements

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and agree the test metric for the performance requirements.

In RAN4#114bis, the performance requirements for CSI prediction were agreed to be defined based on the gamma ratio, defined in the current PMI reporting requirements in 38.101-4.

RAN4#114bis Agreement Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

For the performance requirements of CSI prediction, the throughput gain can be a sufficient metric for reflecting the potential gains of AI/ML-based CSI prediction (i.e., combatting channel aging and enhancing downlink throughput as a result). However, considering throughput only when defining CSI compression performance requirements (i.e., same way used in legacy PMI and Rel-19 CSI prediction requirements) may not be an optimal strategy as the main goal of CSI compression is not only to enhance feedback accuracy and thus downlink throughput, but also to reduce the feedback overhead.

RAN4 to consider both throughput and feedback overhead metrics for the definition of the AI/ML-based CSI compression performance requirements.

For the throughput metric, and similarly to the CSI prediction case, the ratio used in the PMI reporting requirements in TS 38.101-4 can be adopted also in the CSI compression case with some needed updates. We here propose γTH, which represents the relative increase in throughput when the transmitter is configured according to the PMI determined based on the reconstructed UE reports compared to the case when the transmitter is using legacy codebooks (e.g., Rel-16 eType II codebook):

γTH= tue-AIMLtue-CB

Where:

- tue-AIML is 90% of the maximum throughput obtained at SNRue-AIML using the precoders configured according to the post-processed reconstructed UE reports.

- tue-CB is the throughput measured at SNRue-AIML using a legacy codebook (e.g., Rel-16 eType II codebook).

- Note: SNRue-AIML can be calculated in the same way as  SNRue and  SNRfollow1,follow2 defined in section 6.3 of TS 38.101-4.

For the throughput performance requirements of AI/ML-based CSI compression, RAN4 to consider the following metric: γTH= tue-AIMLtue-CB, where:

- tue-AIML is 90% of the maximum throughput obtained at SNRue-AIML using the precoders configured according to the post-processed reconstructed UE reports.

- tue-CB is the throughput measured at SNRue-AIML using a legacy codebook (e.g., Rel-16 eType II codebook).

- Note: SNRue-AIML can be calculated in the same way as  SNRue and  SNRfollow1,follow2 defined in section 6.3 of TS 38.101-4.

As the CSI compression use case aims to reduce the overhead required to feedback the CSI report over the air interface, a reasonable performance metric can be defined to quantize the relative savings provided by this feature in terms of CSI feedback overhead. Hence, we here propose γOH, which represents the relative reduction in CSI feedback overhead when the transmitter is configured according to the PMI determined based on the reconstructed UE reports compared to the case when the transmitter is using legacy codebooks (e.g., Rel-16 eType II codebook):

γOH= OHue-AIMLOHue-CB

Where:

- OHue-AIML is the weighted average of CSI payload per rank using the AI/ML-based CSI compression.

- OHue-CB is the weighted average of CSI payload per rank using a legacy codebook (e.g., Rel-16 eType II codebook).

For the feedback overhead performance requirements of AI/ML-based CSI compression, RAN4 to consider the following metric: γOH= OHue-AIMLOHue-CB, where:

- OHue-AIML is the weighted average of CSI payload per rank using the AI/ML-based CSI compression.

- OHue-CB is the weighted average of CSI payload per rank using a legacy codebook (e.g., Rel-16 eType II codebook).

There are several ways to use γTH and γOH for defining performance requirements for AI/ML-based CSI compression, depending on the testing goals. For instance, if the testing goal would be showing that the performance of AI/ML-based CSI compression is no worse than legacy codebooks, then RAN4 may in this case define two target values for γTH and γOH, where γTH should be greater than or equal to 1 and γOH should be less than or equal to 1. Another example of a testing goal can be to show that with the same throughput performance, the CSI feedback overhead obtained with AI/ML-based CSI compression is reduced compared to legacy codebooks. In this case, RAN4 may define one target for γOH which should be less than 1, while γTH is equal to 1.

Activation delay requirements

The LCM-related aspects for UE-sided use cases were addressed in RAN4 under both the General Aspects and the use case-specific discussions. These discussions rely heavily on the LCM definitions and procedures agreed by RAN1/RAN2. From RAN4 perspective, delay requirements for AI/ML configuration activation, deactivation, switching, and fallback are discussed as part of the LCM requirements. According to agreements stated below, RAN4 agreed to deprioritize switching and fallback requirements and to not define delay requirements for AI/ML configuration deactivation.

RAN4#115 Agreement Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. RAN4#116 Agreement - No need to create deactivation requirements for AI functionality

Alongside deactivation/switching/fallback, activation was also discussed as part of the LCM framework. In RAN4#116 meeting, it was concluded that there is no need to define activation delay requirements for the beam prediction use case based on the feedback from RAN2. According to the LCM framework defined by RAN2 for UE-sided models, the UE has to be ready to start measurements for inference after sending the RRCReconfigurationComplete message. As a result of this understanding, both activation delay and prediction reporting delay would have the same starting and ending points and RAN4 will define CSI prediction reporting delay requirements, so there is no need to define activation delay requirements for the beam prediction use case.

Similarly to the UE-sided models and according to the WID [1], RAN2 will specify the signalling and protocol aspects of LCM enabling AI/ML functionality activation, deactivation, switching, and fallback for the two-sided models. Hence, RAN4 should wait for input from RAN2 regarding this framework to be able to decide whether to define activation delay requirements.

RAN4 to wait for RAN2 progress on the LCM framework for two-sided models to decide about the need for defining activation delay requirements for AI/ML-based CSI compression.

Performance monitoring reporting requirements

We quote below the agreements related to performance monitoring in CSI compression from several RAN1 meetings.

RAN1#117 Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches. RAN1#120bis Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. RAN1#120bis Conclusion UE-side monitoring is feasible. RAN1#120bis Observation - Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports. RAN1#121 Agreement For NW-side performance monitoring, RAN1 concludes that target CSI reporting via legacy codebooks can be used. Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side performance monitoring with additional cost of complexity and overhead at UE side. For UE-side side monitoring, RAN1 concludes that it is feasible, but some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

- According to the above agreements, for NW-side performance monitoring, UE can report target CSI via legacy CSI codebooks or target CSI with CSI codebook enhancement via higher-resolution parameter combination. If UE reports the legacy codebook-based CSI, as in the non-AI/ML scenario, the requirements on such reports should not differ from legacy requirements. However, if the performance monitoring report includes target CSI with CSI codebook enhancement via higher-resolution parameter combination, it is unclear whether RAN4 would need to define new requirements in this case.

In case of NW-side performance monitoring in the two-sided CSI compression use case, RAN4 to wait for RAN1 progress on the report quantity (i.e., target CSI reporting via legacy codebooks or target CSI reporting with CSI codebook enhancement) to decide whether to define new requirements for performance monitoring reporting.

In case of UE-side performance monitoring, UE is expected to report the performance monitoring metric(s) (e.g., a KPI computed based on the output of the CSI reconstruction model, an intermediate KPI computed without reconstructing a target CSI). Such reports are different from legacy PMI reports, hence, RAN4 should consider defining accuracy and delay requirements for performance monitoring reporting from the UE in this case using input from RAN1 (i.e., exact definition of the performance monitoring metric(s)).

In case of UE-side performance monitoring in the two-sided CSI compression use case, RAN4 to wait for RAN1 progress on the performance monitoring metric(s) to be reported by the UE to decide whether to define accuracy and delay requirements for performance monitoring reporting.

Conclusion

The following proposals were made in this contribution:

Proposal 1: RAN4 to consider both throughput and feedback overhead metrics for the definition of the AI/ML-based CSI compression performance requirements.

Proposal 2: For the throughput performance requirements of AI/ML-based CSI compression, RAN4 to consider the following metric: γTH= tue-AIMLtue-CB, where:

	tue-AIML is 90% of the maximum throughput obtained at SNRue-AIML using the precoders configured according to the post-processed reconstructed UE reports.

	tue-CB is the throughput measured at SNRue-AIML using a legacy codebook (e.g., Rel-16 eType II codebook).

	Note: SNRue-AIML can be calculated in the same way as  SNRue and  SNRfollow1,follow2 defined in section 6.3 of TS 38.101-4.

Proposal 3: For the feedback overhead performance requirements of AI/ML-based CSI compression, RAN4 to consider the following metric: : γOH= OHue-AIMLOHue-CB, where:

	OHue-AIML is the weighted average of CSI payload per rank using the AI/ML-based CSI compression.

	OHue-CB is the weighted average of CSI payload per rank using a legacy codebook (e.g., Rel-16 eType II codebook).

Proposal 4: RAN4 to wait for RAN2 progress on the LCM framework for two-sided models to decide about the need for defining activation delay requirements for AI/ML-based CSI compression.

Proposal 5: In case of NW-side performance monitoring in the two-sided CSI compression use case, RAN4 to wait for RAN1 progress on the report quantity (i.e., target CSI reporting via legacy codebooks or target CSI reporting with CSI codebook enhancement) to decide whether to define new requirements for performance monitoring reporting.

Proposal 6: In case of UE-side performance monitoring in the two-sided CSI compression use case, RAN4 to wait for RAN1 progress on the performance monitoring metric(s) to be reported by the UE to decide whether to define accuracy and delay requirements for performance monitoring reporting.

- References

- RP-252445, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#109, September 15-18, 2025, Beijing, China.