3GPP TSG-RAN WG4 Meeting #116bis	R4-2513952

# Prague, Czech Republic, October 13-17, 2025

Source:	Nokia

Title:	Discussion on RRM aspects for enhanced LBCA switching

Agenda item:	7.8.4

Document for:	Discussion

Introduction

The core part of WI:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

  - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

  - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw

    - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

During RAN#109 meeting following objectives for the work item NR_LBCA_Sw_enh for was recorded [1]

Comparing this to the TR 38.768 [2] concluded in Rel-19 one can make following observations about the main changes for this new work item.

- Case 3: aggregated DL reception for SDL and FDD DL bands.

- Band combinations are limited to CA_n12A-n29A and CA_n28A-n67A.

- There is no interferer expected between two carriers.

Discussion

In order to have a better overview on the impacted requirements we divide the requirements based on PCell and SCell when LBCA-sw is configured.

PCell requirements

A critical challenge arises during LBCA switching operations, specifically when transitioning from FDD UL to SDL while the FDD DL remains continuously active. Due to the diverse RF front-end designs across UEs, devices may necessitate switching gaps during switching between FDD UL and SDL. Which carriers that are affected depend on the UE RF design. These gaps are primarily driven by the need to re-tune components such as Phase-Locked Loops (PLLs) or antennas. Furthermore, advanced hardware architectures, such as those employing three Local Oscillators (LOs) to independently serve the FDD DL, FDD UL, and SDL DL carriers, can potentially eliminate the need for PLL re-tuning, thereby avoiding associated switching gaps. This highlights the architectural dependency of switching gap requirements. These potential switching gaps, even if brief, pose a significant problem as they can interrupt the contiguous FDD downlink channel, impacting user experience and data integrity. Therefore, it is needed to RAN4 discuss whether these switching gaps cause any interruption to PCell FDD DL.

The presence of non-interrupted PCell DL depends on UE RF design.

RAN4 to clarify if there is any interruption to PCell DL during PCell UL and SDL switching gap under Rel-20 LBCA-sw enhancement.

If the switching gap does not impact PCell downlink reception, legacy requirements are applied to the PCell instead of Rel-19 LBCA requirements. However, if the switching gap causes interruptions to PCell downlink reception, PCell measurement requirements must account for the potential overlap between the switching gap and PCell measurement periods.

If switching gap does not cause interruption to PCell DL, legacy requirements (requirements without LBCA switching) are applied to PCell instead of Rel-19 LBCA requirements.

Figure 1 illustrates the scenarios where the switching gap for Case 3, impacts or does not impact PCell measurements. For the purpose of this discussion, PCell measurements refer to the reception of SSBs for the PCell and whether these SSBs collide with the switching gap. The requirements for the switching gap in relation to PCell measurements depend on the number of LOs utilized by the UE. It is currently unclear under which conditions, in a dual receiver (RX) configuration, the PCell measurements remain unaffected by the switching gap. Based on our analysis, if the UE is capable of handling switching on one RX (e.g., for SDL and FDD UL) while simultaneously receiving FDD DL on another RX, with each RX operating with its own independent LO, the PCell measurements are expected to remain unaffected. However, in scenarios where both RXs share a single LO, and the LO requires re-tuning during the switching gap, the reception of FDD DL would be impacted. This situation introduces ambiguity and requires further clarification to ensure alignment on the conditions under which PCell measurements are unaffected by the switching gap.

If UE handles FDD DL with an independent LO than SDL, PCell DL is not impacted by switching pattern, thus legacy PCell requierments (before Rel-19) are hold.

Figure 1 Left) Switching gap not affecting PCell measurements. Right) Switching gap affecting PCell measurements

As per WI description, in Rel-20 LBCAsw enh, “the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA” is adopted as base line. Therefore, if SSBs for PCell measurements overlap with switching gap then RAN4 needs to clarify how PCell requirements should be updated for Rel-20 LBCA-sw with dual RX. If interruption is not impacting PCell DL, legacy requirements apply to PCell (not Rel-19 LBCA requirements). If interruption is caused to PCell DL, PCell measurement requirements to take into account possible overlap of switching gap and PCell measurements.

RAN4 to clarify whether/how PCell measurement requirements should be updated for Rel-20 LBCA-sw with dual RX.

By adopting Rel-19 semi static switching mechanism, one probable scenario is when the SSBs of the PCell (FDD DL) are overlapping with switching gaps. Figure 2 illustrates this scenario. In the event of this overlapping, with regards to proposal 1, PCell SSB-based measurements such as, BFD, RLM and PCell measurement delay are impcated.

If there is interruption to PCell DL during PCell UL and SDL switching gap, RAN4 to consider the impact of switching gaps overlapping with SSBs for PCell measurements.

Figure 2 PCell SSBs overlap with switching gap

. SCell requirements

RAN4 introduced in Rel-19 LBCA-specific SCell requirements for:

- SCell activation delay

- TCI state activation delay

- Measurement delay

- BFD/CBD

The requirements take into account the impact of the switching pattern on SCell measurements. This means that the requirement is written so that only the measurements that overlap with the SCell active periods according to the switching pattern are taken into account in the delay. Hence, if some measurements are overlapping with PCell active periods according to the pattern, the delay is extended.

From the SCell perspective, regardless of using Rel-19 or Rel-20 switching patterns, there is no difference in behavior. The SCell remains inactive during certain periods, and some of its measurements may still overlap with PCell uplink active intervals. Hence, we think that the Rel-19 approach can be used regarding SCell requirements, and consequently, as the starting point, Rel-19 SCell requirements can be reused for Rel-20 LBCA as the impact of switching gaps is expected to be similar.

requirements on SCell should apply for Rel-20 LBCA-sw with dual RX.

Conclusion

The following proposals were made in this contribution:

Observation 1: The presence of non-interrupted PCell DL depends on UE RF design.

Proposal 1: RAN4 to clarify if there is any interruption to PCell DL during PCell UL and SDL switching gap under Rel-20 LBCA-sw enhancement.

Proposal 2: If switching gap does not cause interruption to PCell DL, legacy requirements (requirements without LBCA switching) are applied to PCell instead of Rel-19 LBCA requirements.

Observation 2: If UE handles FDD DL with an independent LO than SDL, PCell DL is not impacted by switching pattern, thus legacy PCell requierments (before Rel-19) are hold.

Proposal 3: RAN4 to clarify whether/how PCell measurement requirements should be updated for Rel-20 LBCA-sw with dual RX.

Proposal 4: If there is interruption to PCell DL during PCell UL and SDL switching gap, RAN4 to consider the impact of switching gaps overlapping with SSBs for PCell measurements.

Proposal 5: requirements on SCell should apply for Rel-20 LBCA-sw with dual RX.

- References

[1]	RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Telus, Beijing, China, Sep 15-18, 2025

[2]	R4-2509202, 3GPP TR 38.768 V1.1.0, Low NR Band Carrier Aggregation via Switching