3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513964

Prague, CZ, October 13th – 17th 2025

Agenda Item:	6.8.4.2.1

Source:	Ericsson

Title:	OTA test scope reduction

Document for:	Approval

# 1	Introduction

In Rel-19, there are two requirements identified as the most time-consuming and should be prioritized, i.e., TX IM and RX out-of-band blocking. In RAN4#116, the agreement on TX IM was reached. In this contribution, our view on RX out-of-band blocking is shared.

# 2	Discussion

Issue 1-2-1: Out of band blocking - Agreement on step size of OOBB requirement, keep the following options for further discussion: - Option 1: 1/3 of the minimum supported bandwidth - Option 1a: Frequency range (MHz); Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). FR1; 5; 1. 10; 3. 20; 6. - Option 2: use 2/3 of the test signal configuration channel bandwidths Frequency range (MHz); Minimum supported BS channel bandwidth (MHz); Measurement step size (MHz). FR1; 5; 3. 10; 6. 20; 13.

In RAN4#116, the options for RX The out-of-band blocking are down selected to two options as listed in the WF [1]. Option 1 is to reuse the FR2 approach, and the step size needs to be 1/3 of the minimum supported channel bandwidth. If the step size for the interfering signal is 2/3 of the minimum supported channel bandwidth, there is a chance that the exact frequency of the unwanted mixing product ±3·fCW±n·finternal might not be able to fall within receiver bandwidth, i.e., the receiver bandwidth will be located between the unwanted mixing products of the two adjacent measurement points. In this case, the test would not trigger the failure condition. Therefore, while Option 1 may offer less test reduction, it aligns with this requirement for test accuracy. It provides higher confidence that the receiver’s vulnerabilities are not overlooked.

Option 1 may offer less test reduction, but it provides higher confidence that the receiver’s vulnerabilities are not overlooked.

RAN4 adopt Option 1 for OTA test scope reduction of receiver out-of-band blocking requirement.

# Conclusion

In this contribution, views, and proposals on OTA test scope reduction are presented.

Observation 1	Option 1 may offer less test reduction, but it provides higher confidence that the receiver’s vulnerabilities are not overlooked.

Proposal 1	RAN4 adopt Option 1 for OTA test scope reduction of receiver out-of-band blocking requirement.

# References

R4-2512580, Way Forward for [116][304] NR_BS_RF_Part3_OTA_TRP