3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513966

Prague, CZ, October 13th – 17th 2025

Agenda Item:	6.13.2.1

Source:	Ericsson

Title:	New SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of conformance testing of new BS RF requirements for SBFD. Discussion on conformance testing of BS RF TX and RX requirements for SBFD are presented in separate submissions [1-2].

# 2	Discussion

This contribution proposes conformance testing methods for the new requirements for SBFD, i.e., the transient period between SBFD and non-SBFD operation, in-channel adjacent subband leakage, and in-channel adjacent subband blocking.

Transient period between SBFD and non-SBFD

During SBFD operation, the BS transmits and receives in adjacent subbands within the same carrier. When transitioning between SBFD and non-SBFD operation, a transient period occurs. As shown in the table below which is copied from TS 38.104 (V19.2.0), there are four switching cases between SBFD and non-SBFD, and the two highlighted cases will need two test models in each. To evaluate it, it is proposed to use two test models: the existing TM1.1 to represent non-SBFD operation, and a modified version of TM1.1 that is adapted to reflect SBFD operation. More detailed discussion on the modified TM1.1 can be found in [1].

About the test procedure, using the first switching case - SBFD symbol to DL symbol as an example: The test begins with the modified TM1.1 to establish SBFD mode, which includes simultaneous transmission and reception in adjacent subbands. Then the BS switches to the non-SBFD operation using the legacy TM1.1. Measurements are taken during the switching phase.

- There are four switching cases between SBFD and non-SBFD, and the switching from SBFD to DL or the switching from DL to SBFD will need two test models in each case.

For conformance testing of the switching from SBFD to DL or the switching from DL to SBFD, use two test models: the existing TM1.1 to represent non-SBFD DL operation, and a modified version of TM1.1 to represent SBFD operation.

For conformance testing of the switching from SBFD to UL or the switching from UL to SBFD, use one test model, i.e., the modified version of TM1.1.

Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation (copied from TS 38.104 V19.2.0)

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

In-channel adjacent subband leakage

In SBFD scenarios, a BS may transmit in DL subband while receiving in an adjacent UL subband within the same carrier bandwidth. In such cases, leakage from the inter-sector transmitter into the receiver’s adjacent subband can significantly affect performance. To evaluate in-channel adjacent subbband leakage, it is proposed to use modified versions of TM1.1 and TM1.2, which follows the principles used for absolute ACLR. More detailed discussion on the modified TM1.1 can be found in [1].

For conformance testing of in-channel adjacent subbband leakage, use the modified version of TM1.1 and TM1.2 as the test models.

In-channel adjacent subband blocking

Blocking due to strong inter-sector signals in an adjacent DL subband within the same carrier is another critical issue in SBFD operation. Unlike legacy blocking tests, SBFD blocking occurs due to simultaneous in-channel transmission and reception, and therefore a test model is needed. It is proposed to use a modified version of TM1.1 with the transmitter turned ON to simulate a realistic adjacent subband interferer. More detailed discussion on the modified TM1.1 can be found in [1].

In this test setup, the modified TM1.1 acts as the interfering signal source in the adjacent subband. The receiver performance is then evaluated in the presence of this interferer through sensitivity measurements. This approach reflects the true nature of SBFD operation, where the transmitter and receiver operate simultaneously in adjacent subbands.

- Unlike legacy blocking tests, SBFD blocking may occur due to simultaneous in-channel transmission and reception, and therefore a test model is needed. The test model acts as the interfering signal source in the adjacent subband.

For conformance testing of in-channel adjacent subbband blocking, use the modified version of TM1.1 as the test model.

# Conclusion

In this contribution, discussion on conformance testing of new RF requirements for SBFD-capable BS is presented with the following observations and proposals.

Observation 1	There are four switching cases between SBFD and non-SBFD, and the switching from SBFD to DL or the switching from DL to SBFD will need two test models in each case.

Observation 2	Unlike legacy blocking tests, SBFD blocking may occur due to simultaneous in-channel transmission and reception, and therefore a test model is needed. The test model acts as the interfering signal source in the adjacent subband.

Proposal 1	For conformance testing of the switching from SBFD to DL or the switching from DL to SBFD, use two test models: the existing TM1.1 to represent non-SBFD DL operation, and a modified version of TM1.1 to represent SBFD operation.

Proposal 2	For conformance testing of the switching from SBFD to UL or the switching from UL to SBFD, use one test model, i.e., the modified version of TM1.1.

Proposal 3	For conformance testing of in-channel adjacent subbband leakage, use the modified version of TM1.1 and TM1.2 as the test models.

Proposal 4	For conformance testing of in-channel adjacent subbband blocking, use the modified version of TM1.1 as the test model.

# References

R4-2513967, Impact on SBFD BS RF TX requirements, Ericsson

R4-2513968, Impact on SBFD BS RF RX requirements, Ericsson