3GPP TSG-RAN WG4 Meeting #116bis	R4-2514130

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	On Performance and CSI reporting requirements for 6Rx UE with interference

Agenda item:	7.5.4

Document for:	Discussion

Introduction

It has been agreed in RAN#109 to introduce interference requirements for 6Rx. The following is the work stated in the WID [1]:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

Comparing to the WID from the discussion on 8Rx UE demodulation with interference [2] the described work is quite the same except a clear definition of the number of MIMO layers to be considered.

This effectively means, that defining requirements for 6Rx with interference will likely be very similar to what was agreed to be defined for 8Rx with interference.

Discussion

Since the WID for 6Rx with interference is similar to WID for Rel-18 8Rx with interference, we see it reasonable to use the agreed 8Rx requirements as a starting point for defining interference requirements for 6Rx.

It is given in the WID that the definition of CQI requirements depends on CQI requirements being defined in Rel-19 for 6Rx which is expected to be decided in this meeting. We currently do not see any reason for it not to be agreed to introduce Rel.19 CQI requirements for 6Rx hence in the following we will assume CQI requirements with interference are to be defined for 6Rx, too.

Overall test scope

FDD & TDD

Looking at existing interference requirements from 2Rx/4Rx and also 8Rx interference requirements for both TDD and FDD shall be implemented also for 6Rx.

RAN4 to introduce UE demodulation requirements for both TDD and FDD

PDSCH interference requirements for ATP

In addition, since no requirements for PDSCH absolute physical layer throughput with interference has been introduced in 2Rx/4Rx or 8Rx, we see no reason to cover PDSCH ATP requirements for 6Rx with interference.

Do not cover PDSCH absolute physical layer throughput requirements for 6Rx

Receiver type

As with 2Rx, 4Rx and 8Rx and in alignment with the WID [1] MMSE-IRC receiver needs to be considered.

Consider MMSE-IRC receiver type for 6Rx interference requirements.

FWA and handheld devices

The WID indicates to consider both handheld and FWA devices. In NR_ENDC_RF_Ph4_Demod_6Rx it is still being discussed if one or two sets of requirements are to be defined.

Consider defining 6Rx interference requirements for handheld and FWA devices respectively.

Correlation used in Rel-19 6Rx requirement definition

In NR_ENDC_RF_Ph4_Demod_6Rx the correlation to be used is still being discussed. For 6Rx with interference we have to also consider using the final agreed correlation from NR_ENDC_RF_Ph4_Demod_6Rx.

In NR_ENDC_RF_Ph4_Demod_6Rx the correlation to be used is still being discussed.

When NR_ENDC_RF_Ph4_Demod_6Rx has agreed to a correlation to be used, consider also using the same correlation when defining requirements for 6Rx with interference.

PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

Interference model and power levels

Regarding the interference model we see it reasonable that the same model as used for 2Rx, 4Rx and 8Rx can also be used for 6Rx.

Re-use Rel-17 interference model as baseline with the following:
- Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 inter-cell interference requirements for 2Rx,4Rx and 8Rx.
- Reuse the same interference signal modulation order as 2Rx, 4Rx and 8Rx inter-cell interference requirements, i.e., random 16QAM symbols.

Configurations used for initial simulation alignment

Since the introduced requirements for 8Rx with inter-cell interference has fairly high SNR defined, we see re-use of the configurations agreed to for 8Rx with inter-cell interference to also be a good starting point for simulation analysis in the 6Rx with inter-cell interference case.

Therefor we propose to reuse the configurations from 8Rx inter-cell interference requirements as starting point

Use the configuration provided in below table as starting point for initial simulation alignment with inter-cell interference. If alignment cannot be achieved further extension of the simulation coverage can be discussed.

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank | Rank 2 (HomNet – 2 interference cells) Rank 4 (HetNet – 1 interference cell) |
| Propagation conditions | TDLA30-10 for all configurations with rank 2 and rank 4 |
| MCS | Rank 2: MCS17 Rank 4: MCS13 |
| Antenna and MIMO correlation | Rank 2: 2T6R ULA Low Rank 2: 2T6R ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. Rank 4: 4T6R ULA Low Rank 4: 4T6R ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. |
| Interference cell | 2Tx |
| Frequency shift to the target cell | 300Hz for interference cell 1 100Hz for interference cell 2 |
| SSB Configuration | Same as Rel-17 |
| Transmission rank | Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Requirements to be defined

If alignment is met across companies’ simulation results, requirements can be defined for all above cases proposed in section 2.2.3.

- As PDSCH requirements are defined in 8Rx with inter-cell interference for all our proposed simulation configurations, PDSCH requirements shall also be defined for the same configurations in 6Rx with inter-cell interference.

Define PDSCH requirements with inter-cell interference for all previous proposed configurations. Due to testload it can be considered to down select to at least one antenna correlation for each rank configuration as long as FWA and handheld devices are covered.

CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

The WID states that CQI requirements are only to be introduced in case it is agreed to introduce CQI requirements for 6Rx SU-MIMO. Since the agreement to introduce such requirements are pending the outcome of RAN4#116bis. The following contains our proposals which will be relevant in case 6Rx SU-MIMO requirements are introduced.

Test metric

We see no reason to change the test metric for 6Rx from what was already used in 2Rx, 4Rx and 8Rx CQI requirements with inter-cell interference.

Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx:

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Configurations used for initial simulation alignment

Since the introduced CQI requirements for 8Rx with inter-cell was done with reasonable high gain, we see re-use of the configurations agreed to for 8Rx CQI with inter-cell interference to also be a good starting point for simulation analysis in the 6Rx CQI with inter-cell interference case.

Therefor we propose to reuse the configurations from 8Rx CQI inter-cell interference requirements as starting point.

Use the configuration provided in below table as starting point for initial CQI simulation alignment with inter-cell interference. If alignment cannot be achieved further extension of the simulation coverage can be discussed.

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank for serving cell | Rank 2 |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | 2T6R with ULA Low 2T6R with ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Requirements to be defined

If alignment is met across companies’ simulation results, requirements can be defined for all above cases proposed in section 2.3.2.

As CQI requirements are defined in 8Rx with inter-cell interference for all our proposed simulation configurations, CQI requirements shall also be defined for the same configurations for 6Rx with inter-cell interference.

Define CQI requirements with inter-cell interference for all previous proposed configurations. Due to testload it can be considered to down select to at least one antenna correlation as long as FWA and handheld devices are covered.

PDSCH demodulation with Intra-cell inter-user interference

Intra-cell inter-user interference model

For the intra-cell inter-user interference model the same configuration can be used as was used for 2Rx, 4Rx and 8Rx with intra-cell inter-user interference.

Use the following for intra-cell inter-user interference model:
- Beamforming model: MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4
- MO of paired UE: Random 16QAM

Number of coUEs

In the discussions to introduce intra-cell inter-user interference requirements for 8Rx, it was discussed if more than 1 coUE was to be considered. The outcome of the discussion was that there were not support for introducing requirements with 2 coUEs due to the view from some companies, that it was not likely to be seen in deployment. Nokia did not agree to that statement but since there were no consensus reached the agreement was to introduce requirements for 1 coUE only.

For 6Rx it would also here be possible to define requirements for a configuration with 2 coUEs (i.e., a 2+2+2 configuration). We would of course prefer to have such configuration introduced, however based on the previous discussions in 8Rx we do not see it possible to reach consensus. Due to this, we will not push to have 2 coUE for 6Rx.

Based on the discussions from 8Rx intra-cell inter-user interference we do not see it possible to achieve consensus to introduce requirements with more than 1 coUE.

Consider 1 coUE for introducing PDSCH intra-cell inter-user requirements.

Configurations used for initial simulation alignment

Since the introduced requirements for 8Rx with intra-cell inter-user interference has fairly high SNR defined, we see re-use of the configurations agreed to for 8Rx with intra-cell inter-user interference to also be a good starting point for simulation analysis in the 6Rx case.

Therefor we propose to reuse the configurations from 8Rx inter-cell interference requirements as starting point

Use the configuration provided in below table as starting point for initial PDSCH simulation alignment with intra-cell inter-user interference. If alignment cannot be achieved further extension of the simulation coverage can be discussed.

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank | Rank 2 + 2 |
| Pre-coder | For co-UE: rank 2+2 orthogonal. For orthogonal precoding: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH Propagation conditions: |
| Propagation conditions | TDLA30-10 |
| MCS | MCS17 |
| Antenna and MIMO correlation | 4T6R ULA Low for rank 2+2 4T6R ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx for rank 2+2 |
| PDSCH allocation in frequency domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| PDCCH and PDSCH allocation in time domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| Signal power assumptions | Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal |
| SNR assumptions | SNR = (STargetUE+ SInterfUE)/N |
| Codebook type | Type I Single Panel only 4Tx: (N1, N2) = (2,1) |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Requirements to be defined

If alignment is met across companies’ simulation results, requirements can be defined for all above cases proposed in section 2.4.3.

As PDSCH requirements are defined in 8Rx with intra-cell inter-user interference for all our proposed simulation configurations, PDSCH requirements shall also be defined for the same configurations in 6Rx with intra-cell inter-user interference.

Define PDSCH requirements with intra-cell inter-user interference for all previous proposed configurations. Due to testload it can be considered to down select to at least one antenna correlation as long as FWA and handheld devices are covered.

Release independency

As the first interference requirements were introduced in Rel-17 and the newly added 8Rx interference requirements were agreed to be release independent from Rel-17, we do not see a reason why introduced interference requirements for 6Rx should not also be release independent from Rel-17

All new 6Rx interference requirements introduced shall be release independent from Rel-17.

Applicability rules

Test applicability to skip 2/4Rx requirements

In 8Rx intra-cell inter-user interference discussion applicability rules were introduced which enabled 8Rx UEs passing rank 2 requirements to not also have to pass 4Rx rank 2 requirements. The same applicability rule can be implemented for 6Rx intra-cell inter-user interference requirements.

For 8Rx Inter-cell interference no applicability rules were introduced so also for 6Rx inter-cell interference we do not see the need to introduce applicability rules.

Agreements in test applicability for 2/4Rx from 8Rx can be re-used for 6Rx.

Intra-cell Inter-user interference: Introduce applicability rules for intra-cell inter-user interference so 6Rx UE which pass rank 2 requirements do not have to pass 4Rx rank 2 requirements.
Inter-cell interference: Do not introduce applicability rules for inter-cell interference.

FDD vs. TDD

In 8Rx discussion it was agreed to introduce applicability rules to reduce the overall test load and we see it reasonable to use the same approach for 6Rx inter-cell interference to ensure similar coverage of FDD and TDD as for 8Rx.

We see it reasonable to introduce similar applicability rules between FDD and TDD as was agreed for 8Rx in Rel-19 also for 6Rx inter-cell interference requirements to reduce test load.

Introduce applicability rules such that if UE supporting both TDD and FDD with 6Rx, UE will pass test cases with 2 interference cells (HomNet scenario) with FDD mode, and pass test cases with 1 interference cell (HetNet scenario) with TDD mode.

Conclusion

The following Observations and Proposals were made:

Overall test scope

FDD & TDD

- RAN4 to introduce UE demodulation requirements for both TDD and FDD

PDSCH interference requirements for ATP

Do not cover PDSCH absolute physical layer throughput requirements for 6Rx

Receiver type

Consider MMSE-IRC receiver type for 6Rx interference requirements.

FWA and handheld devices

Consider defining 6Rx interference requirements for handheld and FWA devices respectively.

Correlation used in Rel-19 6Rx requirement definition

- In NR_ENDC_RF_Ph4_Demod_6Rx the correlation to be used is still being discussed.

When NR_ENDC_RF_Ph4_Demod_6Rx has agreed to a correlation to be used, consider also using the same correlation when defining requirements for 6Rx with interference.

PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

Interference model and power levels

Re-use Rel-17 interference model as baseline with the following:
- Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 inter-cell interference requirements for 2Rx,4Rx and 8Rx.
- Reuse the same interference signal modulation order as 2Rx, 4Rx and 8Rx inter-cell interference requirements, i.e., random 16QAM symbols.

Configurations used for initial simulation alignment

Use the configuration provided in below table as starting point for initial simulation alignment with inter-cell interference. If alignment cannot be achieved further extension of the simulation coverage can be discussed.

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank | Rank 2 (HomNet -2 interference cells) Rank 4 (HetNet – 1 interference cell) |
| Propagation conditions | TDLA30-10 for all configurations with rank 2 and rank 4 |
| MCS | Rank 2: MCS17 Rank 4: MCS13 |
| Antenna and MIMO correlation | Rank 2: 2T6R ULA Low Rank 2: 2T6R ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. Rank 4: 4T6R ULA Low Rank 4: 4T6R ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. |
| Interference cell | 2Tx |
| Frequency shift to the target cell | 300Hz for interference cell 1 100Hz for interference cell 2 |
| SSB Configuration | Same as Rel-17 |
| Transmission rank | Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Requirements to be defined

As PDSCH requirements are defined in 8Rx with inter-cell interference for all our proposed simulation configurations, PDSCH requirements shall also be defined for the same configurations in 6Rx with inter-cell interference.

Define PDSCH requirements with inter-cell interference for all previous proposed configurations. Due to testload it can be considered to down select to at least one antenna correlation for each rank configuration as long as FWA and handheld devices are covered.

CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

Test metric

Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx:

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Configurations used for initial simulation alignment

Use the configuration provided in below table as starting point for initial CQI simulation alignment with inter-cell interference. If alignment cannot be achieved further extension of the simulation coverage can be discussed.

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank for serving cell | Rank 2 |
| Propagation conditions | TDLA30-5 for the serving cell AWGN for the interference cell |
| Antenna and MIMO correlation for serving cell | 2T6R with ULA Low 2T6R with ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx. |
| Antenna configuration for the interference cell: | 1T6R |
| Interference power level | INR = 10.04dB for the serving cell. |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

Requirements to be defined

As CQI requirements are defined in 8Rx with inter-cell interference for all our proposed simulation configurations, CQI requirements shall also be defined for the same configurations for 6Rx with inter-cell interference.

Define CQI requirements with inter-cell interference for all previous proposed configurations. Due to testload it can be considered to down select to at least one antenna correlation as long as FWA and handheld devices are covered.

PDSCH demodulation with Intra-cell inter-user interference

Intra-cell inter-user interference model

Use the following for intra-cell inter-user interference model:
- Beamforming model: MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4
- MO of paired UE: Random 16QAM

Number of coUEs

Based on the discussions from 8Rx intra-cell inter-user interference we do not see it possible to achieve consensus to introduce requirements with more than 1 coUE.

Consider 1 coUE for introducing PDSCH intra-cell inter-user requirements.

Configurations used for initial simulation alignment

Use the configuration provided in below table as starting point for initial PDSCH simulation alignment with intra-cell inter-user interference. If alignment cannot be achieved further extension of the simulation coverage can be discussed.

| Parameter | Value |
| --- | --- |
| SCS/CBW | FDD: 15kHz/10MHz TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |
| Rank | Rank 2 + 2 |
| Pre-coder | For co-UE: rank 2+2 orthogonal. For orthogonal precoding: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH Propagation conditions: |
| Propagation conditions | TDLA30-10 |
| MCS | MCS17 |
| Antenna and MIMO correlation | 4T6R ULA Low for rank 2+2 4T6R ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx for rank 2+2 |
| PDSCH allocation in frequency domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| PDCCH and PDSCH allocation in time domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| Signal power assumptions | Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal |
| SNR assumptions | SNR = (STargetUE+ SInterfUE)/N |
| Codebook type | Type I Single Panel only 4Tx: (N1, N2) = (2,1) |
| Other parameters | For other parameters re-use the same configuration as used in Rel-17 requirements. |

As PDSCH requirements are defined in 8Rx with intra-cell inter-user  interference for all our proposed simulation configurations, PDSCH requirements shall also be defined for the same configurations in 6Rx with intra-cell inter-user interference.

Define PDSCH requirements with intra-cell inter-user interference for all previous proposed configurations. Due to testload it can be considered to down select to at least one antenna correlation as long as FWA and handheld devices are covered.

Release independency

All new 6Rx interference requirements introduced shall be release independent from Rel-17.

Applicability rules

Test applicability to skip 2/4Rx requirements

Agreements in test applicability for 2/4Rx from 8Rx can be re-used for 6Rx.

Intra-cell Inter-user interference: Introduce applicability rules for intra-cell inter-user interference so 6Rx UE which pass rank 2 requirements do not have to pass 4Rx rank 2 requirements.
Inter-cell interference: Do not introduce applicability rules for inter-cell interference.

FDD vs. TDD

We see it reasonable to introduce similar applicability rules between FDD and TDD as was agreed for 8Rx in Rel-19 also for 6Rx inter-cell interference requirements to reduce test load.

Introduce applicability rules such that if UE supporting both TDD and FDD with 6Rx, UE will pass test cases with 2 interference cells (HomNet scenario) with FDD mode, and pass test cases with 1 interference cell (HetNet scenario) with TDD mode.

- References

- RP-252957 WID: NR demodulation performance: Phase 6, RAN#109, September 2025

- RP-241297, “NR demodulation performance: Phase 5”, RAN#104, June 2024