3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514166

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Simulation results for L1-RSRP and L1-SINR accuracy

Source: 	Huawei, HiSilicon

Agenda item:	6.13.3.2

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#116, and the outcomes are captured in WF [1]. In particular, it is agreed to define relaxed accuracy requirements based on 24 RB for Case 3 of CSI-RS L1 measurement, where CSI-RS is configured across 2 DL subbands and the CSI-RS BW in each DL subbabd is >= 24 RB. The simulation assumption is approved in the Annex of [2].

In this paper, we will provide our simulation results for L1-RSRP and L1-SINR accuracy based on 24 RB.

Discussion

L1-RSRP

The L1-RSRP simulation is based on assumption in Table 1.

Table 1: simulation assumption for L1-RSRP

| Parameters | Value |
| --- | --- |
| SCS | 15 kHz, 30 kHz, 120 kHz |
| Side condition (SNR) | -3 dB |
| Density (D) | 3 |
| Number of samples (M) | 1 (single sample measurement) |
| Number of PRB | 24, 48 |
| Propagation condition | AWGN |

The simulation results for L1-RSRP is in Table 2.

Table 2: Simulation results for L1-RSRP error

| SCS (kHz) | RB number | L1-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 95% | Max(5%,95%) |
| 15 | 24 | -1.95 | 1.16 | 1.95 |
|  | 48 | -1.18 | 0.83 | 1.18 |
| 30 | 24 | -1.51 | 1.12 | 1.51 |
|  | 48 | -1.04 | 0.83 | 1.04 |
| 120 | 24 | -1.60 | 1.09 | 1.60 |
|  | 48 | -1.14 | -0.82 | 1.14 |

L1-SINR

Case 1A: CMR only

The L1-SINR simulation with CMR only (Case 1A) is based on assumption in Table 3.

Table 3: simulation assumption for L1-SINR with CMR only (Case 1A)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | N/A |
| Side condition (SNR) on CMR | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1 |
| Number of PRBs | 24, 48 |
| Propagation condition | AWGN |

The simulation results for L1-SINR with CMR only is in Table 4.

Table 4: Simulation results for L1-SINR with CMR only (Case 1A)

| SCS (kHz) | RB number | L1-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 95% | Max(5%,95%) |
| 15 | 24 | -1.78 | 1.43 | 1.78 |
|  | 48 | -1.31 | 1.01 | 1.31 |
| 30 | 24 | -1.84 | 1.44 | 1.84 |
|  | 48 | -1.21 | 1.01 | 1.21 |
| 120 | 24 | -1.78 | 1.36 | 1.78 |
|  | 48 | -1.19 | 1.01 | 1.19 |

Case 2A: SSB as CMR and ZP-IMR

The L1-SINR simulation with SSB as CMR and ZP-IMR (Case 2A) is not conducted because the reduction of BW from 48 to 24 RB is only applicable to CSI-RS (NZP-IMR). UE should be able to measure interference on the ZP-IMR on 48 RB across 2 DL subbands and meet the existing accuracy.

Case 2C: SSB as CMR and NZP-IMR

The L1-SINR simulation with SSB as CMR and NZP-IMR (Case 2C) is based on assumption in Table 5.

Table 5: simulation assumption for L1-SINR with SSB as CMR and NZP-IMR (Case 2C)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | 0dB |
| Side condition (SNR) on IMR | 0dB |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1 |
| Number of PRBs for IMR | 24, 48 |
| Propagation condition | AWGN |

The simulation results for L1-SINR with SSB as CMR and NZP-IMR (Case 2C) is in Table 6.

Table 6: Simulation results for L1-SINR with SSB as CMR and NZP-IMR (Case 2C)

| SCS (kHz) | RB number | L1-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 95% | Max(5%,95%) |
| 15 | 24 | -1.28 | 0.96 | 1.28 |
|  | 48 | -0.83 | 0.70 | 0.83 |
| 30 | 24 | -1.13 | 0.90 | 1.13 |
|  | 48 | -0.87 | 0.64 | 0.87 |
| 120 | 24 | -1.18 | 1.00 | 1.18 |
|  | 48 | -0.83 | 0.70 | 0.83 |

Case 2B: CSI-RS as CMR and ZP-IMR

The L1-SINR simulation with CSI-RS as CMR and ZP-IMR (Case 2B) is based on assumption in Table 7.

Table 7: simulation assumption for L1-SINR CSI-RS as CMR and ZP-IMR (Case 2B)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | 0dB |
| Side condition (SNR) on IMR | 0dB |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1 |
| Number of PRBs for IMR | 24, 48 |
| Propagation condition | AWGN |

The simulation results for L1-SINR with CSI-RS as CMR and ZP-IMR (Case 2B) is in Table 8.

Table 8: Simulation results for L1-SINR with CSI-RS as CMR and ZP-IMR (Case 2B)

| SCS (kHz) | RB number | L1-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 95% | Max(5%,95%) |
| 15 | 24 | -1.35 | 0.99 | 1.35 |
|  | 48 | -0.93 | 0.75 | 0.93 |
| 30 | 24 | -1.27 | 0.97 | 1.27 |
|  | 48 | -0.93 | 0.71 | 0.93 |
| 120 | 24 | -1.31 | 1.04 | 1.31 |
|  | 48 | -0.96 | 0.72 | 0.96 |

Case 2D: CSI-RS as CMR and NZP-IMR

The L1-SINR simulation with CSI-RS as CMR and NZP-IMR (Case 2D) is based on assumption in Table 9.

Table 9: simulation assumption for L1-SINR CSI-RS as CMR and NZP-IMR (Case 2D)

The simulation results for L1-SINR with CSI-RS as CMR and NZP-IMR (Case 2D) is in Table 10.

Table 10: Simulation results for L1-SINR with CSI-RS as CMR and NZP-IMR (Case 2D)

| SCS (kHz) | RB number | L1-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 95% | Max(5%,95%) |
| 15 | 24 | -0.89 | 0.88 | 0.89 |
|  | 48 | -0.82 | 0.77 | 0.82 |
| 30 | 24 | -0.91 | 0.85 | 0.91 |
|  | 48 | -0.88 | 0.72 | 0.88 |
| 120 | 24 | -0.85 | 0.86 | 0.86 |
|  | 48 | -0.85 | 0.83 | 0.85 |

Conclusions

In this paper we provided our simulation results for L1-RSRP and L1-SINR accuracy based on 24 RB.

Observation: The accuracy degradation of 24 RB compared to 48 RB is around

- 0.57 dB for L1-RSRP

- 0.56 dB for L1-SINR with CMR only (Case 1A)

- 0 dB for L1-SINR with SSB as CMR and ZP-IMR (Case 2A)

- 0.35 dB for L1-SINR with SSB as CMR and NZP-IMR (Case 2C)

- 0.37 dB for L1-SINR with CSI-RS as CMR and ZP-IMR (Case 2B)

- 0.04 dB for L1-SINR with CSI-RS as CMR and NZP-IMR (Case 2D)

Reference

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon