3GPP TSG-RAN WG4 Meeting #116bis	R4-2514256

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	LG Electronics

Title:	Discussion on co-existence evaluation cases and assumption for A-IoT outdoor scenario

Agenda item:	7.11.3

Document for:	Discussion

Introduction

In 3GPP RAN #109, SID[1] was approved. In this paper, we provide our views on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Discussion

Simulation cases

In SID, it assume A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. SID was noted as follow.

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes:

There are 4 kind of interference for Rel-20 AIoT. WID assume FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.We can see as follow figure.

Fig1. Evaluation cases for Rel-20 A-IoT SID

When it consider above that, it could summarize the evaluation cases as follow. And Rel-20 A-IoT operation assume in-band and standalone operation. And SID also include standalone and inband operation. So RAN4 need to consider both operation. Based on above figure, A-IoT BS interfere to NR UE when transmit R2D in DL spectrum likewise macro NR BS also interfere to A-IoT device. In the aspect of A-IoT device, A-IoT device interfere to  NR macro BS also when NR transmit UL, it interfere to A-IoT BS in UL spectrum.

Table1. Simulation cases for Rel-20 A-IoT outdoor scenario

| Case No. | Direction | Aggressor | Victim | Mode |
| --- | --- | --- | --- | --- |
| e | D2R | A-IoT Device | NR UL(BS) | Standalone and inband |
| f | D2R | NR UL(UE) | Reader | Standalone and inband |
| c | R2D | NR DL(BS) | A-IoT Device | Standalone and inband |
| d | R2D | Reader | NR DL(UE) | Standalone and inband |

Proposal 1: Evaluation cases should be decided as follow table

Table1. Simulation cases for Rel-20 A-IoT outdoor scenario

| Case No. | Direction | Aggressor | Victim | Mode |
| --- | --- | --- | --- | --- |
| e | D2R | A-IoT Device | NR UL(BS) | Standalone and inband |
| f | D2R | NR UL(UE) | Reader | Standalone and inband |
| c | R2D | NR DL(BS) | A-IoT Device | Standalone and inband |
| d | R2D | Reader | NR DL(UE) | Standalone and inband |

Deployment parameters

RAN4 should decide simulation parameter for Co-existence evaluation. We suggest follow table for simulation assumption.

Proposal 2: Deployment parameters should be decided as follow table2

Table2. Deployment parameters

| Parameter | D4T1 |
| --- | --- |
| Carrier frequency | 900 MHz (Band n8) |
| BW for NR | 10MHz with 15KHz SCS |
| R2D BW | R2D：180kHz, for 15kHz SCS |
| D2R BW | 15kHz |
| Waveform | R2D: OOK D2R: BPSK |
| A-IoT DL power control | No |
| A-IoT UL power control | FFS Need to consider follow agreement from RAN1(R1-2506567) Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects - Open loop and/or closed loop power control - Required measurements (e.g., signal strength) - Required information at the device - Required information at reader side - Signaling overhead and power consumption |
| Traffic model | Full buffer |
| NR BS deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around,. Inter-NR BS distance: 750m NR BS height: 25m |
| A-IoT reader deployment | frequency reuse 1:3(same with NR BS) |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 50m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| NR BS Inter-site distance | MCL of 70 dB |
| Pathloss model | UMa NLOS defined in TR38.901 Penetration loss：FFS It need to consider agreement of RAN1(R1-2506567) •For both R2D and D2R oX = {0, 20, 80} dB •Note: Example use cases for the values above: o20 dB for a Device inside a car with metallized car windows o80 dB for a Device inside a metallic shipping container oCompanies can report additional example use cases Shadow fading margin：For UMa NLOS: 6dB |

A-IoT reader and device  RF parameters

Based on RAN1 agreement, A-IoT reader and device should be decided as follow table.

Proposal 3: simulation parameter related A-IoT reader and device should be decided as follow table3 and table4

Table3. A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | standalone: 43 dBm in-band：38dBm |
| A-IoT reader receiver Noise Figure (dB) | 5 |
| A-IoT reader antenna gain (dBi) | 17 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB with 100kHz offset) |
| ACS | FFS |
| Antenna configuration | Same as legacy NR BS |

Table4. A-IoT device RF parameters

| A-IoT device parameters | Device 2b | Device C |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0dBi | 0dBi |
| A-IoT device output power（dBm） | -10dBm | -3 or 0 dBm |
| A-IoT device noise figure (dB) | 15dB | 13 dB |
| Guard band | [0PRB] | [0PRB] |
| ACLR | FFS | FFS |
| ACS | FFS | FFS |

NR macro BS and UE RF parameters for co-existence evaluation

NR macro BS and UE RF parameters could be reused with TR38.769.

Proposal 4: NR macro BS and UE RF parameter should be reused in TR38.769 as follow table5 and table6.

Table5. NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25(UMa) |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

Table6. NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

- 2. Conclusion

In this contribution, we want to share some views on ambient IoT co-existence evaluioatn cases and simulation assumption and proposals are made as following:

Proposal 1: Evaluation cases should be decided as follow table

Table1. Simulation cases for Rel-20 A-IoT outdoor scenario

| Case No. | Direction | Aggressor | Victim | Mode |
| --- | --- | --- | --- | --- |
| e | D2R | A-IoT Device | NR UL(BS) | Standalone and inband |
| f | D2R | NR UL(UE) | Reader | Standalone and inband |
| c | R2D | NR DL(BS) | A-IoT Device | Standalone and inband |
| d | R2D | Reader | NR DL(UE) | Standalone and inband |

Proposal 2: Deployment parameters should be decided as follow table2

Table2. Deployment parameters

| Parameter | D4T1 |
| --- | --- |
| Carrier frequency | 900 MHz (Band n8) |
| BW for NR | 10MHz with 15KHz SCS |
| R2D BW | R2D：180kHz, for 15kHz SCS |
| D2R BW | 15kHz |
| Waveform | R2D: OOK D2R: BPSK |
| A-IoT DL power control | No |
| A-IoT UL power control | FFS Need to consider follow agreement from RAN1(R1-2506567) Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects - Open loop and/or closed loop power control - Required measurements (e.g., signal strength) - Required information at the device - Required information at reader side - Signaling overhead and power consumption |
| Traffic model | Full buffer |
| NR BS deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around,. Inter-NR BS distance: 750m NR BS height: 25m |
| A-IoT reader deployment | frequency reuse 1:3(same with NR BS) |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 50m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| NR BS Inter-site distance | MCL of 70 dB |
| Pathloss model | UMa NLOS defined in TR38.901 Penetration loss：FFS It need to consider agreement of RAN1(R1-2506567) •For both R2D and D2R oX = {0, 20, 80} dB •Note: Example use cases for the values above: o20 dB for a Device inside a car with metallized car windows o80 dB for a Device inside a metallic shipping container oCompanies can report additional example use cases Shadow fading margin：For UMa NLOS: 6dB |

Proposal 3: Simulation parameter related A-IoT reader and device should be decided as follow table3 and table4

Table3. A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | standalone: 43 dBm in-band：38dBm |
| A-IoT reader receiver Noise Figure (dB) | 5 |
| A-IoT reader antenna gain (dBi) | 17 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB with 100kHz offset) |
| ACS | FFS |
| Antenna configuration | Same as legacy NR BS |

Table4. A-IoT device RF parameters

| A-IoT device parameters | Device 2b | Device C |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0dBi | 0dBi |
| A-IoT device output power（dBm） | -10dBm | -3 or 0 dBm |
| A-IoT device noise figure (dB) | 15dB | 13 dB |
| Guard band | [0PRB] | [0PRB] |
| ACLR | FFS | FFS |
| ACS | FFS | FFS |

Proposal 4: NR macro BS and UE RF parameter should be reused in TR38.769 as follow table5 and table6.

Table5. NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25(UMa) |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

Table6. NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

- 3. Reference

RP-252964, “Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices” LG Electronics, RAN#109, Aug. 2025.

[2]  TR38.769, Study on solutions for ambient IoT (Internet of Things) V2.0.0

[3]  R1-2506567, Session notes for 10.3 Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, RAN1#122, Aug,2025

[4] R1-2506575, LS on BS Tx power assumption for coverage evaluation in A-IoT,RAN1#122,Aug, 2025