3GPP TSG-RAN4 Meeting #116-bis	R4-2514266

# Prague, Czechia, 13th Oct 2025 - 17th Oct 2025

# Agenda item:			7.3.2

# Source:					Eutelsat Group

# Title:						Scoping of VSAT testability study

# Document for:		Information

# Overview

This paper proposes additional details from an NTN operator perspective for the scoping of the new study on test methods for VSAT agreed at RAN #109 in [1].

.

# Discussion

The new WID in [1] defined the objects as:

Objectives for this core part work item are as follows - Define enhanced test methodologies to cover NTN Ka and Ku bands - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority - FR1 OTA test method extension is not precluded, if also applicable - Study whether and how to support circular polarization measurements - Linear polarization is the baseline - Specify the required quiet zone to cover VSAT device type based on industry needs - 60cm QZ as a starting point - Specify preliminary MU assessment for test methodologies

## Antenna aperture

The initial target is for a 60 cm quiet zone as a starting point. A typical VSAT for LEO uses a 45 x 45 cm flat panel antenna which would require a QZ of 63.6 cm.

Proposal 1: To facilitate applicability to the most common small VSAT type it is proposed to use a 45 cm x 45 cm antenna aperture requiring a 63.6 cm QZ.

Other common antenna apertures are 20 x 20, 35 x 35, 65 x 65 and 78 x 78 requiring QZ of 28.3, 49.5, 91.9 and 110.3 cm.

## Device size and weight

| Antenna dimension cm | Antenna height cm | Overall device size | Device weight |
| --- | --- | --- | --- |
| 20 x 20 | 2 | To be confirmed | < 1 kg |
| 35 x 35 | 4 | To be confirmed | < 3 kg |
| 45 x 45 | 4 | To be confirmed | < 6 kg |
| 65 x 65 | 4 | To be confirmed | < 10 kg |

A 78 x 78 aperture is also used for GSO connectivity

Details on overall VSAT size will be provided. Due to the size of the devices, white box testing with manufacturer declaration of antenna location will be a likely solution.

## Beam pointing and measurement accuracy

Typically, 0.01° in Phi  and 0.002° in theta. 2. Measurement accuracy < 0.5dB as a target  (+/- 0.25dB). The extent of beam pointing performance testing requires further discussion as this is linked to ephemeris data and VSAT propagator performance.

Proposal 2: The extent of beam pointing performance testing requires further discussion as this is linked to ephemeris data and VSAT propagator performance.

## Beam steering in the test system

In normal operation the VSAT will steer its beam towards the satellite based on the ephemeris data and VSAT propagator. In an OTA test system, it is not expected that this will be emulated as the SAN probe antenna will be fixed in space which is not a real scenario. The consequence is that for test purposes, the VSAT is likely to be locked on boresight and manually positioned to point towards the SAN probe antenna.

Proposal 3: For test purposes, the VSAT shall be locked on boresight and manually positioned to point towards the SAN probe antenna.

## Pointing stability

Proposal 4: Pointing stability should be considered as an operational requirement of the system and not be tested in 3GPP.

## Antenna gain and pattern

Antenna gain is considered an operational requirement however; it is a regulatory requirement to measure the antenna pattern. The regulatory requirements apply the UL and DL.

Proposal 5: Antenna pattern is required to be measured in the UL and DL for regulatory purposes.

For practical reasons due to lack of RF connectors, it is not possible to measure the antenna gain directly, but only via the L or S band interface between the modem and upconverting antenna system.

Proposal 6: Due to lack of RF connectors, the antenna gain can only be measured via the L or S band modem port.

## Spectrum mask

Proposal 7: The spectrum mask should be assessed at the DUT level using EIRP and not on the antenna gain.

## Polarization

The use of linear polarization is limited to a few GEO cases. For all practical purpose circular polarization is the operational assumption. Supporting CP using existing LP test systems as the baseline is necessary.

Proposal 8: Circular polarization is essential for the main use cases and should be the priority.

# Summary

Proposal 1: To facilitate applicability to the most common small VSAT type it is proposed to use a 45 cm x 45 cm antenna aperture requiring a 63.6 cm QZ.

Proposal 2: The extent of beam pointing performance testing requires further discussion as this is linked to ephemeris data and VSAT propagator performance.

Proposal 3: For test purposes, the VSAT shall be locked on boresight and manually positioned to point towards the SAN probe antenna.

Proposal 4: Pointing stability should be considered as an operational requirement of the system and not be tested in 3GPP.

Proposal 5: Antenna pattern is required to be measured in the UL and DL for regulatory purposes.

Proposal 6: Due to lack of RF connectors, the antenna gain can only be measured via the L or S band modem port.

Proposal 7: The spectrum mask should be assessed at the DUT level using EIRP and not on the antenna gain.

Proposal 8: Circular polarization is essential for the main use cases and should be the priority.

# References

[1] RP-252903 “WID: Enhancement of UE OTA test method and requirements for NR”, RAN VC (Vivo)