3GPP TSG-RAN WG4 Meeting #116bis	R4-2514287

Prague, Czech Republic, October 13 – 17, 2025

Source:	Nokia

Title:	On PDSCH Requirements for Non-Collocated Intra-Band EN-DC/NR-CA Phase 2

Agenda item:	6.2.3

Document for:	Discussion

Introduction

The description of the normative works on the Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2 is documented in [1]. It is driven from operators’ perspective that UE requirements for non-co-located deployment are essential to enhance EN-DC/NR-CA available areas [1]. Specifically, for Rel-19, RAN4 needs to work on the UE requirements for Type 4 FWA UE.

The objective of the performance part of the Work Item (WI) is described in [1] as follows:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify the corresponding performance requirements for type 4a/4b capable FWA UE. - RRM performance requirements - PDSCH demodulation performance requirements

In this contribution, we provide our initial views on the PDSCH requirements for Type 4 FWA UE to support non-collocated intra-band EN-DC/NR-CA deployment.

Discussion

In this Section, the UE types will first be briefly described to provide background information on Type 4 UE and the differences to other UE types. Afterwards, observations and proposals for the corresponding PDSCH requirements for Type 4 FWA UE will follow.

UE Types

The UE types are defined in [2] as follows:

Table 1. UE Types [2]

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

As shown in Table 1 above, the classification on the UE types when supporting EN-DC/NR-CA deployments are based on the number of antennas and the power imbalance, with additional frequency separation between the 2 CCs. Comparing Type 2 and Type 4, for example, the difference on the number of antennas, is 4 for Type 2 and 6 or 8 for Type 4a or 4b, respectively.

PDSCH Requirements

The requirements for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for 2-layer case were specified for Type 2 UE in Rel-18 [1]. In Rel-19, to better support the intra-band non-collocated EN-DC/NR-CA deployment scenarios, the requirements for Type 4 FWA UE need to be specified.

The following objective of the core part WI from [1] can be used as a reference for defining the corresponding PDSCH requirements:

### Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Prioritize the following scenario of the number of CCs for Type 4a/4b - Non-collocated EN-DC - B42: multiple contiguous CCs, all collocated - Based on the requested band combinations - n77/n78: one CC - Non-collocated NR-CA - n77/n78: Non-contiguous two CCs, non-collocated - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

While the objective of the performance part in [1], as shown in Section 1, is only stating “Specify the corresponding performance requirements for type 4a/4b capable FWA UE: PDSCH demodulation performance requirements”, one may refer to the revised WID from Rel-18 Support of intra-band non-collocated EN-DC/NR-CA deployment in [3] which provides more information on the targeted PDSCH requirements as shown below:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Phase II: - Phase II work will get started after the feasibility in phase I is confirmed - Specify PDSCH demodulation requirements for non-collocated scenarios for intra-band non-contiguous NR-CA - Define PDSCH demodulation performance requirement based on the applicable MRTD and power imbalance values.

The focus on Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2 demodulation performance work would be on specifying PDSCH demodulation requirements for NR-CA based on the applicable MRTD and power imbalance as defined in the objective of Core part WI shown above. The assumptions are up-to 4-layer DL MIMO per CC, MRTD>CP and RF power imbalance of up to 25dB.

As in Rel-18 the requirements for Type 2 UE have been defined, and the main difference between Type 4 FWA UE and Type 2 UE is the number of antennas (which relate also to the number of layers), for Rel-19, a higher number of layers should be considered for Type 4 FWA UE. Referring to TS 38.101-4 [4] Clause 5.2A.2.6, for 2Rx case, the following parameters are used: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for lower power carrier and 256QAM with code rate 0.69 for the higher power carrier.

One needs to note here that the 25dB power difference is hard on the TEs. Hence, AWGN channels are used to get enough power out of them. The traditional AWGN itself may not be MIMO/multi-layer compatible, and RAN4 needs to further check and discuss this issue.

The objective of the core part provides the following assumptions: up-to 4-layer DL MIMO per CC, MRTD>CP and up to 25 dB power imbalance.

With power imbalance of up to 25 dB, each CC can have different MCS and different number of layers.

For 2Rx case, TS 38.101-4 Clause 5.2A.2.6 uses the following parameters: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for lower power carrier and 256QAM with code rate 0.69 for the higher power carrier

Type 4a FWA UE has 4 antennas for CC#1 and 2 antennas for CC#2, while Type 4b FWA UE has 4 antennas for both CCs.

The 25dB power difference is hard on the TEs. While AWGN channels can be used to get enough power out of them, they may not be MIMO/multi-layer compatible.

RAN4 to specify PDSCH requirements for NR-CA with 2 CCs with different MCSs and number of layers for the 2 CCs to reflect the power imbalance, in which at least one of the CC shall take the maximum possible number of layers.

The following parameters can be used as a starting point: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for the lower power carrier and 256QAM with code rate 0.69 for the higher power carrier.

In the case of Type 4a FWA UE, the lower power carrier is assumed to use the lower number of Rx antennas, and, hence, the lower number of layers.

RAN4 to discuss how to enable up-to 4-layer if AWGN channels are used.

Conclusion

In this contribution, we provide our views on Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2. The following Observations and Proposals were made:

Observation 1: The objective of the core part provides the following assumptions: up-to 4-layer DL MIMO per CC, MRTD>CP and up to 25 dB power imbalance.

Observation 2: With power imbalance of up to 25 dB, each CC can have different MCS and different number of layers.

Observation 3: For 2Rx case, TS 38.101-4 Clause 5.2A.2.6 uses the following parameters: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for lower power carrier and 256QAM with code rate 0.69 for the higher power carrier

Observation 4: Type 4a FWA UE has 4 antennas for CC#1 and 2 antennas for CC#2, while Type 4b FWA UE has 4 antennas for both CCs.

Observation 5: The 25dB power difference is hard on the TEs. While AWGN channels can be used to get enough power out of them, they may not be MIMO/multi-layer compatible.

Proposal 1: RAN4 to specify PDSCH requirements for NR-CA with 2 CCs with different MCSs and number of layers for the 2 CCs to reflect the power imbalance, in which at least one of the CC shall take the maximum possible number of layers.

Proposal 2: The following parameters can be used as a starting point: SCS 30KHz, BW 40 MHz, QPSK with code rate 0.30 for the lower power carrier and 256QAM with code rate 0.69 for the higher power carrier.

Proposal 3: In the case of Type 4a FWA UE, the lower power carrier is assumed to use the lower number of Rx antennas, and, hence, the lower number of layers.

Proposal 4: RAN4 to discuss how to enable up-to 4-layer if AWGN channels are used.

- References

- RP-242083, Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), 3GPP RAN#105, KDDI, Melbourne, Australia, September 9-12, 2024

- R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, 3GPP RAN4#104-bis-e, KDDI, Online Meeting, October 10-19, 2022

- RP-233513, Support of intra-band non-collocated EN-DC/NR-CA deployment, 3GPP RAN#102, KDDI, Edinburgh, Scotland, December 11-15, 2023

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V18.8.0, 06-2025