3GPP TSG-RAN WG4 Meeting #116-bis				 R4-2514304

Prague, CZ, 13th - 17th October, 2025

Source:	Qualcomm Incorporated

Title:	Power boosting for extended inner MPR regions

Agenda item:	4.6.1.2

Document for:	Approval

# 1.	Introduction

In RAN4#116 there was discussion on using rel-18 power boosting for the extended inner region created by bandwidth expansion. Most companies wanted to study this further and so a decision on power boosting in the extended inner region was deferred. In this paper we further present our views on this topic.

# 2. 	Discussion

When BW extenstion is allowed on one or both sides of the original CBW  the inner region becomes larger as indicated in figure 1 where the left figure illustrates the inner and outer regions corresponding to the original non-extended CBW and the right shows the inner and outer regions when the original CBW is extended on both sides. A comparision of the 2 plots shows that the inner region has grown by the red shaded region illustrated in the extended CBW figure.

Figure 1: Increase in inner region with BW extension

In release 18 power boosting was defined for the inner region for Pi/2 BPSK and QPSK signals. Based on the capabilites powerBoosting-pi2BPSK-QPSK-r18 and powerBoosting-pi2BPSK-QPSK-Modified-r18 powerboost is possible in both the enhanced inner region as defined in [1] and the region that encompasses the inner region but is outside the enhanced inner region.

In our opinion the power boost feature defined in release 18 can be naturally extended to the release 19 MPR reduction feature where the inner region is expanded by BW expansion. Using the Rel-18 power boost capability in the expanded inner region benefits overall system performance by enabling higher transmit powers for inner RB allocations.

Our simulations indicate that both with and without FDSS a power boost of at least 1 dB is possible in the inner and extended inner regions with and without bandwidth expansion. Figure 2 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS in the absence of bandwidth expansion. This represents the rel-18 baseline power boost scenario.

Figure 2 – Rel-18 baseline power boost scenario

Figure 3 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS and with bandwidth expansion of CBW/2 on the lower side of the spectrum.

Figure 3 –  Power boost with and without FDSS with BW expansion on lower side of spectrum

Figure 4 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS and with bandwidth expansion of CBW/2 on the upper side of the spectrum.

Figure 4 –  Power boost with and without FDSS with BW expansion on upper side of spectrum

Figure 5 illustrates a scatter plot of the inner and enhanced inner region with and without FDSS and with bandwidth expansion of CBW/2 on both sides of the spectrum.

Figure 5 –  Power boost with and without FDSS with BW expansion on both sides of spectrum

Comparing figure2 with figures 3, 4 and 5 we see that the 1dB power boost that was agreed in rel-18 for the inner and enhanced inner regions is also possible for the expanded inner and expanded enhanced inner regions created by bandwidth expansion.

Proposal 1: For a UE that supports MPR reduction through bandwidth expansion allow the same level of power boost in the expanded inner and expanded enhanced  inner regions as in the original inner and enhanced inner regions.

We propose that if a UE supports MPR reduction through bandwidth expansion then the enhanced inner allocation region within the inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low_new + P1  ≤  RBStart  ≤  RBStart,High_new - P1

Where

RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low)

RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB)

P1 = MIN{12,CEIL(2+NRB/25)}

Where RBShift_low and RBShift_high  are the RB extensions corresponding to CBW/2 or CBW/4 bandwidth extensions

Proposal 2: If a UE supports MPR reduction through bandwidth expansion then the expanded enhanced power inner allocation region within the expanded inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low_new + P1  ≤  RBStart  ≤  RBStart,High_new - P1

Where

RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low)

RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB)

P1 = MIN{12,CEIL(2+NRB/25)}

Where RBShift_low and RBShift_high  are the RB extensions corresponding to CBW/2 or CBW/4 bandwidth extensions

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal 1: For a UE that supports MPR reduction through bandwidth expansion allow the same level of power boost in the expanded inner and expanded enhanced  inner regions as in the original inner and enhanced inner regions.

Proposal 2: If a UE supports MPR reduction through bandwidth expansion then the expanded enhanced power inner allocation region within the expanded inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low_new + P1  ≤  RBStart  ≤  RBStart,High_new - P1

Where

RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low)

RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB)

P1 = MIN{12,CEIL(2+NRB/25)}

Where RBShift_low and RBShift_high  are the RB extensions corresponding to CBW/2 or CBW/4 bandwidth extensions

# References

[1] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0