3GPP TSG-RAN WG4 Meeting #116bis	R4-2514310

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	On General aspects in AIML Mobility

Agenda item:	7.12.2

Document for:	Discussion

Introduction

The study on AI/ML Mobility was concluded in RAN4#116 in the scope of Rel-19 SI ‎[1] and the new WI of Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR [2] was approved in RAN#109 meeting.

In this paper, we provide our views on general aspects related to AI/ML Mobility use cases including:

- Impact beam level L3-RSRP predictions

- Generalization aspects related to AI/ML Mobility

Discussion

Use cases for L3 beam level predictions

Regarding L3 beam-level predictions, following agreements were reached in RAN2#127 and RAN2#127bis :

RAN2#127 FS_NR_AIML_Mob - Release 19 Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

RAN2#127bis FS_NR_AIML_Mob - Release 19 Agreements - To keep two filtering options on the table and up to company to report. - One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. - In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

However, as per the new WID [2], it was agreed that L3 beam level predictions are not supported, as mentioned in the following extract from the Objective section of the WID:

The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported.

RAN4 to focus first on L3 cell level RSRP (Point C) prediction and to not define any requirements for L3 beam-level prediction (Point E).

Generalization aspects related to AI/ML Mobility

The generalization capability of a functionality can impact the performance of the functionality, for example if the functionality is not generalized enough across a set of scenarios, this will result in frequent switch of the functionality resulting in the degradation of the performance.

Less generalized functionalities across a set of scenarios can result in frequent switching of functionality/configurations resulting in performance degradation.

This issue is more important in case of AI/ML Mobility use case since the AI/ML functionality need to have sufficient accuracy in changing field conditions and configurations.

In case of AI/ML Mobility, due to changing sites configurations in the field, the generalization related aspects become more important.

To mitigate the problems caused by less generalized functionalities, it is important to ensure that the functionality is generalized enough. Ensuring the generalization aspects of functionality across different scenarios and conditions is not straightforward. At the outset we need the scenarios and conditions that needs to be identified and aligned. KPIs for generalization need to be defined. And then the tests need to be carried out on all the scenarios and based on the validation result it can be decided if the functionality is generalized enough. To carry out all these identified steps, it becomes imperative to have a test framework to test the generalization aspects of a functionality.

RAN4 to consider defining requirements and test cases to verify generalization capabilities of AI/ML configuration for AI/ML mobility use case.

Conclusion

In this paper we provide our view on the general aspects of AI/ML Mobility use case, we particularly addressed the following topics:

- Impact beam level L3-RSRP predictions

- Generalization aspects related to AI/ML Mobility

The following Observations and Proposals were made:

Proposal 1: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and to not define any requirements for L3 beam-level prediction (Point E).

Observation 1: Less generalized functionalities across a set of scenarios can result in frequent switching of functionality/configurations resulting in performance degradation.

Observation 2: In case of AI/ML Mobility, due to changing sites configurations in the field, the generalization related aspects become more important.

Proposal 2: RAN4 to consider defining requirements and test cases to verify generalization capabilities of AI/ML configuration for AI/ML mobility use case.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, OPPO, Interdigital, RAN#109 meeting, 15 – 18 September 2025, Beijing, China.