3GPP TSG-RAN WG4 Meeting #116bis	    										R4-2514312

Prague, Czech, October 13th -17th, 2025

Source: 	Apple

Title: 	On PSD limitation for A-IOT in band operation

Agenda Item:	7.11.3

Document for:	Discussion

# Introduction

In RAN1#122 meeting, RAN1 discussed BS Tx power assumption and R2D transmission bandwidth assumption and sent the conclusion to RAN4 in [1].

Agreement For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations Agreement For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 180kHz as starting point - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

There is one open issue on PSD which may impact evaluations both in RAN4 and in RAN1. This contribution will provide analysis on the reasonable PSD limitation.

# Discussion

First of all, we would like to explain what PSD means in the LS. Our understanding is that the PSD discussion in the LS is about differentiating between regular NR transmissions and A-IoT reader transmissions for in-band deployment. The rationale is to avoid BS to concentrate too much of its power in the RBs for A-IoT, which might cause excessive interference to NR. So basically, it is recommended to consider some PSD limitation at BS reader for A-IoT.

In RAN4, no requirement is defined on PSD difference between A-IOT RB PSD and NR RB PSD for A-IOT in-band operation till now. While taking NB-IOT in-band operation as the starting point from 38l.104 (copied below), it is similar as for the intended purpose. The minimum PSD difference between IOT REs and NR REs supported is +6dB, no upper limit is defined. [+6dB] could be used as the starting point for lower bound of PSD difference for Ambient IOT in-band operation. For upper bound, further evaluation in RAN4 is needed.

Proposal 1: It is proposed that [+6dB] is used as the starting point for lower bound of Ambient IOT RE dynamic range in-band operation.

Proposal 2: It is proposed to determine an upper limit later in RAN4 based on further evaluation.

Proposal 3: LS to RAN1 is preferred as the PSD impact both RAN1 and RAN4 evaluations.

# Conclusion

This contribution analysed PSD limitation between Ambient IOT RBs and NR RBs for Ambient IOT in band operation. Based on the analysis in this contribution, we have the following proposals.

Proposal 1: It is proposed that [+6dB] is used as the starting point for lower bound of Ambient IOT RE dynamic range in-band operation.

Proposal 2: It is proposed to determine an upper limit later in RAN4 based on further evaluation.

Proposal 3: LS to RAN1 is preferred as the PSD impact both RAN1 and RAN4 evaluations.

# Reference

[1] R1-2506575, LS on BS Tx power assumption for coverage evaluation in A-IoT, RAN1#122