3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2514330

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	In-band PSD FCC requirement for 4.9HGHz band

Source: 	Skyworks Solutions, Inc.

Agenda Item:	5.11.4	UE RF requirements.	[NR_TDD_band_4900MHz_US]

Document for:	Approval

# 1	Background

In RAN#109, the 4.9GHz WI [1] was updated to continue the work based on the initial one quarter study. In RAN4#116, one contribution [2] discussed the in-band power limitations in the 4.9GHz band FCC rules. In this contribution, we discuss how the in-band PSD FCC limit not only imposes power limitations for HPUE but also for small allocation cases including for PC3.

# Discussion

## 2.1 Power limitation due to the FCC in-band PSD limit

For PC3 and HPUE, the FCC rules set power limitations in three different ways:

- Conducted power per channel bandwidth

- Radiated power (EIRP) per channel bandwidth

- In-band PSD limit of 21dBm/MHz.

Based on these limitations, especially the conducted power, the contribution [2] concluded that:

- PC3 and PC2 can be enabled for all channel bandwidths

- PC1.5 can be enabled for CBW ≥ 15MHz and a mechanism should be introduced to limit its power for 10MHz

- PC1 can be enabled for CBW ≥ 20MHz and a mechanism should be introduced to limit its power for 10MHz

If this is true based on conducted power and fully allocated channel, the 21dBm/MHz in-band PSD should also be verified when partial allocations are scheduled in UL. Table 1 calculates the required back-off to meet both the in-band PSD and conducted power limits for different NRBs that correspond to either partial allocations or one of the proposed CBW NRBs.

In the back off column:

- The green highlight means that the UE can reach PCmax +2dB

- The yellow highlight means that the UE power is limited by the conducted power limit

- The orange highlight means that the UE power is limited by 21dBm/MHz PSD limit

Table 1: Required back-off per power class versus partial/full allocation up to 50MHz CBW

|  |  |  |  | PC3+2dB |  | PC2+2dB |  | PC1.5+2dB |  | PC1+2dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB |  | minCBW [MHz] | BW [MHz] | PSD [dBm/MHz] | back-off [dB] | PSD [dBm/MHz] | back-off [dB] | PSD [dBm/MHz] | back-off [dB] | PSD [dBm/MHz] | back-off [dB] |
| 15KHz | 30KHz |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 0.18 | 25.0 | 4.0 | 28.0 | 7.0 | 31.0 | 10.0 | 33.0 | 12.0 |
| 2 | 1 |  | 0.36 | 25.0 | 4.0 | 28.0 | 7.0 | 31.0 | 10.0 | 33.0 | 12.0 |
| 4 | 2 |  | 0.72 | 25.0 | 4.0 | 28.0 | 7.0 | 31.0 | 10.0 | 33.0 | 12.0 |
| 6 | 3 |  | 1.08 | 24.7 | 3.7 | 27.7 | 6.7 | 30.7 | 9.7 | 32.7 | 11.7 |
| 8 | 4 |  | 1.44 | 23.4 | 2.4 | 26.4 | 5.4 | 29.4 | 8.4 | 31.4 | 10.4 |
| 10 | 5 |  | 1.8 | 22.4 | 1.4 | 25.4 | 4.4 | 28.4 | 7.4 | 30.4 | 9.4 |
| 12 | 6 |  | 2.16 | 21.7 | 0.7 | 24.7 | 3.7 | 27.7 | 6.7 | 29.7 | 8.7 |
| 14 | 7 |  | 2.52 | 21.0 | 0.0 | 24.0 | 3.0 | 27.0 | 6.0 | 29.0 | 8.0 |
| 16 | 8 |  | 2.88 | 20.4 | 0.0 | 23.4 | 2.4 | 26.4 | 5.4 | 28.4 | 7.4 |
| 28 | 14 |  | 5.04 | 18.0 | 0.0 | 21.0 | 0.0 | 24.0 | 3.0 | 26.0 | 5.0 |
| 32 | 16 |  | 5.76 | 17.4 | 0.0 | 20.4 | 0.0 | 23.4 | 2.4 | 25.4 | 4.4 |
| 52 | 24 | 10 | 9.36 | 15.3 | 0.0 | 18.3 | 0.0 | 21.3 | 1.0 | 23.3 | 3.0 |
| 79 | 38 | 15 | 14.22 | 13.5 | 0.0 | 16.5 | 0.0 | 19.5 | 0.0 | 21.5 | 1.2 |
| 106 | 51 | 20 | 19.08 | 12.2 | 0.0 | 15.2 | 0.0 | 18.2 | 0.0 | 20.2 | 0.0 |
| 133 | 65 | 25 | 23.94 | 11.2 | 0.0 | 14.2 | 0.0 | 17.2 | 0.0 | 19.2 | 0.0 |
| 160 | 78 | 30 | 28.8 | 10.4 | 0.0 | 13.4 | 0.0 | 16.4 | 0.0 | 18.4 | 0.0 |
| 216 | 106 | 40 | 38.88 | 9.1 | 0.0 | 12.1 | 0.0 | 15.1 | 0.0 | 17.1 | 0.0 |
| 270 | 133 | 50 | 48.6 | 8.1 | 0.0 | 11.1 | 0.0 | 14.1 | 0.0 | 16.1 | 0.0 |

Observations:

- All fully allocated CBWs can be supported by PC3 and PC2 at maximum power +2dB

- >10MHz fully allocated CBW can be supported by PC1.5 at maximum power +2dB

- >15MHz fully allocated CBW can be supported by PC1 at maximum power +2dB

- However, for partial allocations, back off is required to meet in band PSD for:

  - <14/7RB for PC3 for 15 and 30kHz SCS respectively

  - <28/14RB for PC2 for 15 and 30kHz SCS respectively

  - <44/33RB for PC1.5 and PC1 for 15 and 30kHz SCS respectively

Proposal:

- A mechanism should be developed to limit the UE maximum output power for partial allocations failing the FCC in-band PSD limit for PC3 and PC2

- This mechanism is also used to limit the power for 10MHz CBW for PC1.5 and 10/15MHz for PC1.

# Conclusion

In this contribution, we analyse potential power limitations due to both the conducted power and the in-band PSD FCC limits.

Proposal:

- A mechanism should be developed to limit the UE maximum output power for partial allocations failing the FCC in-band PSD limit for PC3 and PC2

- This mechanism is also used to limit the power for 10MHz CBW for PC1.5 and 10/15MHz for PC1.

# References

[1] RP-252929	Revised WID on Introduction of NR TDD 4.9GHz band for US operation, Fujitsu Limited, RAN#109

[2] R42510411 General aspects regarding the 4940–4990 MHz band, Qualcomm Incorporated, RAN4#116