3GPP TSG RAN4 Meeting #116bis		R4-2514333
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Impact of configuring 2 SMTC periodicities over multiple SMTC occasions in NTN

Agenda item:	4.25.3

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

RAN2 has agreed that up to 7 SMTCs might be configured by the serving cell in one frequency layer with up to 2 different SMTC periodicities. RAN2 has reached further agreements in their design and had notified RAN4 in the LS R2-2506298.

Discussion

RAN2 LS

In the previous meeting, RAN4 has discussed the requirements for UEs configured with up to 6 SMTCs and two different periodicities in one frequency layer. In their LS to RAN4, RAN2 has notified about new developments in their feature design as follows:

For downlink coverage enhancement, RAN2 further made the following agreements related to SMTC enhancement: - We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. - In connected mode the NW will only configure up to 4 STMC. - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

We will provide comments for each of the points in the agreements made by RAN2:

- “We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)”

This agreement does impact RAN4 discussions, it only confirms that there are no backwards compatibility issues with legacy UEs, as only the UEs that support the new feature (DL Coverage enhancements) are affected by the new feature.

For UEs that do not support the new feature, legacy requirements are still applicable.

- “The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs)”

- “RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements.”

In a general sense the number of SMTCs to be measured do not impact RAN4 progress either, as the total number of SMTCs is re-confirmed to be limited to the number equal to UE capability. For the second part, this confirmation by RAN2 revises RAN4 understanding of “round-robin” utilization of all SMTCs configured and “longer requirements” by UE implementation when the number of SMTCs configured in idle mode exceeds the UE capability. Therefore, in order to capture RAN2 understanding, RAN4 needs to enforce in its specification the downselection of SMTC opportunities.

In IDLE/INACTIVE mode, if the UE is configured with more SMTCs than supported by UE capability in a frequency layer, including the SMTC related to the serving cell, and location information is associated to each SMTC, the UE is expected to downselect the number of SMTCs, based on the location information, to match the maximum supported by UE capability.

The SMTC associated to the serving cell has to be always within the set of SMTCs selected by the UE to perform measurements.

- “In connected mode the NW will only configure up to 4 STMC.”

At first glance this agreement seems to be in line with legacy RAN4 specification. However, at a closer look, UEs are not mandate to support 4 SMTCs. The actual number of SMTCs supported by one UE which may be up to 4)  might be smaller than 4. Therefore, RAN4 has to notify in their LS Reply to RAN2 that:

In connected mode, the NW is not expected to configure a number of SMTCs that exceeds UE capability. If a UE is configured with more SMTCs than supported, the UE will only measure the first X SMTCs (where X is the number of supported SMTCs by this UE).

The latest agreement in RAN2 ls is the most crucial point in their LS, as indicates that if RAN4 does not agree on requirements for two SMTC periodicities, RAN2 will limit the application of this feature in NR. We will focus in this point in a dedicated subsection.

Two SMTC periodicities within one frequency layer.

In their LS to RAN4, RAN2 indicates that:

- “RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. “

RAN2 is thereby confirming the implementation of 2 SMTC periodicities configured in parallel. RAN4 has addressed the issue and has reached the following conclusion :

Agreement (ad-hoc): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For Ttarget_cell_SMTC_period in paging interruption requirements: - Ttarget_cell_SMTC_period is the SMTC periodicity corresponding to the physical cell ID of the target cell - For Kmulti_SMTC, K_satellite, Kp, Klayer1_measurement, and Kgap in RRC Connected mode intra/inter-frequency measurement requirement: - To be confirmed in maintenance phase: - Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. - Klayer1_measurement: Calculate scaling factor Klayer1_measurement based on the union of the SMTCs

The proposal above, is simple and addresses the problems of collisions. However, it might in several cases lead to a significant increase in the total requirement beyond the necessary. This is better explained by looking at Figure 1. In this figure is configured with two SMTCs. SMTC 1 is configured with 20 ms periodicity, while SMTC 2 is configured with 160 ms periodicity. All occasions of SMTC1 are overlapped by SMTC2 occasions. The assumption is that in certain scenario SMTC1 and SMTC2 cannot be measured in the same instant, for example, due to association to different satellites.

Figure 1. Two SMTCs (SMTC1 and SMTC2) configured with different periodicities. SMTC 1 is configured with 20 ms periodicity, while SMTC 2 is configured with 160 ms periodicity. All occasions of SMTC2 are overlapped by SMTC1 occasions.

If the proposal is confirmed and the max value is accepted, the value of Kmulti_SMTC will be equal to 2 for both cases, which means that the time-to-detect, time-to-measure and time-to-evaluate a cell will be multiplied by 2 for both cases. Observe that, although 1/8 of the SMTC1 occasions are colliding with other SMTCs, which means that the other 7/8 occasions collision-free.

In the example of Figure 1, the requirements related to SMTC1 are expanded by 2, even though only 1/8 of the SMTC occasions are colliding with other SMTCs.

Likewise, for the SMTC with 160 ms, the requirements for measurement procedures will be relaxed by a factor of 2 if the regular SMTC sharing previously adopted in NTN is used in this scenario. This would be considering that the “collision” occasions will be equally split between SMTC1 and SMTC2, meaning that half of the SMTC2 occasions will be deferred to measurements in SMTC1, even though, SMTC1 had 7 collision-free opportunities already in this period.

It seems more efficient to establish that:

When SMTCs with different periodicities collide, for measurements purposes, the SMTC with longer periodicity shall always take precedence over the SMTC with shortest periodicity.

Using this new rule, the total time to have 5 SMTCs available for measurements will be maintained unmodified alternating in  85/105 ms, whereas the total time to have 5 SMTCs available for measurements will drop from 1285 ms to 645 ms, a significant gain of 640 ms.

In the example above, the total time for the UE to have 5 measurement opportunities in SMTC2 will be enhanced from 1285 ms to 645 ms.

- The gains are even more pronounced if more collisions are considered. Consider, for example, the case of Figure 2. In this example, in the worst case scenario, there is a collision among 4 SMTCs. This would cause the requirements for measurements to be relaxaed by a factor of 4. However, as previously discussed, if the longer periodicity takes precedence over the short periodicity, SMTC2 will only have to share its opportunities in collisions with SMTC4, and that would cause a relaxation factor equal to 2. Now, let’s take a look at SMTC1 and SMTC3. In this new example, some (1/8) of their opportunities are colliding with SMTCs with larger periodicity (which take precedence), and the other 7/8 are to be split equally between the two.

Figure 2 – Four SMTCs with different periodicities and offsets.

- In both cases, Figure 1 and Figure 2, if we consider that the SMTC with shorter period doesn’t take precedence over the SMTCs with longer periodicity, then we will consider the value of K_multi_SMTC only for the occasions shared between SMTCs with same periodicity.

The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity.

But by doing so, the amount of SMTC opportunities left for SMTC1 in Figure 1 and SMTCs 1 and 3 in Figure 2 will be smaller (by a factor of 7/8) then they would be if these SMTCs did not collide with SMTCs with larger periodicities. In some scenarios this can be even smaller fraction. In order to compensate for the lost measurement opportunities, we propose to add an additive factor in the measurement requirements for SMTCs with shorter periodicity.

For SMTCs with shorter periodicity that overlap with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions.

Figure 3 – Example with four configured SMTCs with different periodicities and offsets. SMTC occasions which overlap with SMTCs with different periodicities will be used for SMTCs with the largest periodicity

One example of the requirements considering that some of the measurement occasions will be skipped in some SMTCs is given below.

Table 1 - Example of measurement period

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( (5+ Lmulti_SMTC) x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x (5+ Lmulti_SMTC) x Kmulti_SMTC x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil((5+ Lmulti_SMTC) x Kmulti_SMTC x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1:If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Using the example in Figure 3, Lmulti_SMTC would always be 0 for the measurements of SMTC2 and SMTC4, the SMTCs with largest periodicity. In SMTC1 and 3, this term would be equal to the number of measurement occasions within the measurement period that overlap with the SMTC2 and SMTC4.

Conclusion

In this paper we discussed the impact of the new measurement framework proposed by RAN2 and RAN1 to handle the extended SSB periodicity in NTN under the topic of DL coverage enhancements. Based on our discussion and analysis, we made the following list of proposals:

Observation 1: For UEs that do not support the new feature, legacy requirements are still applicable.

Proposal 1: In IDLE/INACTIVE mode, if the UE is configured with more SMTCs than supported by UE capability in a frequency layer, including the SMTC related to the serving cell, and location information is associated to each SMTC, the UE is expected to downselect the number of SMTCs, based on the location information, to match the maximum supported by UE capability.

Proposal 2: The SMTC associated to the serving cell has to be always within the set of SMTCs selected by the UE to perform measurements.

Proposal 3: In connected mode, the NW is not expected to configure a number of SMTCs that exceeds UE capability. If a UE is configured with more SMTCs than supported, the UE will only measure the first X SMTCs (where X is the number of supported SMTCs by this UE).

Observation 2: In the example of Figure 1, the requirements related to SMTC1 are expanded by 2, even though only 1/8 of the SMTC occasions are colliding with other SMTCs.

Proposal 4: When SMTCs with different periodicities collide, for measurements purposes, the SMTC with longer periodicity shall always take precedence over the SMTC with shortest periodicity.

Observation 3: In the example above, the total time for the UE to have 5 measurement opportunities in SMTC2 will be enhanced from 1285 ms to 645 ms.

Proposal 5: The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity.

Proposal 6: For SMTCs with shorter periodicity that overlayp with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions.

- References

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT R2-2506298, LS on additional agreements on SMTC enhancements, RAN WG2 Meeting #131, Bengaluru, India, 25th - 29thAugust, 2025

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN WG2 Meeting #131, Bengaluru, India, 25th - 29thAugust, 2025

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm (Moderator), Bengaluru, India, 25th - 29thAugust, 2025