3GPP TSG RAN4 Meeting #116bis		R4-2514338
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Replying to RAN2 LS on CQI reporting in MSG3

Agenda item:	4.26.2

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) to introduce the contention-based message 3 (CB-MSG3), where the UE skips the Msg1 and Msg2 in the Random Access procedure, for faster data dump. This was indicated by RAN2 in their LS to RAN4 (R2-2405769).

RAN4 has initially provided its views on the matter analyzing the potential impacts on the timing requirements of such feature and captured the basic requirements for feature functionality in previous meetings.

RAN2 has further progressed in its work and has agreed to include CQI reporting in MSG3 for anchor carrier but not for non-anchor carrier. This was indicated to RAN4 in R2-2506299

In this document we analyze the impact of RAN2 agreement and how to address this from RAN4 perspective.

Discussion

RAN2 has discussed whether CQI reporting in CB-Msg3 EDT procedure shall be supported or not. RAN2 has come to the following agreement (R2-2506299):

- We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4)

From RAN4 perspective, we need to careful analyse how this impact the requirements for CQI reporting and whether new requirements are needed. In the legacy specifications, the UE may include a CQI report in Msg3 if configured to do so.

From RAN2 point of view, the expected content of the CQI reporting is defined in TS 36.331, as follows (highlights are our own, to provide clarity on the discussion):

#### 5.3.3.3a Actions related to transmission of RRCConnectionResumeRequest message If the UE is resuming the RRC connection from a suspended RRC connection, the UE shall set the contents of RRCConnectionResumeRequest message as follows: … 1> if the UE is a NB-IoT UE: 2> if the UE supports DL channel quality reporting in MSG3 and cqi-Reporting is present in SystemInformationBlockType2-NB: 3> set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the carrier where the random access response is received as specified in TS 36.133 [16]; NOTE 0: The downlink channel quality measurements use measurement period T1 or T2, as defined in TS 36.133 [16].

So, RAN2 defines that the CQI reporting must included the latest result of the downlink channel, according to measurements performed in period T1 or T2, according to the definitions of RAN4 spec (TS 36.133).

However, the measurement periods T1 or T2, in TS 36.133,  were developed considering a scenario where MSG1 (PRACH attempt) and MSG2 (RAR) takes place:

#### 6.6A.2.6 MSG3-based channel quality report for UE Category NB1 The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category NB1 as defined in TS 36.331 [2] section 5.3.3.3, 5.3.3.3a, and 5.3.7.4. The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1% with the parameters specified in Table 6.6A.2.6-1. Table 6.6A.2.6-1: NPDCCH transmission parameters for downlink quality reporting Parameters; Values. DCI format; Format N1. Number of information bits (excluding CRC); 23bits. System bandwidth; 200kHz. Aggregation level; 2. DRX; OFF. For channel quality reporting in the anchor carrier, the reported NPDCCH repetition level shall be derived from the channel quality measured in the period T1 or T2 in the carrier where the random access response is transmitted, where - T1 is the period before NPRACH transmission used for NRSRP measurement for enhanced coverage level estimation - T2 is the period from the beginning of the random access response to the beginning of PUSCH format 1 for DL channel quality reporting. For channel quality reporting in the non-anchor carrier, the reported NPDCCH repetition level shall be derived from the channel quality measured in the period T2 in the carrier where UE monitors Random Access Response where T2 is defined above. The NPDCCH repetition level for CQI-NPDCCH-NB and CQI-NPDCCH-Short-NB is chosen from the supported NPDCCH repetition levels [3]. The report mapping is defined in 9.1.22.15. The UE shall satisfy the downlink channel quality measurement accuracy requirements as specified in 9.1.22.16.

So, first of all, it is important to notice that none of the time periods above are applicable to the case of CB-MSG3 EDT transmission:

- T1 is the period before NPRACH transmission, but there is no NPRACH transmission in CB-MSG3 EDT

- “T2 is the period from the beginning of the random access response to the beginning of the PUSCH format 1 for DL channel quality reporting”, which again is invalid because there is no random access response in CB-MSG3 EDT

Neither T1 nor T2 can be unequivocally defined for the context of CB-MSG3 EDT transmission.

Therefore, the definition of a new timing window for the CQI measurement report is necessary.

For the case where CB-MSG3 EDT is applied, revise the definition to T1 to make it suitable for the CB-MSG3 EDT transmission.

The problem on updating CQI report in retransmissions

According to the RAN2 agreements in this WI, the following procedures were added for CB-MSG3 EDT:

- the UE shall transmit cb-Msg3-NumOfReplicas replicas in the CB-Msg3 window, which has a size windowSize in the range is 10ms (n1) : 1.28s (n128).

- the UE will then monitor for a response in the cb-Msg3-ResponseWindow, which has a range of 240-10240 subframes i.e. 240 ms to 10.24 s.

- if the UE does not find a response, the UE may backoff (if the backoff indicator was provided) and reattempt CB-Msg3 in a next window. The BI range is 0-524s for NB-IoT.

The backoff procedure is specified in RAN2 (MAC) specifications according to the following:

5.1x.2 CB-Msg3 transmission Before the CB-Msg3 transmission, the MAC entity shall select cb-Msg3-NumOfReplicas UL grants for CB-Msg3 transmission as follows: - select the next upcoming CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level; - randomly select cb-Msg3-NumOfReplicas PUSCH resources in the time domain within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by cb-Msg3-TimeResource associated with the selected enhanced coverage level; - randomly select a frequency domain PUSCH resource for each selected time domain resource provided by prb-AllocationInfoSet (npusch-SubCarrierSetList for NB-IoT) associated with the selected enhanced coverage level. (…) #### 5.1x.3 CB-Msg4 reception After the CB-Msg3 transmission, the MAC entity shall: - start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT; - monitor the PDCCH while CB-Msg3ResponseTimer is running; - if notification of a reception of a PDCCH transmission is received from lower layers; and - if the CB-Msg4 is successfully decoded: - if the CB-Msg4 contains a CB Backoff Indicator subheader: - set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used. - else: - set the CB-Msg3-EDT backoff parameter value to 0 ms. […] - if the CB-Msg3ResponseTimer expires: - when MAC entity has finished decoding of all received CB-Msg4 and if CMR reception is not successful: - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - increment CB_MSG3_TRANSMISSION_COUNTER_CE by 1; - if CB_MSG3_TRANSMISSION_COUNTER_CE = cb-Msg3-MaxAttemptNum for the corresponding enhanced coverage level + 1: - discard the CB-RNTI; - consider the CB-Msg3 transmission unsuccessfully completed; - indicate the transmission using CB-Msg3-EDT has failed to the upper layers. - else: - based on the CB-Msg3-EDT backoff parameter, select a random backoff time according to a uniform distribution between 0 and the CB-Msg3-EDT backoff Parameter value; - delay the subsequent CB-Msg3 transmission by the backoff time; - proceed to the CB-Msg3 transmission (see clause 5.1x.2).

Now, consider the scenario described in Figure 1, where the UE transmits a CB-MSG3 attempt and fails to receive a response to that (CB-MSG4 failure) within the CB-Msg3ResponseTimer as defined in RAN2 specifications and highlighted above. By RAN2 specifications the UE has to attempt the retransmission at the first CB-MSG3 window after the expiration of the backoff timer.

The UE has to attempt the retransmission of a CB-MSG3 at the first CB-MSG3 window after the expiration of the backoff timer.

Figure 1 Issue of CQI determination after failed CB-Msg3 transmission.

If current procedure is followed, the UE shall perform a new CQI measurement for reporting purposes right before the retransmission of the CB-MSG3. However, there is a fundamental difference between this case and the legacy case (NPRACH). In legacy specifications, the UE can make the CQI measurements before transmitting the PRACH (T1) or between the RAR and Msg3 transmission (T2). If there is a failed RACH/Contention resolution in legacy, the UE will always have the opportunity for a new measurement in the period between RAR and Msg3 (T2) if the UE has performed a backoff, regardless whether T1 is too short.

In legacy cases the UE always have the opportunity for a new CQI measurement in the period between RAR and MSG3 (T2).

In the CB-Msg3 case, as T2 is not applicable, the UE will not have this opportunity if T1 is not long enough for the UE to complete the measurements.

There is now a fundamental problem: between the detection of the CB-Msg3 failure and the next CB-Msg3 retransmission attempt there is only the backoff timer. Therefore, a new time period T4 can be defined for the UE to measure the channel quality during the backoff timer.

Proposal 2: In case of CB-Msg3-EDT response timer expiry (i.e. a first CB-Msg3 transmission has failed), T4 is the period from the beginning of the backoff time, to the beginning of the next CB-Msg3-EDT transmission window, in which the UE can measure the channel quality.

It is important to note that the backoff timer is a random variable drawn between 0 and the the CB-Msg3-EDT backoff parameter value.  If the backoff timer is too short, there might not be enough time for the UE to complete the measurements.

When a CB-Msg3 retransmission attempt is triggered, the backoff timer might be too short for the UE to complete the CQI measurement.

Therefore, if CB-Msg3 is configured and the UE determines that the CQI measurement cannot be completed, the UE should be allowed to skip the next CB-Msg3 Tx window to complete the measurement. The UE can then transmit the CB-Msg3 and include the new CQI measurement in subsequent CB-Msg3 Tx window. Therefore, we propose:

In case of CB-Msg3 retransmission, if the back-off timer is below [2] DRX Cycles and the UE cannot complete the CQI report measurements before the next CB-Msg3 attempt, the UE is allowed to postpone the next CB-Msg3 transmission to a later CB-Msg3 transmission window, to complete the channel quality measurement.

Another way to avoid this problem is to allow the UE to determine the previously reported CQI is still valid if the interval between two  CB-Msg3 transmission attempts is short (such that the CQI is not expected to change considerably):

In case of CB-Msg3 retransmission, if the interval between the current retransmission and the previous CB-Msg3 transmission is below 4 DRX Cycles, the UE is allowed to reuse the CQI reported in the previous CB-Msg3 transmission.

Alignment of core requirements with RAN2 design

In RAN2 specification, the implementation of the UE feedback upon reception of the CB-MSG3-EDT response, is specified as follows:

### 6.2.x MAC header for CB-Msg4 The MAC header is of variable size and consists of the following fields: - E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate at least another subheader follows. The subsequent subheader can be E/T/R/BI MAC subheader, E/T/R/R/H/TA/C MAC subheader or E/T/LCID/(F)/(L) MAC subheader. The E field is set to "0" to indicate that either a MAC CMR, a MAC SDU, or padding starts at the next byte; - T: The Type field indicates the type of the MAC subheader. The T field is set to "00" to indicate the presence of a CB Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of a CMR in the subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in the subheader. The value "11" is reserved for future use. The size of the Type field is 2 bits; - H: The H field is a flag indicating the presence of the HARQ ACK resource field in the corresponding MAC CMR. For NB-IoT, it also indicates the presence of the 4-bit R fields preceding the HARQ ACK resource field in the same MAC CMR; […] ### 6.2.y MAC payload for CB-Msg4 The MAC CMR is of variable size and consists of the following fields: - UE Contention Resolution Identity: This field contains the first 48 bits of the uplink CCCH SDU; - R: Reserved bit, set to "0"; - HARQ ACK resource: This field indicates the resource used to transmit the HARQ ACK for this MAC CMR. For BL UEs and UEs in CE mode A, the size of this field is 2 bits (see HARQ-ACK resource offset in clause 5.3.3.1.13 of TS 36.212 [5]). For NB-IoT UEs, the size of this field is 4 bits (see HARQ-ACK resource in clause 6.4.3.2 of TS 36.212 [5]). For BL UEs and UEs in CE mode A, if this field is not present as indicated in the corresponding MAC CMR subheader and the Timing Advance Command field is present, set these bits to "0"; […]

In RAN4 specification, the following was specified:

#### 6.6A.4.2 Correct behaviour when receiving a CB-Msg4 over CB-RNTI The UE shall send HARQ-ACK if the Contention Resolution is successful. […]

There is a mismatch between both specifications, as the RAN2 specification clearly establishes that the HARQ is optionally configurable by the network. Therefore, we propose:

Capture in the specification that: the UE shall send HARQ-ACK if the contention resolution is successful only if HARQ feedback is configured in Msg4.

Conclusion

In this paper we analyzed the need for updates on the RAN4 requirements to introduce requirements related to CB-MSG3 feature and the following observations and proposals were made:

Observation 1: Neither T1 nor T2 can be unequivocally defined for the context of CB-MSG3 EDT transmission.

Proposal 1: For the case where CB-MSG3 EDT is applied, revise the definition to T1 to make it suitable for the CB-MSG3 EDT transmission.

Observation 2: The UE has to attempt the retransmission of a CB-MSG3 at the first CB-MSG3 window after the expiration of the backoff timer.

Observation 3: In legacy cases the UE always have the opportunity for a new CQI measurement in the period between RAR and MSG3 (T2).

Observation 4: When a CB-Msg3 retransmission attempt is triggered, the backoff timer might be too short for the UE to complete the CQI measurement.

Proposal 2: In case of CB-Msg3 retransmission, if the back-off timer is below [2] DRX Cycles and the UE cannot complete the CQI report measurements before the next CB-Msg3 attempt, the UE is allowed to postpone the next CB-Msg3 transmission to a later CB-Msg3 transmission window, to complete the channel quality measurement.

Proposal 3: In case of CB-Msg3 retransmission, if the interval between the current retransmission and the previous CB-Msg3 transmission is below 4 DRX Cycles, the UE is allowed to reuse the CQI reported in the previous CB-Msg3 transmission.

Proposal 4: Capture in the specification that: the UE shall send HARQ-ACK if the contention resolution is successful only if HARQ feedback is configured in Msg4.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”,

- R1-2407548 “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

- TS 36.133 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.6.0

- TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, 3GPP, Release 18.6.0