3GPP TSG RAN4 Meeting #116bis		R4-2514339
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	On test cases for CB-MSG3 in TS 36.133

Agenda item:	6.21.2

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) to introduce the contention-based message 3 (CB-MSG3), where the UE skips the Msg1 and Msg2 in the Random Access procedure, for faster data dump. RAN4 has decided to include this new feature in RAN4 specifications, such that the correct UE behavior is captured when skipping RACH Msg1/2. In the previous meeting, RAN4 has decided to include one test case for “Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT” [1].

Discussion

RAN4 has agreed to include the test case for CB-msg3-EDT in normal coverage. In this document we discuss some of the test case configurations for this case.

In the legacy test case for Random Access, when UE performs the Msg1 transmission, the test case for normal coverage, i.e., test case A.13.3.2.1 test the UE behavior regarding the following functionalities:

- Random Access Response Reception

- No Random Access Response Reception

- Receiving a NACK on Msg3

- Reception of an incorrect message over temporary C-RNTI (contention based “loss”)

- Reception of a correct message over temporary C-RNTI (contention based “win”).

- Power ramp-up in the re-attempts.

While some of the functionalities defined above do not have a parallel on CB-Msg3-EDT functionality (cases 1-3 in the list above), some can also be found in this new feature with their respective modifications (cases 4-6). Besides, there is a similar function to the case 2 above (no-reception within a response within the monitoring timer (CB-Msg3ResponseTimer). We believe that all these functionalities that are specified in the core part for the CB-Msg3-EDT functionality shall be tested.

Besides, CB-Msg3-EDT also introduced a new functionality related to the transmission of replicas which specifies the number of replicas to be transmitted:

#### 6.6A.4.1 Correct behaviour when transmitting CB-Msg3 UE shall also execute the CB-Msg3-EDT procedure defined in clause 5.1x in TS 36.321 [17] using the NPUSCH configuration contained in CB-Msg3-ConfigSIB-NB-r19 in TS 36.331 [2].The UE shall transmit CB-Msg3 replicas within the CB-Msg3 window, according to network configuration.

We believe that this new functionality (e.g. the transmission of replicas) ought to be tested together with the ones previously mentioned.

The RAN4 test case for CB-Msg3 shall test the following functionalities:

  - Reception of an incorrect message over CB-RNTI

  - Reception of a correct message of CB-RNTI

  - No CB-Msg3 response within CB-Ms3ResponseTimer

  - The calculated UE Tx power at every CB-Msg3 retransmission according to the respective formula

  - The right amount of transmission of replicas.

Another practical aspect to be considered, based on latest LS received by RAN2 and currently being discussed in the core part is related to the inclusion of CQI reporting on CB-MSG3.

RAN4 to include CQI reporting on the test cases for CB-MSG3.

Practicalities of test cases

One important aspect of the testability of points “a” and “b” above (reception of correct/incorrect message over CB-RNTI) is that this is only testable if the network configures the UE to transmit the HARQ feedback upon reception of CB-Msg3. Therefore, we propose:

The HARQ feedback upon reception of Msg4 shall be configured in the configuration of gNB parameters in the test case.

One important thing to be considered is that, even if CB-MSG3 is configured by network, a UE capable of CB-MSG3-EDT transmission is the one in control (based on UE implementation) in selecting whether the UE is transmitting over a NPRACH or over a CB-MSG3-EDT.

RAN4 shall discuss how to enforce the CB-MSG3 transmission for the test case.

Conclusion

In this paper we analyzed the need for updates on the RAN4 requirements to introduce performance requirements related to CB-MSG3 feature and the following proposal was made:

Proposal 1: The RAN4 test case for CB-Msg3 shall test the following functionalities:

a.	Reception of an incorrect message over CB-RNTI

b.	Reception of a correct message of CB-RNTI

c.	No CB-Msg3 response within CB-Ms3ResponseTimer

d.	The calculated UE Tx power at every CB-Msg3 retransmission according to the respective formula

e.	The right amount of transmission of replicas.

Proposal 2: RAN4 to include CQI reporting on the test cases for CB-MSG3.

Proposal 3: The HARQ feedback upon reception of Msg4 shall be configured in the configuration of gNB parameters in the test case.

Proposal 4: RAN4 shall discuss how to enforce the CB-MSG3 transmission for the test case.

- References

- R4-2512148, “WF on RRM requirements for IoT_NTN_Ph3”, Mediatek, RAN4#116, Bengaluru, 25th - 29th August, 2025