3GPP TSG RAN4 Meeting #116bis		R4-2514345
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Issues on pre-compensation gap in TS 36.102

Agenda item:	4.27.3

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  The issue of segmentation gap was discussed based on RAN1 inquiry to RAN4 and a response was provided on previous meeting [2]. In the RAN4 response, which contains RAN4 agreements some issues were identified after the meeting and that requires re-evaluation of such agreements. In this document we discuss these issues.

Discussion

In the contents of the previous sent LS reply to RAN1, RAN4 has provided the following answer (highlights are our own and used here to facilitate the discussion):

At the RAN4#116 meeting, RAN4 reached the following agreement: - For frequency error requirement (TS 36.102): “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes, pre-compensation gap is not applicable for band 249. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.” (…) As a general assumption for both frequency error and timing error, no pre-compensation gap is needed before the beginning of each uplink burst of 8 consecutive transmitted subframes. However, when segmentation is applied (i.e. network configures the segmentation), UE pre-compensation gap may or may not be used for timing adjustment between 2 consecutive segments. Pre-compensation gap is not needed for the first segment. Therefore, RAN4 estimates that the use of pre-compensation gaps on the UE side may be allowed but is optional depending on the UE implementation. RAN4 also considers that segmentation can be applied during the transmission of an uplink burst of 8 consecutive transmitted subframes.

Let’s evaluate the contents above and explain the issues with such agreements (the parts highlighted in green refer to frequency requirements whereas the parts highlighted in yellow in the box above refer to timing requirements:

- “For frequency error requirement (…) pre-compensation gap is not applicable for band 249”: From frequency point of view, this agreement is inconsequential as the potentially small interval to be introduced by an eventual pre-compensation gap would not be sufficient to harm significantly the phase continuity of the transmission. Moreover, it indicates that the frequency error variation within 8 ms does not seem a significant concern to RAN4.  However, by saying that pre-compensation gap is not applicable for this band has undesired consequences: it precludes it to be used for timing adjustments.

The absence (or introduction) of the pre-compensation gap is inconsequential for the frequency error requirements in RAN4.

By stating that the pre-compensation gap is not applicable in band 249, we are preventing it to be used for timing adjustments.

However, things are considerably different when timing requirements are considered. Let’s breakdown our message to RAN1 to indicate what are our concerns with the current phrasing in the LS:

- “RAN4 also considers that segmentation can be applied during the transmission of an uplink burst of 8 consecutive transmitted subframes.” : This indicates that RAN4 agrees that segmentation can be configured and are applicable within bursts of 8 ms.

- no pre-compensation gap is needed before the beginning of each uplink burst of 8 consecutive transmitted subframes.: We agree with that statement, there is significant preparation time before the burst of 8 consecutive UL subframes such that the UE can adjust its timing without the pre-compensation game at the beginning of a new transmission.

- UE pre-compensation gap may or may not be used for timing adjustment between 2 consecutive segments. : Here is the part where we have identified a problem that will be discussed in the next paragraphs.

First of all, the segmentation gap was introduced exactly for this situation to prevent an error. If the UE performs timing adjustments between two consecutive segments, and the timing advance is decreasing (RTT is becoming shorter), the UE is delaying its transmission as smaller timing pre-compensation is needed. In this case there will be a natural timing adjustment gap between two consecutive subframes and there is no need for the segmentation gap.

However, when the timing advance of the UE is increasing, the UE needs to make an earlier transmission of the next segment. This will likely cause the beginning of the second segment to overlap with the end of the first segment in time. This is physically impossible. To resolve this conflict issue, RAN1 has introduced the segmentation gap.

If the timing advance is decreasing between consecutive segments, the segmentation gap is not needed. However, if the timing advance is increasing between two consecutive segments, the segmentation gap is needed.

This issue has been identified and addressed by RAN1 in TS 36.213, clause 16.1.2

- For regular transmissions:

When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

- For segmented transmissions:

For a NB-IoT UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of time units (…)

And further in TS 36.211 section 10.1.3:

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1 for FDD, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled.

Based on the observations above, we propose:

For the cases where the UE is configured with segmentation within the burst of 8 consecutive UL transmissions, the segmentation shall be applied if the timing advance is increasing between the consecutive segments.

Notify RAN1 of the new RAN4 understanding.

Conclusion

In this paper we discussed the TDD punctured pattern for IoT NTN UEs. The following Observations and Proposals were made:

Observation 1: The absence (or introduction) of the pre-compensation gap is inconsequential for the frequency error requirements in RAN4.

Observation 2: By stating that the pre-compensation gap is not applicable in band 249, we are preventing it to be used for timing adjustments.

Observation 3: If the timing advance is decreasing between consecutive segments, the segmentation gap is not needed. However, if the timing advance is increasing between two consecutive segments, the segmentation gap is needed.

Proposal 1: For the cases where the UE is configured with segmentation within the burst of 8 consecutive UL transmissions, the segmentation shall be applied if the timing advance is increasing between the consecutive segments.

Proposal 2: Notify RAN1 of the new RAN4 understanding.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2512550,  Introduction of IoT-NTN TDD mode (IoT_NTN_TDD-Core),   Iridium,  Bengaluru, India, August 25th – 29th, 2025