3GPP TSG RAN4 Meeting #116bis		R4-2514348
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Performance Requirements for NTN UEs on Ku band operation

Agenda item:	5.10.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the discussions for the implementation of RRM requirements related to Ku Band have LED to the implementation of this new feature in the standard. One of the main difference of the new feature is that in the Ku Band,  both FR1-NTN and FR2-NTN numerology might be used, at the discretion of the network.

In this document we provide our views on test cases for Ku band utilization in NR NTN.

Discussion

In RAN4, RRM performance requirements and test specifications are used to evaluate whether UE is performing as expected based on the features configured and the conditions of the scenario. For that, the set of test cases must be designed to cover the relevant aspects of a new feature/functionality.

The new set of bands introduced in Ku band is a new functionality on RRM point of view and has introduced the following modifications in the specifications:

- Timing requirements for VSAT UEs served by NGSO

- Hard and soft satellite switching of VSAT UEs.

- FR1-NTN numerology for NTN UEs.

First of all, one important point that has never been tested before is the FR1-NTN numerology for VSAT UEs. Because of that, RAN4 has to introduce tests that for UEs that support FR1-NTN numerology in Ku Band. One simple way to achieve that is create new reference configurations for VSAT UEs that are related to FR1-numerology (or reuse the old configurations for FR1 numerology) in the tests designed for VSAT UEs.

Introduce in all available test cases for VSAT UEs a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

A second aspect to be taken into account is that there are new timing requirements for mobile VSATs working under NGSO. This new functionality also needs to be tested.

If the mobility aspect of a VSAT UE is relevant enough to be distinguished via VSAT Type whether a UE is mobile or fixed, it is important that they are tested according to the type reported.

For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE.

Introduce NGSO configuration for transmit timing test case of a mobile VSAT.

A third and relevant aspect to be included is related to Hard and Soft satellite switching. It is important that new tests are included to cover for such mobility features in Ku band.

Introduce new test cases for hard and soft satellite switching for VSAT UEs.

Conclusion

In this paper we provided our views on the NTN operation in the KU band. We focused our discussion in the new set of test cases to be introduced for VSAT UEs.

Proposal 1: Introduce in all available test cases for VSAT UEs a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Observation 1: If the mobility aspect of a VSAT UE is relevant enough to be distinguished via VSAT Type whether a UE is mobile or fixed, it is important that they are tested according to the type reported.

Proposal 2: For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE.

Proposal 3: Introduce NGSO configuration for transmit timing test case of a mobile VSAT.

Proposal 4: Introduce new test cases for hard and soft satellite switching for VSAT UEs.

- References

- R4-2512121, WF on RRM requirements for NR_NTN_Ku_bands, Apple, 3GPP TSG-RAN WG4 Meeting #116