3GPP TSG RAN4 Meeting #116bis		R4-2514349
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	On introducing HD-FDD on the Ku Band

Agenda item:	7.6.4.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, comprised with 3 main objectives:

- HP-UE in NTN

- HD-FDD for Ku band operation (both in FR1 and FR2 numerology)

- DL intra-band, intra-SAN CA for NTN

The different objectives were specified with different timelines for the completion of work. For the RRM part, the current work is focused on discussing potential impacts of the HD-FDD implementation in Ku band. In the current phase of work the focus will be in Ku band functioning with FR1 numerology. In this document, we will provide our initial views on the HD-FDD aspects implementation in Ku Band.

Discussion

For the most part, HD-FDD impact on RAN4 requirements is dependent on implementation and design of RAN1. The reason is that there must be rules for handling collisions of UL transmissions and DL reception at the UE when there is an overlap of occasions.

However, the work item description indicates that RAN4 shall focus on the work previously done for RedCap, and try as much as possible leverage on such design:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

For a UE in connected mode, the UL and DL collisions are more difficult to predict in NTN due to the large propagation delays and specially the propagation delay difference between two UEs in the same cell, this leads to several potential collisions, as previously identified by RAN1:

- Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching

For cases 1,2, 5 and 6 RAN1 has decided during Rel-19 to leave those collisions to be handled by existing collision rules. Whereas RAN1 addressed cases 3 and 4:

Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS. Conclusion For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3, - Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. - UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle. For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note1. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note1. - When PDCCH ordered PRACH transmission collides with DL reception, it is up to UE implementation whether to prioritize the PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all these other cases - This is signaled by one UE specific RRC parameter - If DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note1: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Those are the rules to be used as baseline for the conflict between DL and UL in Ku band with FR1 numerology. We propose to use RAN1 conclusion as baseline and look for the cases where the current collision handling rules do not fit Ku band operation or to find collision cases that are not yet covered by RAN1 design.

RAN4 to adopt as baseline the collision handling as designed by RAN1 for RedCap NTN. RAN4 to evaluate if more cases of collision handling need to be addressed.

At the end of the investigation, RAN4 to inform RAN1 about the findings, such that RAN1 can create a feature at their specification covering for such collision handling.

For several of the collision handlings identified, the problem can be drastically mitigated if the location of the UE or the TA estimation is reported to the network. Although this has been set as an optional feature for RedCap (and regular NR NTN) devices, Ku band devices tend to be more complex in their implementation. Besides, the Ku band design and specificities already hamper the UE with significant overhead during mobility procedures (where there is significant scheduling interruption/restriction) in special related to satellite switching and inter-satellite HO. The “duty cycle” is further restricted in a HD-FDD operation. Therefore, it might be beneficial for the Ku Band UEs to support TA estimation.

RAN4 to discuss recommending to RAN1 that  TA reporting should become a mandatory feature for Ku Bands supporting HD-FDD.

In regard of mobility scenarios, where HD-FDD may cause scheduling restrictions or delay in measurements (or cause the measurement requirements to not be applicable due to collisions between SMTC and UL transmissions), RAN4 shall look for enhancements in gap-less intra-satellite measurement to apply meaningful requirements and measurement procedures that can coexist with UL transmissions and HD-FDD operation.

RAN4 to study potential enhancements in intra-satellite measurements to support mobility measurements in HD-FDD mode.

Conclusion

In this paper we discussed the impact of introducing HD-FDD for Ku band with FR1 numerology. Our proposals for this topic were provided as follows:

Proposal 1: RAN4 to adopt as baseline the collision handling as designed by RAN1 for RedCap NTN. RAN4 to evaluate if more cases of collision handling need to be addressed.

Proposal 2: At the end of the investigation, RAN4 to inform RAN1 about the findings, such that RAN1 can create a feature at their specification covering for such collision handling.

Proposal 3: RAN4 to discuss recommending to RAN1 that  TA reporting should become a mandatory feature for Ku Bands supporting HD-FDD.

Proposal 4: RAN4 to study potential enhancements in intra-satellite measurements to support mobility measurements in HD-FDD mode.

- References

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, RAN WG4 Meeting #116, Bengaluru, India, 25th - 29thAugust, 2025

- on SMTC enhancements, RAN WG2 Meeting #129bis, Wuhan, China, 7 - 11 Apr, 2025