3GPP TSG-WG4 Meeting # 116bis	R4-2514353

# Prague, Czech, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to 38.133 on positioning reporting delay requirements (Rel-19) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add positioning reporting delay requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New clauses for positioning reporting delay requirements are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Positioning reporting delay is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 9.9.X and 4.5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <START OF CHANGE 1>

#### 9.9.X Positioning Reporting Requirements for UE-based direct AI/ML Positioning

The positioning reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the positioning report over the air interface. This requirement assumes that the positioning report is not delayed by other LPP signalling on the DCCH. This positioning reporting delay excludes a delay uncertainty resulted when inserting the positioning report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the positioning report is transmitted on the PUSCH with subframe or slot or subslot duration. This position reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

When receiving a RequestLocationInformation message from the LMF via LPP [22, 34], the UE shall be able to send the positioning response, defined in [34], from configured PRS resources for configured TRPs on configured positioning frequency layers, within

Treport, total=Tmeas+Tinf+Treport, LPP,

where Tmeas is the time taken by the UE to perform the required measurements, Tinf is the time taken to process or infer positioning-related data, and Treport, LPP is the time taken to prepare and transmit the report to the network.

## <END OF CHANGE 1>

## <START OF CHANGE 2>

#### 4.5.X Positioning Reporting Requirements for UE-based direct AI/ML Positioning

The positioning reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. The UE will transition to RRC_CONNECTED state prior to transmitting the positioning report.

For positioning performed by a UE in RRC_IDLE state, the positioning reporting delay excludes all of the following:

- additional delay caused other LPP signalling on the DCCH,

- delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

- any delay caused by unavailability of UL resources to transmit the measurement report,

- the time needed to transition to RRC_CONNECTED state to report the measurements.

If a positioning measurement is performed in RRC_IDLE state, when receiving a RequestLocationInformation message from the LMF via LPP [34], the positioning reporting requirements defined in clause 9.9.X is re-used for the positioning reporting delay requirement.

## <END OF CHANGE 2>