3GPP TSG-RAN WG4 Meeting #116bis	R4-2514354

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Nokia

Title:	On RAN4 Impacts for Measurement Event Prediction in AIML Mobility

Agenda item:	7.12.4

Document for:	Discussion

Introduction

The study of AI/ML-based mobility concluded at RAN4#116 [1] in the scope Rel-19 SI [2]. During SI, it was agreed that RAN4 will define relative and absolute accuracy requirements for the potential RSRP report corresponding to the predicted event occurrence. However, other performance metrics and requirements have not yet been defined and require further discussions during the WI.

This contribution continues that discussion and provides views focusing primarily on the requirements and performance aspects of indirect measurement-event prediction.

Discussion

Regarding performance metrics and requirements for event prediction, the following agreements were reached at RAN4#116, RAN4#115, and RAN4#114-bis, respectively [1].

RAN4#116 Agreement: - On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase. - Capture the candidate proposals in the WF for information purpose.

RAN4#115: Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

RAN4#114bis Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

Indirect Measurement Event Prediction

For indirect measurement-event prediction in intra-frequency scenarios—such as temporal-domain Case A, temporal-domain Case B, or the frequency-domain case shown in Figure 1—a Radio Resource Management (RRM) measurement-prediction model first predicts the measurement result(s) using AI/ML techniques. The predicted results, together with optionally included historical measurement data from the same cell, are then used to determine whether a measurement event will occur at a specific future time. The subsequent post-processing phase operates without further AI/ML involvement.

Figure 1: Indirect measurement event prediction.

RAN2 has agreed on four models for indirect measurement-event prediction: time-domain Case A, time-domain Case B, frequency-domain, and spatial-domain prediction. Based on current progress, RAN2 is primarily focusing on the two time-domain models. Case A targets FR2, aiming to predict A3 events to enable earlier handover (HO) preparation and improve HO execution performance. Case B addresses scenarios where measurements may be skipped, aiming to predict A3 events under such conditions to reduce measurement overhead while maintaining comparable HO performance. Figure 2 illustrates an example of time-domain Case A RRM measurement prediction.

Figure 2: Time domain case A RRM measurement prediction

In the context of time-domain indirect measurement event prediction Case A, an A3 measurement event may be predicted prior to its actual occurrence, and this prediction may be used to support optimized mobility decisions. According to TR 38.744, two options for handover execution decision are defined: one option relies on the legacy measurement event as the basis for triggering the handover, while the other relies on the predicted measurement event. In both cases, the prediction may be used to initiate early handover preparation to enhance handover performance.

In time-domain indirect measurement event prediction Case B, the objective is to reduce the number of required downlink RRM measurements by allowing the UE to skip certain measurement occasions. The skipped measurements are predicted using a machine learning model trained on historical measurement data. This enables a reduction in measurement overhead while maintaining handover performance at a level comparable to that achieved with full measurement reporting.Two examples for skipping patterns are shown in Figure 3.

Figure 3: Examples of time domain case B measurement prediction

It has been agreed in RAN2 to study the measurement event prediction also with time domain case B RRM measurement prediction in FR1. In this case, the PW is typically short, and predictions are not made for every measurement period, so any gain that can be expected from the future predictions is low. On the other hand, all mobility decisions are made based on measurement-event-triggered reporting based on actual measurements and predictions, which may set higher requirements for the accuracy of the predictions.

RAN2 has considered the following scenarios for indirect measurement-event prediction using temporal-domain methods;

- Case A – The UE predicts a probable future measurement event that may support mobility-optimization decisions, without measurement reduction.

- Case B – The UE predicts measurements for measurement-reduction purposes, with no significant impact on future mobility performance.

RAN2 Performance Metric for Indirect Measurement Event Prediction

However, in RAN2, F1 score with the following details has been agreed as the evaluation metric for AI/ML measurement event prediction [TR 38.744].

The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

Although the F1 score is an accepted metric in RAN2 for evaluating measurement event prediction performance, its testing and interpretation may present challenges due to several practical aspects.

One key consideration is the type of inference triggering used for measurement event prediction. Prediction may be performed either periodically or in an event-based manner. In the periodic case, the prediction is inferred at every measurement sampling instance when an A3 event is configured. In contrast, event-based inference is triggered when a specific prediction condition is met, such as when the A3 entry condition becomes true, and the prediction then estimates whether the condition will be maintained for the time-to-trigger (TTT) duration.

The choice between periodic and event-based prediction affects the timing and interpretation of performance counters, including the initialization of counters and the frequency of updates. In many cases, the starting point of the prediction window and the update mechanism for counters are not clearly defined, introducing ambiguity in F1 score evaluation. For example, in the periodic case, predictions may be triggered well before the A3 condition is relevant (e.g., when the serving cell RSRP is still significantly better than the target cell), resulting in counter behaviors and F1 scores that differ from those obtained under event-based inference, where predictions begin only once the A3 entry condition is satisfied.

The starting point of F1 score calculation, and the choice between periodic and event-based prediction affects the timing and interpretation of performance counters for F1 score metric.

From a network perspective, false event detections are generally more detrimental than missed event detections especially for indirect event prediction based on temporal domain Case-A measurement prediction. A false event detection can trigger unnecessary handover preparation procedures, leading to wasted signaling effort and the reservation of resources (e.g., target cell admission capacity, RRC/NG signaling, buffer allocation) that ultimately go unused if the UE was never actually in need of a handover. This inefficiency can cascade—causing increased network load, degraded resource utilization, and even impact on other users if the reserved resources block more urgent sessions.

By contrast, a missed event detection—while still undesirable—tends to have a lower cost to the network, particularly when the UE is also configured to report based on actual measurement thresholds in addition to prediction-based triggers. In such cases, the network can still respond to the UE’s measurement-based event reports, meaning the risk is primarily a delayed handover (or similar performance as in legacy) rather than wasted resources.

Because the F1 score gives equal weight to precision and recall, it inherently treats false detections and missed detections as equally important. This symmetry in the metric does not reflect the operational priorities of a live network, where avoiding false positives often takes precedence over avoiding false negatives. Consequently, optimizing solely for F1 score may lead to prediction models that perform well statistically but are suboptimal from a network cost-efficiency perspective.

From a network perspective, false event detections are generally more detrimental than missed event detections. However, this asymmetry in impact is not reflected in the F1 score calculation, as the metric assigns equal weight to precision and recall.

Additionally, if there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, i.e., each target candidate cell is its own data sample in the evaluation, or if predicting the right target candidate cell is included in the metrics calculation. In the latter case, RAN2 should discuss how the metrics are calculated.

In practice, verifying whether an AI/ML-based UE satisfies a required F1 score for A3 measurement event prediction is challenging due to the significant testing overhead. Evaluating F1 scores reliably requires collecting large volumes of labeled data across diverse scenarios to capture true positives, false positives, and false negatives. This demands repeated field or lab trials under controlled conditions, making the process time-consuming and resource intensive.

Testing the UE with F1 score as the metric for A3 measurement event prediction is not clear especially for multiple target cell scenarios and comes with significant testing overhead.

RAN4 Performance Metric, Requirements, and Testing for Event-based Measurement Reporting

Based on TS 38.133, for event reporting based on pure measurement, the UE shall send one Event A3-triggered measurement report within the defined measurement reporting delay. The UE shall not send event-triggered measurement reports if the reporting criteria are not met. The rate of correct events observed during repeated tests shall be at least 90%. Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements specified in TS 38.133 clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2).

For legacy event-based measurement reporting, the measurement reporting delay is defined as the time between the occurrence of an event that triggers a measurement report and the moment when the UE begins transmitting the measurement report over the air interface. This requirement assumes that the report is not delayed by other RRC signalling on the DCCH. The measurement reporting delay excludes any timing uncertainty associated with inserting the measurement report into the TTI of the uplink DCCH.

Currently, several tests have been defined in 38.133 to verify that the UE correctly reports an event, e.g., “TS38.133; A.6.6.1.1 SA event triggered reporting tests without gap under non-DRX.” In this test, two FR1 cells operate on the same frequency: Cell 1 acts as the PCell and Cell 2 as a neighbor. The UE is configured for Event A3 reporting and experiences two consecutive time periods, T1 and T2. During T1 the UE has no timing information for Cell 2; during T2 Cell 2 starts transmission, with the Time-to-Trigger set to zero and no L3 filtering applied.

The UE is required to send exactly one Event A3 measurement report within 800 ms from the start of T2, not report unless the trigger criteria are satisfied, and achieve at least a 90 % success rate across repeated runs. The neighbor cell’s SSB index does not need to be read for the test, and overall observed delays may exceed the specified measurement-reporting delay by up to twice the DCCH TTI due to TTI insertion uncertainty.

Figure 4: Example of testing setup for measurement event reporting[TS 38.133]

For legacy event-based measurement reporting, the measurement reporting delay is defined as the time between the occurrence of an event that triggers a measurement report and the moment when the UE begins transmitting the measurement report over the air interface.

Legacy definition of measurement reporting delay may not be applicable for measurement event prediction, especially for event prediction based on temporal domain Case-A.

RAN4 Performance Metric and Requirements for Indirect Measurement Event Prediction

For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable. An early prediction allows the network to preemptively prepare target cells, reduce HO latency, and improve user experience during mobility. However, the standard F1 score evaluation does not differentiate between early and late predictions. It treats a prediction occurring before the ground-truth event the same as one occurring after—despite their vastly different operational impact. In reality, a late A3 prediction may lead to HO failure or interruption, whereas an early one enhances robustness.

For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable.

For the time domain case B measurement event prediction evaluation, as the gain is in measurement reduction, it is expected the UE to predict the event within a short prediction window. So, it seems possible to define a same performance metric for both temporal domain case A and B, with difference performance values and consider case specific requirements.

For the time domain case B measurement event prediction, a similar performance metric definition as that of temporal domain case A.

Based on the above discussions regarding the requirements and performance metrics for AI/ML-based indirect measurement event prediction, the following KPIs may be considered for evaluation. The choice between time-domain Case A and Case B may impose different performance requirements due to the distinct objectives and constraints of each approach. As detailed in Figure 4, the following KPIs are identified as relevant for indirect measurement event prediction:

Figure 5: Indirect measurement event prediction

- Event Prediction Window (Prediction Horizon): Refers to the future time interval within which the UE predicts the entry and/or exit conditions of a configured measurement event (e.g., A3). For time domain Case A the prediction window is considered as multiple measurement samples (close to measurement prediction Case A prediction window) and shorter or no lead time for Case-B where prediction is closely aligned to the event occurrence.

- Event Prediction Accuracy: Captures both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence. It consists of two components:

  - Outcome Accuracy: whether the prediction correctly identifies event occurrence or non-occurrence. From a network perspective, false event detections are generally more detrimental than missed detections. So outcome accuracy should be calculated with weighting that reflects this operational asymmetry.

  - Temporal Accuracy: Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance.

- Event Prediction Reporting Delay: Defined as the time interval between the point at which the UE initiate the event prediction and the point at which the UE transmits the corresponding event prediction report over the air interface.

- Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value.

- Number of Cells/SSBs for which Prediction is Supported: Indicates the capability of the UE to simultaneously predict measurement events for multiple cells or SSBs.

RAN4 to consider following performance metrics for indirect measurement event prediction.

- Event Prediction Accuracy: Captures both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence. It consists of two components:

  - Outcome Accuracy: whether the prediction correctly identifies event occurrence or non-occurrence.

  - Temporal Accuracy: Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance.

- Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value.

RAN4 to consider following configurations for indirect measurement event prediction requirements

- Event Prediction Window (Prediction Horizon): Refers to the future time interval within which the UE predicts the entry and/or exit conditions of a configured measurement event (e.g., A3).

- Event Prediction Reporting/Inference Delay: Defined as the time interval between the point at which the UE initiate the event prediction and the point at which the UE transmits the corresponding event prediction report over the air interface.

- Number of Cells/SSBs for which Prediction is Supported: Indicates the capability of the UE to simultaneously predict measurement events for multiple cells or SSBs.

Measurement Reporting

Regarding measurement reporting for event prediction, following agreements have been achieved in RAN4#115.

RAN4#115; Agreement:​ In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

According to 3GPP TS 38.133, the accuracy requirements for RSRP measurements reported in event-triggered measurement reports are aligned with those specified for periodic measurement reporting. Specifically, the same accuracy definitions and tolerances apply to both reporting modes, ensuring consistency across different measurement procedures. These requirements are detailed in Clause 10.1.2.1 for Frequency Range 1 (FR1) and Clause 10.1.3.1 for Frequency Range 2 (FR2).

- Similar accuracy definition and requirements have been considered for Even-based measurement reporting and periodic measurement in TS 38.133.

For event-based temporal domain measurement reporting based on indirect event prediction, it is proposed that accuracy requirements similar to those already defined/under definition for measurement prediction be adopted. At this stage, there appears to be no need for defining additional or separate accuracy specifications beyond what is considered for predicted measurements. It should also be recognized that the applicable tolerance or accuracy levels for predicted measurements in such scenarios may be influenced by several key factors, including length of the observation window used for collecting measurements, the duration of the prediction window over which the event or measurement is forecasted, etc.

For indirect measurement event prediction use case, for potential report that includes the predicted RSRPs, RAN4 to assume a similar RSRP accuracy definition for predicted measurements. The applicable tolerance or accuracy levels may be influenced by several key factors, including length of the observation window, the duration of the prediction window, etc.

Testing aspects of Measurement Event Prediction

For event prediction testing, in Case A, one approach is to configure the UE to also trigger a conventional (actual) event alongside the prediction, enabling direct comparison between predicted events and those triggered by actual measurements. In Case B (temporal domain), ground truth can be derived from known channel conditions in conducted tests or from UE measurement reports (e.g., obtained under identical, repeated conditions). This enables verification of the UE’s predicted events against a reliable reference. In this regard, RAN4 could draw inspiration from and potentially reuse the agreed evaluation method for indirect measurement prediction.

For indirect event prediction, UE event reports based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, can be used to analyse the occurrence of the real event.

For the testing of event prediction, RAN4 may use UE measurement event reporting based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, for analysing the occurrence of the real event.

For the testing of event prediction, RAN4 may wait further progress on testing procedures for measurement prediction and consider reusing the agreed prediction evaluation approach for indirect event prediction.

Conclusion

In this paper we provide our view on the Rel-19 AI/ML mobility measurement event prediction scenario to be discussed in RAN4 working group.

The following Observations and Proposals were made:

- RAN2 has considered the following scenarios for indirect measurement-event prediction using temporal-domain methods;

- Case A – The UE predicts a probable future measurement event that may support mobility-optimization decisions, without measurement reduction.

- Case B – The UE predicts measurements for measurement-reduction purposes, with no significant impact on future mobility performance.

The starting point of F1 score calculation, and the choice between periodic and event-based prediction affects the timing and interpretation of performance counters for F1 score metric.

From a network perspective, false event detections are generally more detrimental than missed event detections. However, this asymmetry in impact is not reflected in the F1 score calculation, as the metric assigns equal weight to precision and recall.

Testing the UE with F1 score as the metric for A3 measurement event prediction is not clear especially for multiple target cell scenarios and comes with significant testing overhead.

For legacy event-based measurement reporting, the measurement reporting delay is defined as the time between the occurrence of an event that triggers a measurement report and the moment when the UE begins transmitting the measurement report over the air interface.

Legacy definition of measurement reporting delay may not be applicable for measurement event prediction, especially for event prediction based on temporal domain Case-A.

For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable.

For the time domain case B measurement event prediction, a similar performance metric definition as that of temporal domain case A.

- RAN4 to consider following performance metrics for indirect measurement event prediction.

- Event Prediction Accuracy: Captures both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence. It consists of two components:

  - Outcome Accuracy: whether the prediction correctly identifies event occurrence or non-occurrence.

  - Temporal Accuracy: Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance.

- Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value.

RAN4 to consider following configurations for indirect measurement event prediction requirements

- Event Prediction Window (Prediction Horizon): Refers to the future time interval within which the UE predicts the entry and/or exit conditions of a configured measurement event (e.g., A3).

- Event Prediction Reporting/Inference Delay: Defined as the time interval between the point at which the UE initiate the event prediction and the point at which the UE transmits the corresponding event prediction report over the air interface.

- Number of Cells/SSBs for which Prediction is Supported: Indicates the capability of the UE to simultaneously predict measurement events for multiple cells or SSBs.

- Similar accuracy definition and requirements have been considered for Even-based measurement reporting and periodic measurement in TS 38.133.

For indirect measurement event prediction use case, for potential report that includes the predicted RSRPs, RAN4 to assume a similar RSRP accuracy definition for predicted measurements. The applicable tolerance or accuracy levels may be influenced by several key factors, including length of the observation window, the duration of the prediction window, etc.

For indirect event prediction, UE event reports based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, can be used to analyse the occurrence of the real event.

For the testing of event prediction, RAN4 may use UE measurement event reporting based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, for analysing the occurrence of the real event.

For the testing of event prediction, RAN4 may wait further progress on testing procedures for measurement prediction and consider reusing the agreed prediction evaluation approach for indirect event prediction.

- References

- R4-2509057, Topic summary for [116][214] FS_NR_AIML_Mob_Part1, Nokia, Bengaluru, India, August 25th – August 29th, 2025.

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.